3GPP TSG RAN WG4 Meeting #88	R4-1810603

Gothenburg, Sweden, August 20 – 24, 2018

Agenda Item:	7.4.2

Source: 	Ericsson

Title:	On SFTD for NR-NR DC

Document for:	Approval

# Introduction

At RAN#80, it was agreed that NR-NR DC, i.e. dual connectivity between NR PCell and PSCell, is to be included in Rel-15 as a late drop [1]. From physical layer operation point of view, only synchronous operation is supported in Rel-15, while asynchronous operation will be introduced in later release.

Although, due to synchronous operation in Rel-15, there may not be a need for SFTD reporting between NR PCell and NR PSCell in Rel-15 time frame, there is still a need for SFTD reporting between NR PCell and NR neighbour cell (inter-frequency SFTD). The reason is that NR cells in different bands or on different carriers may use different sycnhronization sources, e.g. due to multi-vendor realization of the network, and that it  therefore may be timing drift between the systems over time. Precise knowledge of the timing of inter-frequency neighbour cells is required for the PCell to be able to configure UEs with appropriate MG and SMTC window for inter-frequency mobility measurements.

In this contribution we are proposing core requirements for inter-frequency SFTD between NR PCell and NR neighbour cell. RAN4 has some questions to RAN2 still outstanding regarding SFTD of additional neighbour cells on monitored and non-monitored carriers [2]. Therefor, in this contribution we are limiting the scope to the scenario where no PSCell is configured.

# Discussion

## Inter-frequency SFTD with NR PCell

Inter-frequency SFTD with NR PCell and NR neighbour cell is needed for the case when NR cells in different bands are using different synchronization sources, by which there may be timing drift between the systems over time [3]. For this case, the NR PCell does not know which SMTC window and MG to configure for the UE to find the neighbour cell. The UE then has to carry out a search over all possible timings, similar to how it is carried out for inter-RAT SFTD with EUTRA PCell and NR neighbour cell. We therefore propose that the same mechanism is used for inter-frequency SFTD with NR PCell.

Proposal 1: Inter-frequency SFTD with NR PCell shall reuse the same mechanism as used for inter-RAT SFTD with EUTRA PCell.

One difference to the inter-RAT SFTD with EUTRA PCell case is that NR cell detection resources may already be tied up by intra-frequency measurements, including serving cell measurements, in configured SMTC windows. For inter-RAT SFTD with EUTRA PCell, we have not covered such resource constraints in Rel-15, which for instance would result if the UE is carrying out inter-RAT NR measurements on one carrier and is requested to carry out inter-RAT SFTD measurement on another NR carrier. Instead it has been assumed that the network shall de-configure conflicting inter-RAT and inter-frequency measurements when an inter-RAT SFTD measurement is requested. The same approach is not feasible for inter-frequency SFTD measurements with NR PCell, since it potentially would jeopardize the connection to the serving cell. We therefore propose that the core requirements shall take resource constraints with respect to serving cell measurements in SMTC windows into account. The effect is that the inter-frequency SFTD measurement with NR PCell will take somewhat longer time than the corresponding inter-RAT SFTD measurement with EUTRA PCell.

Proposal 2: The core requirement for inter-frequency SFTD measurement with NR PCell shall take resource constraints, arising from serving cell measurements, into account. Hence a longer SFTD measurement period than for corresponding inter-RAT SFTD measurement with EUTRA PCell is anticipated.

Regarding measurement performance requirements, since both the PCell and the neighbour cell belong to NR, a somewhat tighter measurement accuracy than achievable for e.g. EN-DC can potentially be attained. We therefor propose that for inter-frequency SFTD with NR PCell, the measurement performance requirements shall reflect that there is a constant number of REs carrying synchronization and PBCH demodulation reference signals available in each SSB regardless of cell bandwidth.

Proposal 3: The measurement performance requirement for inter-frequency SFTD measurement with NR PCell shall reflect that there is a constant number of REs carrying SS and PBCH DM-RS, and which can be used for time tracking, in each SSB. Hence the measurement performance requirement may differ from corresponding requirements involving an EUTRA cell.

## SFTD with NR PCell and NR PSCell

According to the WI exception sheet [1], only synchronous mode of NR-NR DC with respect to physical layer operation is supported in Rel-15. Hence there is no need to introduce SFTD with NR PCell and NR PSCell in Rel-15 time frame.

Proposal 4: Since only synchronous mode of NR-NR DC is introduced in Rel-15, no requirement on SFTD between NR PCell and NR PSCell is to be introduced in Rel-15 time frame.

# Summary and Conclusions

The following proposals are made:

Proposal 1: Inter-frequency SFTD with NR PCell shall reuse the same mechanism as used for inter-RAT SFTD with EUTRA PCell.

Proposal 2: The core requirement for inter-frequency SFTD measurement with NR PCell shall take resource constraints, arising from serving cell measurements, into account. Hence a longer SFTD measurement period than for corresponding inter-RAT SFTD measurement with EUTRA PCell is anticipated.

Proposal 3: The measurement performance requirement for inter-frequency SFTD measurement with NR PCell shall reflect that there is a constant number of REs carrying SS and PBCH DM-RS, and which can be used for time tracking, in each SSB. Hence the measurement performance requirement may differ from corresponding requirements involving an EUTRA cell.

Proposal 4: Since only synchronous mode of NR-NR DC is introduced in Rel-15, no requirement on SFTD between NR PCell and NR PSCell is to be introduced in Rel-15 time frame.

A draft CR on introduction of inter-frequency core requirements for SFTD is provided in [4].

# References

- RP-181473 “Rel-15 Work Item Exception for New Radio Access Technology – Core”, NTT DOCOMO, Inc.

- R4-1808442 “Draft reply LS on SFTD measurements”, ZTE

- R4-1808763 “Remaining issues on requirements of SFTD measurement”, NTT DOCOMO, Inc.

- R4-1810604 “Draft CR 38.133 Introduction of inter-frequency SFTD core requirements”, Ericsson