3GPP TSG-RAN WG4 Meeting #88                	R4-1810881

Montreal, Canada, July 2nd – 6th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for NR PRACH

Agenda item:	7.13.2.4

Document for:	Discussion

Introduction

In this contribution we introduce simulation results using the parameters as agreed in [1].

Simulation assumptions

Simulation assumptions as agreed for NR PRACH in [1] are shown below.

Table 1: Simulation assumptions for NR PRACH

| Preamble format | A2 |
| --- | --- |
| Number of Tx | 1 |
| Number of Rx | 2 |
| SCS and CBW | 30kHz SCS: 40 120kHz SCS: 100 |
| Ncs | FR1:10, 46, 69 FR2:0, 10, 69 |
| Logical sequence index | 0 |
| Carrier Frequency [GHz] | 4,30 |
| Propagation Channel | AWGN |

Simulation results

Simulation results for NR PRACH using the parameters from Table 1, and False alarm probability less than 0.1% are shown below in Table 2 for different Ncs.

Table 2: Initial simulation results for PRACH format A2

| SCS [KHz] | CBW (MHz) | Ncs | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- | --- |
| 30 | 40 | 10 | -15.0 |
|  |  | 46 | -14.7 |
|  |  | 69 | -14.6 |
| 120 | 100 | 0 | -14.1 |
|  |  | 10 | -14.7 |
|  |  | 69 | -14.2 |

Conclusion

In this contribution we have presented simulation results for different Ncs.

- References

- R4-1809369, Way forward on NR PRACH demodulation requirements, Ericsson