3GPP TSG-RAN4 Meeting 88 	R4-1810909

# Göteborg, Sweden, 20st – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 37.843 Beam sweeping annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A beam sweeping test signal has been discussed in previous contributions [1-2] and is needed to decorrelate the pattern to allow for much sparser grids for TRP. The proposed text can be placed in a separate Annex or in a sub annex together with other concepts relevant for TRP measurements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description of beam sweeping test signal |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TRP requirements at harmonic frequencies will be lengthy and costly without considering beam sweeping during TRP measurement on a spherical grid. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## ----start of changes----

# Annex X.Y Beam sweeping test mode

For emissions with a high beam-forming gain correct assessment of TRP requires small angular steps. In a measurement where the EUT is mounted on a turntable this turns into a large number of mechanical rotations of the EUT and hence lengthy measurements. One efficient way to mitigate this is to use electrical beam sweeping as a complement to mechanical rotations.

## X.Y.1 Simulation results

Figure X.Y.1 In the left figure a grid of beams for a 10x10 array is depicted together with the corresponding element pattern. In the right picture two different settings of a spectrum analyser are simulated. In the “Max hold” case the maximum EIRP over all beams is detected at every angular point, and in the “Average” case the average EIRP over all beams is detected. The latter case is the correct setting to use for TRP assessment.

| Figure X.Y.2 The beam sweeping at the 2nd harmonic frequency is simulated using an embedded element pattern. Left plot depicts the EIRP patterns of individual beams. Right plot depicts the max hold and beam average EIRP patterns. |  |
| --- | --- |

In the example of Fig. X.Y.1 the reference step is 11.6 degrees (ϕref=(λ/5λ) radians). For the harmonic in Fig. X.Y.2 the reference step is 5.8 degrees. The full sphere average of the EIRP is the TRP. For simplicity only EIRP data in a horizontal cut is used and average EIRP denotes an angular average in the horizontal plane using different step sizes. In the case where a fixed beam is used it is seen that the average EIRP is not correct when the sampling step is larger than the reference step. However, when the EIRP pattern is first averaged over all beams (blue curve) the average EIRP is virtually constant for any angular step up to 36 degrees. The beam sweeping range is selected to completely avoid the forming of grating lobes and the angular step between two lobes is selected as the Half Power Beamwidth (HPBW) of a broadside beam.

| Fundamental frequency | 2nd harmonic |
| --- | --- |
| Figure X.Y.2 Calculation of average EIRP in the azimuth cut by using different angular step sizes. The yellow curve corresponds to a fixed broad side beam, and all the green curves correspond to steered but fixed beams. The variation in TRP for different beams at the harmonic is due to grating lobe effects. |  |

## X.Y.2 Measurement results

The effect of beam sweeping to measured data is depicted in Figures X.Y.3 and X.Y.4

Figure X.Y.3 EIRP values for measured data in a horizontal cut and versus frequency (in relative scale).

Figure X.Y.4 To the left measured data for a horizontal cut are shown at a single frequency. A significantly broader pattern is detected for the beam sweeping signal (spectrum analyser in average mode). To the right the angular average of the EIRP values are depicted for different step sizes. ## X.Y.3 Design of beam sweeping test signal The following design criteria have been identified - Full transmit power is used for all beams. - Beam directions of the SSB block are used, or alternatively the horizontal beam separation shall be the HPBW in the azimuth direction and the vertical beam separation shall be the HPBW in the vertical direction evaluated for a broadside beam. - Lobes with high scan loss are avoided. (Scan loss is the reduction in peak gain when the beam is swept.) The motivation for the third criterion is to have a fairly constant TRP for the used beams. The scan loss can be due to both loss in element pattern gain but also due to adverse mutual coupling effects. In extreme cases scanning the beam can lead to full or partial scan blindness. This implies that a large portion of the transmitter power is reflected back into the antenna array, and hence the TRP of such lobes will be low. Removing lobes with high scan loss gives a beam average TRP that is close to the TRP for any individual beam. As an example: We try to keep the entire grating lobes out of the visible region for and Ncol×Nrow array. Ideally this leads to sinθmax=Nrow-1λNrowΔh-1sinϕmax=Ncol-1λNcolΔv-1 An Ncol×Nrow=16×8 array with horizontal element separation Δh=0.55λ and vertical element separation Δv=0.6λ leads to sinθmax=7λ8 0.6λ-1≈0.46⇒θmax≈27∘

sinϕmax=15λ16 0.55λ-1≈0.70⇒ϕmax≈44∘ The HPBW≈asinλw where w is the array width in the corresponding direction. This leads to the HPBWs HPBWvertical≈asinλ8 0.6λrad≈12∘ And HPBWhorizontal≈asinλ16 0.55λrad≈6.5∘ This would lead to a total of about 5 beams in the vertical direction and 14 in the horizontal direction.

## ----end of changes----

[1] R4-1807483 On beam sweeping for TRP measurements

[2] R4-1809097 Beam sweeping – in-band measurement results