3GPP TSG-RAN WG4 #88	R4- 1811049

Gothenburg, Sweden, 20th – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR37.843 OTA adjacent channel selectivity, narrow-band blocking, and general blocking in a Near Field Test Range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no text capturing detailed procedures or MU in Near Field clause for adjacent channel selectivity, narrow band blocking and in-band blocking. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Conformance test procedure and MU budget are included for Near Field Test Range. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text is needed for OTA Blocking. If not included, the current text doesn’t reflect what it is measured in Near Field Test Range |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 10.3.5.3	Near

##### 10.3.5.3.1	General

This method measures the adjacent channel selectivity, narrow-band blocking, and general blocking in near field test range. The measurement system setup is as depicted in figure 10.3.5.3.1-1. A signal generator, and a power combiner is added to the OTA sensitivity setup in order to emulate the wanted plus interfering signals at the AAS BS location.

Figure 10.3.5.3.1-1: Example of a Near Field measurement system setup for adjacent channel selectivity, general blocking and narrowband blocking

##### 10.3.5.3.2 	Test method limitations and scope

The same limitations as the ones described in TR 37.842 subclause 10.3.2.1.4.2 apply for blocking type of measurements since an OTA sensitivity measurement is required in order to make sure the measurement is done for the OSDD declared.

##### 10.3.5.3.3 	Calibration

Calibration shall be done with the procedure shown in 37.842 [4] subclause 10.3.2.1.4.3 for both the frequencies of wanted and interfering signals with power combiner/coupler.

##### 10.3.5.3.4 	Procedure

The testing procedure consists of the following steps:

- Perform a full 3D OTA sensitivity measurement by following the procedure depicted in TR 37.842 subclause 10.3.2.1.4.3

  - This measurement step is needed in order to make sure the AAS BS is aligned with the beam peak direction pairs as described in TS37.145-2 subclause 6.2

- Configure RF signal generator for wanted signal to set the signal at the required level and at reference measurement channel.

- Configure RF signal generator for interferer to set the signal at the required level and at reference measurement channel. The interferer’s center frequency offset should be swept as required.

- Measure the throughput in near field of the wanted signal according to annex E of the 3GPP TS 36.141 [??].

- Repeat steps 2-4 for all conformance test directions.

- Repeat steps above per involved band for multi-band RIB(s).

##### 10.3.5.3.5 	MU assessment

###### 10.3.5.3.5.1 	MU Budget

The MU is estimated based on MU in the conducted specification 36.141, the MU for EIS in section 10.3.3 and an estimated uncertainty for matching of test equipment and DUT or test antenna.

Table 10.3.5.3.5.1-1: Near Field uncertainty budget format for mismatch error

| UID | Description of uncertainty contribution | Details in anex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 2 | Standing wave between DUT and test range antenna | C2-2 |

###### 10.3.5.3.5.2 	MU Value

The 2 sigma matching uncertainty is 0.294 dB for  f ≦ 3GHz and 0.294 dB for  3GHz < f ≦ 4.2 GHz .

Test system uncertainty is obtained by follwing equations:

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect

Where

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

And we assume  MUmatching_conducted= MUmatching_OTA

Test system uncertainty for ACS, narrow band blocking, and general band blocking is shown in Table 10.3.5.3.5.2-1. Note that all uncertainties in table are 1.96σ values.

Table 10.3.5.3.5.2-1 Test system uncertainty of ACS, narrow band blocking, and general band blocking for Near Field.

| Frequency range |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |  |
| --- | --- | --- | --- | --- |
| ACS and narrow band blocking | MUwanted_conducted | ±0.7 dB | ±1 dB |  |
|  | MUinterfer_conducted | ±0.7 dB | ±1 dB |  |
|  | MUmatching_conducted | ±0.294 dB | ±0.294 dB |  |
|  | MUwanted | ±0.635dB | ±0.956 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUinterfer | ±0.635 dB | ±0.956 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.24 dB | 1.24 dB |  |
|  | MUTestEquipiment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA | ±0.294 dB | ±0.294 dB |  |
|  | MUchamber_OTA | ±0.8 dB | ±0.8 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |
|  | MUACS_OTA | 1.63 dB | 2 dB |  |