3GPP TSG-RAN WG4 #88	R4- 1811255

Gothenburg, Sweden, 20th - 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.843 OTA OBUE measurement in Near Field Test Range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding measurement procedure for OTA OBUE measurement in Near Field Test setup |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for OTA OBUE measurement procedure for in-band TRP requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedure is needed for OTA OBUE requirement with respect to the Near Field test method. If not included, then Near Field test method is excluded from this OTA requirement for conformance requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 10.4.3.4	Near Field

##### 10.4.3.4.1	General

##### The system is depicted in section 10.2.2.4.1. In case of OTA OBUE measurement, NF to FF transform is not needed since TRP is computed based on power density measured in Near Field by sampling properly the power density for OBUE.

##### 10.4.3.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in section 10.2.2.4.2

##### 10.4.3.4.3 	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

- Configure the beam of the AAS BS according to the required conditions for the TRP test

- Set the AAS BS to transmit the test signal at the maximum power according to the appropriate test model e.g. E-TM1.1.

- Measure the power density at a number of points (Theta;Phi) in the beam according to the chosen measurement grid. Refer to Annex X for details about measurement grids for TRP measurements

- Calculate TRP from the measured power density as in Annex X

The measured power in step 3 is the power density for OBUE test for each carrier arriving at the measurement equipment connector.

##### 10.4.3.4.4 	MU assessment

###### 10.4.3.4.4.1 	MU Budget

# Table 10.4.3.4.4.1-1: Near field test range uncertainty contributors in power density pattern measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2: Near Field AAS power density pattern measurement |  |
| 1 | Axes Intersection | B4-1 |
| 2 | Axes Orthogonality | B4-2 |
| 3 | Horizontal Pointing | B4-3 |
| 4 | Probe Vertical Position | B4-4 |
| 5 | Probe H/V pointing | B4-5 |
| 6 | Measurement Distance | B4-6 |
| 7 | Amplitude and Phase Drift | B4-7 |
| 8 | Amplitude and Phase Noise | B4-8 |
| 9 | Leakage and Crosstalk | B4-9 |
| 10 | Amplitude Non-Linearity | B4-10 |
| 11 | Amplitude and Phase Shift in rotary joints | B4-11 |
| 12 | Channel Balance Amplitude and Phase | B4-12 |
| 13 | Probe Polarization Amplitude and Phase | B4-13 |
| 14 | Probe Pattern Knowledge | B4-14 |
| 15 | Multiple Reflections | B4-15 |
| 16 | Room Scattering | B4-16 |
| 17 | DUT support Scattering | B4-17 |
| 18 | Positioning | B4-21 |
| 19 | Probe Array Uniformity | B4-22 |
| 20 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | B4-23 |
| 21 | Insertion loss of receiver chain | B4-24 |
| 22 | Uncertainty of the absolute gain of the probe antenna | B4-25 |
| 23 | Measurement Equipment (spectrum Analyzer) | E |
| 24 | Measurement repeatability - Positioning Repeatability | B4-27 |
| 25 | Test System Frequency Flatness | X |
|  | Stage 1: Calibration measurement |  |
| 26 | Uncertainty of network analyser | E |
| 27 | Mismatch of receiver chain | B4-29 |
| 28 | Insertion loss of receiver chain | B4-30 |
| 29 | Mismatch in the connection of the calibration antenna | B4-31 |
| 30 | Influence of the calibration antenna feed cable | B4-32 |
| 31 | Influence of the probe antenna cable | B4-33 |
| 32 | Uncertainty of the absolute gain of the calibration antenna | E |
| 33 | Short term repeatability | B4-35 |

###### Note: Refer to TR 37.842 for description of each uncertainty term

###### 10.4.3.4.4.2 	MU Value

# Table 10.4.3.4.4.2-1: Near field test range uncertainty assessment for power density pattern measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 19 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 20 | Mismatch of receiver chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 21 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 22 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 23 | Measurement Equipment (Spectrum Analyzer) | 0.41 | 0.56 | Gaussian | 1.00 | 1 | 0.41 | 0.56 |
| 24 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| 25 | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1.00 | 1 | [0.25] | [0.25] |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 26 | Network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 27 | Mismatch of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 28 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 29 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 30 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 32 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 33 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | [0.59] | [0.70] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | [1.15] | [1.37] |

###### MU for OTA OBUE is the RSS of MU per point measurement (table 10.4.1.4.4.2-1) and TRP systematic error (Annex X):

###### MUTRP=MUper-point2+TRPSysterr2

For OTA OBUE measured in Near Field setup the MU is:

f≤3GHz

[MUTRP=0.592+0.752 =1.87dB]

3GHz≤f≤4.2GHz

[MUTRP=0.702+0.752 =2dB]