3GPP TSG-RAN WG4 Meeting #88 	R4-1811263

# Gothenburg, Sweden, 20-24 August, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.101-3 | CR | CRNum | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR TS 38.101-3: Changes to NS_04 A-MPR for Type 1 UEs to align with RAN1 Assumptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN1 decided that the RAN4 assumption that dynamic power sharing capable UEs (Type 1) get the NR allocations in time use for calculating E-UTRA A-MPR is not always be valid. Changes were needed to align the NS_04 intra-band EN-DC A-MPR with RAN1 assumptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modifications to the NS_04 A-MPR for dynamic power sharing UEs, so that equal A-MPR is is used only when the NR allocations arrive in time to be used by the E-UTRA side, otherwise E-UTRA uses A-MPR from 36.101-1 and all A-MPR is applied to NR. NR A-MPR for dyanmic power sharing UEs is currently TBD because new measurements are needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN4 assumptions for NS_04 A-MPR do not align with RAN1 assumptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3.1.2, 6.2B.3.1.2.1, 6.2B.3.1.2.2, 6.2B.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

##### 6.2B.3.1.2	A-MPR for NS_04

When the UE is configured for B41/n41 intra-band contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this subclause.

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If FIM3,low_block,low < 2490.5 MHz

Channel Configuration Case B. A-MPRIM3 defined in Subclause 6.2B.3.1.2.1

Else

Channel Configuration Case A. A-MPRIM3 defined in Subclause 6.2B.3.1.2.2

where

-	FIM3,low_block,low = (2 * Flow_channel,low_edge) – Fhigh_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently:

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA, A-MPRIM3 )

A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3 )

For UEs supporting dynamic power sharing:

If the UE receives the NR allocation in time to be considered for E-UTRA Power Control:

A-MPR = MAX(A-MPRsingle,E-UTRA, A-MPRsingle,NR, A-MPRIM3 ), with backoff applied equally to E-UTRA and NR

Else:

E-UTRA uses A-MPR as defined in TS 36.101

NR uses A-MPRIM3,NR-only = TBD

where

-	A-MPRsingle,E-UTRA is the A-MPR defined for the E-UTRA transmission in [4]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in [2]

6.2B.3.1.2.1	A-MPRIM3 for NS_04 to meet -13 dBm / 1MHz

For the UE is configured with channel configurations Case A or Case C, the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3,-13 = MA

Where MA is defined as follows

MA = 	14	;	 0 ≤ A < 0.5

9	;	 0.5 ≤ A < 1.0

7	; 	1.0 ≤ A < 2.0

5	; 	2.0 < A

Where:

For UEs supporting dynamic power sharing,

A = NRB_alloc,E-UTRA * 12* SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR

For UEs not supporting dynamic power sharing,

For E-UTRA

A = NRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR

For NR

A = 12* SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR

6.2B.3.1.2.2	A-MPR for NS_04 to meet -25 dBm / 1MHz

For the UE is configured with channel configurations Case B or Case D, the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3,-13 = MA

Where MA is defined as follows

MA = 	14	; 	0 ≤ A < 1.0

13	; 	1.0 ≤ A < 2.0

12	; 	2.0 ≤ A < 5.0

11	; 	5.0 < A

Where:

For UEs supporting dynamic power sharing,

A = NRB_alloc,E-UTRA * 12* SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR

For UEs not supporting dynamic power sharing,

For E-UTRA

A = NRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR

For NR

A = 12 * SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR

< any intermediate sub-clauses omitted >

#### 6.2B.3.2	Intra-band non-contiguous EN-DC

For intra-band EN-DC band combinations with additional requirements the A-MPR allowed are specified in Table 6.2B.3.1-1 for combinations of network signalling values indicated in E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR allowed below is in addition to the MPR requirements specified in sub-clause 6.2B.2.

Table 6.2B.3.1-1: Allowed power reduction for EN-DC

| DC configuration | Requirement (sub-clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (subclause) |
| --- | --- | --- | --- | --- |
| DC_41A_n411 | 6.6.3.3.19 and 6.6.2.2.2 of [4] and 6.5.2.3.2 and 6.5.3.3.1 of [2] | NS_04 | NS_04 | 6.2B.3.2.2 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. |  |  |  |  |

##### 6.2B.3.2.1	A-MPR for NS_04

When the UE is configured for B41/n41 intra-band non-contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this subclause.

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If AND( FIM3,low_block,high < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

Channel Configuration Case C. A-MPRIM3 defined in Subclause 6.2B.3.1.2.1

Else

Channel Configuration Case D. A-MPRIM3 defined in Subclause 6.2B.3.1.2.2

where

-	FIM3,low_block,high = (2 * Flow_channel,high_edge ) – Fhigh_channel,low_edge

-	FIM3,high_block,low = (2 * Fhigh_channel,low_edge) – Flow_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Flow_channel,high_edge is the uppermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,low_edge is the lowermost frequency of upper transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2750 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Subclause 6.5B.2.1.2.2.

The UE determines the value of A-MPRACLRoverlap as follows:

If  Wgap < BWchannel,E-UTRA + BWchannel,NR

A-MPRACLRoverlap = 4 dB

Else

A-MPRACLRoverlap = 0 dB

where

-	Wgap = Fhigh_channel,low_edge - Flow_channel,high_edge

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA, A-MPRIM3, A-MPRACLRoverlap )

A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3, A-MPRACLRoverlap )

For UEs supporting dynamic power sharing

If the UE receives the NR allocation in time to be considred for E-UTRA Power Control:

A-MPR = MAX(A-MPRsingle,E-UTRA, A-MPRsingle,NR, A-MPRIM3, A-MPRACLRoverlap ), with backoff applied equally to E-UTRA and NR

Else:

E-UTRA uses A-MPR as defined in TS 36.101

NR uses A-MPRIM3,NR-only = TBD

where

-	A-MPRsingle,E-UTRA is the A-MPR defined for the E-UTRA transmission in [4].

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in [2].

<Unchanged sections omitted>