3GPP TSG-RAN WG4 Meeting 88	R4-1811411

Göteborg, Sweden, 20 – 24 August 2018

Agenda Item:	7.13.1.6

Source:	Ericsson

Title:	Demodulation performance impacts due to simplified propagation channel models

Document for:	Discussion

# 1	Introduction

RAN4#AH1807 discussed the simplification of TDL channel models for NR UE demodulation and CSI reporting requirements and agreed with the way forward [1].

- Simplification methods - Option 1: Choose strongest paths that contribute to [X%] of total power - Option 1: X=90% for TDL-A; X=87% for TDL-C - Option 2: X = 90% or 95% for TDL-A and TDL-C - Option 2: Choose 9 paths for TDL-A and TDL-C using the frequency correlation method - TDL-A: Tap#1-6, 9, 11, 15 defined in Table 7.7.2-1 of TR 38.901 - TDL-C: Tap#1-2, 4-8, 13-14 defined in Table 7.7.2-3 of TR 38.901 - Number of path: less than or equal to 12 is acceptable from test equipment point of view

In this contribution, we compare the option 1 and option 2 with regard to the UE demodulation performance.

# 2	Simplified TDL propagation channel models

## 2.1	Simplification procedure

As RAN4#87 discussed, the resolution of delay path grids for TDL channel models in TR38.901 [2] is 1 pico-second (1ps). To avoid the complexity of the test setting, RAN4 discussed to assume the simplification of the channel model.

The simplification method consists of:

Step 1: Remove the path if the path power is less than threshold. Option 1: Choose strongest paths that contribute to [X=90,95]% of total power. Option 2: Choose [Y=9] strongest paths that contribute for NLOS PDPs. Step 2: Apply the desired delay spread after removing the weak paths. Option 1: Adjust the delay position to keep the delay spread to the desired delay spread (e.g., 30ns). Option 2: Keep the original delay position. This results the effective delay spread may be smaller than the desired delay spread. Step 3: Decide the path grid based on resolution, and round the path delay position close to the grid. Set the grid step size to [5] ns. Step 4: If two or more paths are rounded to the same grid τ, the path power at τ is set as a sum of all the path power values (in linear) rounded to τ. Step 5: Normalize the power so that the relative power of the strongest path becomes 0dB.

## 2.2	Simplified TDL-A (DS=30ns)

Figure 1 and Figure 2 compare simplification methods for the TDL-A channel model, i.e., X=90/95% or Y=9 paths. These figures show the path delay profile and frequency correlation characteristics. The difference between Figure 1 and Figure 2 is the delay position adjustment. Figure 1 is the case we adjust the delay position to keep the effective delay spread to the desired delay spread (30ns). On the other hand, Figure 2 does not adjust the delay position after trimming. Table 1 and Table 2 show the calculated delay position and their relative power values. From the figures, it is observed Figure 2 keeps the similar frequency correlation characteristics although the effective DS is smaller than 30ns.

Table 1	Tap delay and relative power for the simplified TDL-A with Desired DS=30ns. Adjust the delay position to keep DS to 30ns.

| Y = 9 paths |  | X = 90% |  | X = 95% |  |
| --- | --- | --- | --- | --- | --- |
| Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] |
| 30 | 0 | 20 | 0 | 0 | -15.4 |
| 35 | -8 | 25 | -6 | 15 | 0 |
| 40 | -10.2 | 30 | -5.2 | 20 | -6 |
| 45 | -5.2 | 35 | -7.6 | 25 | -4.3 |
| 50 | -11.9 | 95 | -8.6 | 30 | -9.5 |
| 60 | -9.5 | 125 | -12.8 | 75 | -8.6 |
| 145 | -8.6 | 150 | -13.3 | 85 | -14.4 |
|  |  |  |  | 100 | -12.8 |
|  |  |  |  | 120 | -13.3 |
|  |  |  |  | 160 | -14.7 |
|  | DS=29.6ns |  | DS=30.1ns |  | DS=30.1ns |

Table 2	Tap delay and relative power for the simplified TDL-A with Desired DS=30ns. Keep the original delay position.

| Y = 9 paths (X=86%) |  | X = 90% |  | X = 95% |  |
| --- | --- | --- | --- | --- | --- |
| Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] |
| 10 | 0 | 10 | 0 | 0 | -15.4 |
| 15 | -5.1 | 15 | -5.1 | 10 | 0 |
| 20 | -5.1 | 20 | -5.1 | 15 | -5.1 |
| 25 | -9.5 | 25 | -9.5 | 20 | -5.1 |
| 55 | -8.6 | 55 | -8.6 | 25 | -9.5 |
|  |  | 75 | -12.8 | 55 | -8.6 |
|  |  | 90 | -13.3 | 65 | -14.4 |
|  |  |  |  | 75 | -12.8 |
|  |  |  |  | 90 | -13.3 |
|  |  |  |  | 120 | -14.7 |
|  | DS=11.7ns |  | DS=18.3ns |  | DS=22.8ns |

Figure 1	Example of simplified TDL-A (Desired DS=30ns). Adjust the delay position to keep DS to 30ns.

Figure 2	Example of simplified TDL-A (Desired DS=30ns). Keep the original delay position.

## 2.3	Simplified TDL-C (DS=300ns)

We have done the same exercise for TDL-C DS=300ns case. Figure 3 and Figure 4 compare two simplification methods for TDL-C channel models with desired DS of 300ns. Table 3 and Table 4 show the path positions and their relative power values.

Table 3	Tap delay and relative power for the simplified TDL-C with Desired DS=300ns. Adjust the delay position to keep DS to 300ns.

| Y = 9 paths (X=90%) |  | X = 90% |  | X = 95% |  |
| --- | --- | --- | --- | --- | --- |
| Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] |
| 0 | -4.4 | 0 | -4.4 | 0 | -4.4 |
| 205 | -1.2 | 180 | -1.2 | 150 | -1.2 |
| 215 | 0 | 190 | -2.5 | 155 | -2.5 |
| 230 | -5.2 | 195 | -3.5 | 160 | -3.5 |
| 620 | 0 | 200 | -5.2 | 165 | -5.2 |
| 630 | -2.2 | 555 | 0 | 455 | 0 |
| 640 | -3.9 | 560 | -2.2 | 460 | -2.2 |
| 1200 | -5.1 | 570 | -3.9 | 465 | -3.9 |
|  |  | 690 | -7.1 | 470 | -7.4 |
|  |  | 1070 | -5.1 | 565 | -7.1 |
|  |  | 1135 | -6.8 | 585 | -10.7 |
|  |  |  |  | 665 | -11.1 |
|  |  |  |  | 875 | -5.1 |
|  |  |  |  | 930 | -6.8 |
|  |  |  |  | 1545 | -8.7 |
|  | DS=299.2ns |  | DS=300.3ns |  | DS=300.3ns |

Table 4	Tap delay and relative power for the simplified TDL-C with Desired DS=300ns. Keep the original delay position.

| Y = 9 paths (X=90%) |  | X = 90% |  | X = 95% |  |
| --- | --- | --- | --- | --- | --- |
| Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] |
| 0 | -6.9 | 0 | -6.9 | 0 | -6.9 |
| 65 | 0 | 65 | 0 | 65 | 0 |
| 70 | -7.7 | 70 | -7.7 | 70 | -7.7 |
| 190 | -2.5 | 190 | -2.5 | 190 | -2.5 |
| 195 | -2.4 | 195 | -2.4 | 195 | -2.4 |
| 370 | -7.6 | 200 | -9.9 | 200 | -9.9 |
|  |  | 240 | -9.6 | 240 | -9.6 |
|  |  | 370 | -7.6 | 245 | -13.2 |
|  |  | 390 | -9.3 | 280 | -13.6 |
|  |  |  |  | 370 | -7.6 |
|  |  |  |  | 390 | -9.3 |
|  |  |  |  | 650 | -11.2 |
|  | DS=92.1ns |  | DS=102.0ns |  | DS=126.4ns |

Figure 3	Example of simplified TDL-C (Desired DS=300ns). Adjust the delay position to keep DS to 300ns.

Figure 4	Example of simplified TDL-C (Desired DS=300ns). Keep the original delay position.

# 3	Simulation results

We have compared different channel models with two PDSCH demodulation configurations: QPSK and 256QAM as shown in Table 5. Figure 5 and Figure 6 show our ideal simulation results with different channel models. Note the metric PDSCH BLER is given by the number of NACKs over the number of PDSCH transmissions including up to 4 HARQ retransmissions)

For TDL-A (Figure 5), QPSK result shows X=95% and X=80% are deviated from the original PDP, and 256QAM result shows X=95% case is also deviated but not so large deviation compared with X=90% and X=81%.

For TDL-C (Figure 6), 256QAM result shows X=70% model is deviated significantly compared with the original PDP model.

Table 5	Simulation parameters for PDSCH

|  | Case 1 | Case 2 |
| --- | --- | --- |
| Propagation channel | Model 1: TDL-A DS=30ns, Doppler=10Hz Model 2: TDL-C DS=300ns, Doppler=100Hz |  |
| Frequency range | FR1 | FR1 |
| UE system bandwidth | 100MHz | 100MHz |
| SCS | 30kHz | 30kHz |
| Antenna configuration and MIMO layers | 2x2 Low, 2 layers | 2x2 Low, 2 layers |
| MCS | MCS2 (QPSK 0.33) | MCS23 (256QAM 0.75) |
| Max HARQ transmission | 4 | 4 |

Figure 5	PDSCH Simulation results for TDL-A with MCS2.

Figure 6 PDSCH 	simulation results for TDL-A with MCS23.

Figure 7	PDSCH simulation results for TDL-C with MCS2.

Figure 8	PDSCH simulation results for TDL-C with MCS23.

# 4	Conclusion

Observation 1: For TDL-A, X=95% results in the smallest performance impact regardless of DS adjustment with regard to the 70% of the maximum throughput.

Observation 2: For TDL-C, X=90% or 95% without DS adjustment results in the smallest performance impact with regard to the 70% of the maximum throughput.

# 5	References

R4-18093938, “Way Forward on Channel Model for NR UE Demodulation and CSI Requirements”, Huawei.

3GPP TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.

# Appendix

Table 7.7.2-1. TDL-A

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0.0000 | -13.4 | Rayleigh |
| 2 | 0.3819 | 0 | Rayleigh |
| 3 | 0.4025 | -2.2 | Rayleigh |
| 4 | 0.5868 | -4 | Rayleigh |
| 5 | 0.4610 | -6 | Rayleigh |
| 6 | 0.5375 | -8.2 | Rayleigh |
| 7 | 0.6708 | -9.9 | Rayleigh |
| 8 | 0.5750 | -10.5 | Rayleigh |
| 9 | 0.7618 | -7.5 | Rayleigh |
| 10 | 1.5375 | -15.9 | Rayleigh |
| 11 | 1.8978 | -6.6 | Rayleigh |
| 12 | 2.2242 | -16.7 | Rayleigh |
| 13 | 2.1718 | -12.4 | Rayleigh |
| 14 | 2.4942 | -15.2 | Rayleigh |
| 15 | 2.5119 | -10.8 | Rayleigh |
| 16 | 3.0582 | -11.3 | Rayleigh |
| 17 | 4.0810 | -12.7 | Rayleigh |
| 18 | 4.4579 | -16.2 | Rayleigh |
| 19 | 4.5695 | -18.3 | Rayleigh |
| 20 | 4.7966 | -18.9 | Rayleigh |
| 21 | 5.0066 | -16.6 | Rayleigh |
| 22 | 5.3043 | -19.9 | Rayleigh |
| 23 | 9.6586 | -29.7 | Rayleigh |

Table 7.7.2-2. TDL-B

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0.0000 | 0 | Rayleigh |
| 2 | 0.1072 | -2.2 | Rayleigh |
| 3 | 0.2155 | -4 | Rayleigh |
| 4 | 0.2095 | -3.2 | Rayleigh |
| 5 | 0.2870 | -9.8 | Rayleigh |
| 6 | 0.2986 | -1.2 | Rayleigh |
| 7 | 0.3752 | -3.4 | Rayleigh |
| 8 | 0.5055 | -5.2 | Rayleigh |
| 9 | 0.3681 | -7.6 | Rayleigh |
| 10 | 0.3697 | -3 | Rayleigh |
| 11 | 0.5700 | -8.9 | Rayleigh |
| 12 | 0.5283 | -9 | Rayleigh |
| 13 | 1.1021 | -4.8 | Rayleigh |
| 14 | 1.2756 | -5.7 | Rayleigh |
| 15 | 1.5474 | -7.5 | Rayleigh |
| 16 | 1.7842 | -1.9 | Rayleigh |
| 17 | 2.0169 | -7.6 | Rayleigh |
| 18 | 2.8294 | -12.2 | Rayleigh |
| 19 | 3.0219 | -9.8 | Rayleigh |
| 20 | 3.6187 | -11.4 | Rayleigh |
| 21 | 4.1067 | -14.9 | Rayleigh |
| 22 | 4.2790 | -9.2 | Rayleigh |
| 23 | 4.7834 | -11.3 | Rayleigh |

Table 7.7.2-3. TDL-C

| Tap # | Normalized delays | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -4.4 | Rayleigh |
| 2 | 0.2099 | -1.2 | Rayleigh |
| 3 | 0.2219 | -3.5 | Rayleigh |
| 4 | 0.2329 | -5.2 | Rayleigh |
| 5 | 0.2176 | -2.5 | Rayleigh |
| 6 | 0.6366 | 0 | Rayleigh |
| 7 | 0.6448 | -2.2 | Rayleigh |
| 8 | 0.6560 | -3.9 | Rayleigh |
| 9 | 0.6584 | -7.4 | Rayleigh |
| 10 | 0.7935 | -7.1 | Rayleigh |
| 11 | 0.8213 | -10.7 | Rayleigh |
| 12 | 0.9336 | -11.1 | Rayleigh |
| 13 | 1.2285 | -5.1 | Rayleigh |
| 14 | 1.3083 | -6.8 | Rayleigh |
| 15 | 2.1704 | -8.7 | Rayleigh |
| 16 | 2.7105 | -13.2 | Rayleigh |
| 17 | 4.2589 | -13.9 | Rayleigh |
| 18 | 4.6003 | -13.9 | Rayleigh |
| 19 | 5.4902 | -15.8 | Rayleigh |
| 20 | 5.6077 | -17.1 | Rayleigh |
| 21 | 6.3065 | -16 | Rayleigh |
| 22 | 6.6374 | -15.7 | Rayleigh |
| 23 | 7.0427 | -21.6 | Rayleigh |
| 24 | 8.6523 | -22.8 | Rayleigh |