3GPP TSG-RAN WG4 Meeting RAN4#88	R4-1811725

Gothenburg, Sweden, 20 -24 Aug, 2018

Agenda Item:	7.13.1.6

Source:	Huawei, Spirent

Title:	Way forward on channel model for NR performance requirements

Document for:	Approval

Ad-hoc attendees: [Samsung, Intel, Huawei, Qualcomm, Ericsson, NTT DoCoMo, ANRITSU, R&S, Keysight, Spirent]

# Channel model contributions

| R4-1809858 | Simplification of TDL channel models | Intel Corporation |
| --- | --- | --- |
| R4-1809859 | Channel models for FR2 demodulation requirements | Intel Corporation |
| R4-1810158 | Demodulation performance impacts due to simplified propagation channel models | Ericsson |
| R4-1810159 | Simplified propagation channel models based on TDL-D/E | Ericsson |
| R4-1810194 | Views on channel models | NTT DOCOMO, INC. |
| R4-1810345 | Simplification of TDL Channel Models | Qualcomm Europe Inc. (Spain) |
| R4-1810865 | Channel Model simplification for FR2 | Rohde & Schwarz |
| R4-1811178 | Discussion on NR FR1 channel model for UE demodulation performance and CSI requirements | Huawei, HiSilicon |
| R4-1811179 | Draft CR: FR1 channel model for UE demodulation and CSI performance requirements | Huawei, HiSilicon |
| R4-1811316 | On the selection of Option 1 vs. Option 2 for channel model definition | Keysight Technologies UK Ltd |
| R4-1811317 | LS to RAN1 on the applicability of TR 38.901 TDL channel models | Keysight Technologies UK Ltd |
| R4-1810339 | Doppler Spread for NR FR2 UE Performance Tests | Qualcomm |

# Issue 1: Simplified method for TDL model (FR1)

Previous RAN4 agreements:

- Simplification methods

- Option 1: Choose strongest paths that contribute to [X%] of total power

- Option 1: X=90% for TDL-A; X=87% for TDL-C

- Option 2: X = 90% or 95% for TDL-A and TDL-C

- Option 2: Choose 9 paths for TDL-A and TDL-C using the frequency correlation method

- TDL-A: Tap#1-6, 9, 11, 15 defined in Table 7.7.2-1 of TR 38.901

- TDL-C: Tap#1-2, 4-8, 13-14 defined in Table 7.7.2-3 of TR 38.901

- Number of path: less than or equal to 12 is acceptable from test equipment point of view

Proposals from companies:

| Intel | Proposal #1: Use simplification method of choosing strongest paths contributing to 90% of total power for TDL-A, TDL-C and 85% of total power for TDL-B. (Taps provided) |
| --- | --- |
| Ericsson | Simulation results not available |
| Qualcomm | Choose N = [11] strongest paths for each channel model to simplify the TDL channel models in TR 38.901. After choosing the N strongest paths, compute the new RMS delay spread for the pruned channel model, normalize the normalized delay of each path with the computed RMS delay spread to ensure that each channel model after pruning has an RMS delay spread of 1. |
| Huawei | Option 2 with 9 taps (with taps provided up to 12 taps) |

Way forward on channel models simplification for FR1

- Simplification methods

  - Option 1: Choose strongest paths that contribute to [X%] of total power

  - Option 2: Choose N paths for TDL-A and TDL-C using the frequency correlation method

- Use option 1 and option 2 to generate up to 12 taps after 5 ns quantization for following FR1 models

  - RMS delay spread TDL-A (30 ns)

  - RMS delay spread TDL-B (100 ns)

  - RMS delay spread TDL-C (300 ns)

- Further study the frequency correlation characteristics of channel models for Option 1 and 2

- e.g. The most suitable model is selected based on the visual inspection of the quantized FCF. The lower the correlation the better, preferably below 0.6 in full frequency range up to 100 MHz

- Make final model down-selection in RAN4 88bis

# Issue 2: Channel model for FR2

Previous RAN4 agreement:

- Option 1

- TDL PDPs can be generated from CDL taking into account Tx/Rx beamforming or existing PDPs in TE 38.901 could be reused

- Each tap is modelled based on the Jakes Doppler fading model

- Option 2

- This option considers channel model methodology with non-Jakes spectrum. Multi-path fading propagation conditions between the gNB emulator and test chamber probe is modelled as Clustered Delay Line (CDL).

- Detailed model description is provided in TR 38.810 and TR 38.901

- The model results in Non-Jakes Doppler spectrum

Working assumption: Channel Model Option 1 will be used for FR2.

Companies will provide the simulation results based on working assumption

Companies are requested to provide more input to confirm the working assumption in August meeting. And final confirmation of working assumption will be based on majority companies’ view.

Companies’ proposals:

| Intel | Proposal #1: For FR2 UE demodulation and CSI requirements define the following channel model PDPs and delay spread as: - TDL-A; 5ns DS RMS - TDL-B; 10ns DS RMS - TDL-C; 20ns DS RMS Proposal #2: For UE demodulation and CSI reporting requirements in FR2 use Max Doppler as 45Hz, 130Hz and 350Hz respectively for low, medium and high speed respectively Proposal #3: Confirm working assumption to use TDL based channel models to define UE demodulation and CSI requirements in FR2 |
| --- | --- |
| Ericsson | The difference between TDL-A/B/C and TDL-D/E is the LOS path is included or not. Considering the radio propagation characteristics, RAN4 should also consider the channel models including LOS path for FR2. (taps provided) |
| Keysight | Proposal 1: Use channel model option 2 for FR1 and FR2 If Proposal 1 is not agreed at this time: Proposal 2: Send the LS in [5] to RAN WG1 asking for clarification on the advice in TR 38.901 that TDL models using Jakes spectrum are not appropriate for MIMO scenarios Proposal 3: Study the difference from a UE perspective between the modified TDLs proposed in [3] with the channels generated using the CDL approach as received through a single probe. |
| QC | Proposal 1: Use 100 Hz (~3Km/hr) and 300Hz (~10Km/hr) Doppler spreads for NR FR2 demodulation performance tests. |

Agreement: TDL models may not necessarily characterize the spatial FR2 propagation conditions for all scenarios.

Confirm Working assumption: Channel Model Option 1 will be used for FR2.

TDL Model

- TDL-A: 30 ns RMS delay spread

- FFS if additional models are needed

  - Option 1: TDL-C: {60-80} ns RMS delay spread

  - Option 2: TDL-D: [10-30] ns RMS delay spread

  - Other models are not precluded

Doppler spread

- Low speed: 75 Hz for simulation assumptions

- High speed: 300 Hz for simulation assumptions

Channel models of simulation alignment

- Model 1: TDL-A: 30 ns 75 Hz for CSI / PDCCH/ PDSCH/ PBCH

- Model 2: TDL-A: 30 ns 300 Hz for PDCCH/ PDSCH / PBCH

- Note: use channel models without simplifications for simulation alignment in RAN4 88bis

Channel models simplification

- The maximum number of taps: [12]

- 5 ns quantization grid (200MHz sampling frequency)

- Follow same procedure as for FR1 to generate the final channel models

# Issue 3: High speed train scenarios

Previous RAN4 agreements:

    - High speed train scenarios:

- FFS maximum Doppler shift

- Need further discussion on the channel model

- FFS SCS and CP length for high speed performance requirements

Companies’ proposals:

| NTT DoCoMo | Proposal 1: For FR1, performance test should cover UE mobility of up to 500 km/h to achieve same mobility requirement as LTE. Proposal 2: For FR2, performance test should cover UE mobility of at least up to 120 km/h, accordingly. Observation 1: In order to achieve similar test coverage for LTE, high-speed test should cover followings. - TDL channel with high Doppler frequency - HST scenario - HST-SFN scenario |
| --- | --- |

Agreement: Use HST model as described on section B3 TS36.101 for FR1 for initial simulation purposes (max Doppler shift specified in table)

  - FFS introduction of new requirements

  - This topic will be discussed in agenda for PDSCH demodulation and other related agendas in the next meeting.