3GPP TSG-RAN WG4 Meeting #88bis	R4-1812493

Chengdu, China, 8 - 12 Oct, 2018

Agenda item:	7.6.2

Source:	Dish Network

Title:	UE Co-existence for Intra-band EN-DC

Document for:	Approval

# 1	Introduction

In previous meeting, some modifications to UE co-existence spurious emissions were proposed [1]. A draft revision of [1] included even more impacted bands. This paper provides our view on this topic.

# 2	Discussion

UE Spurious emission co-existence requirements are typically very stable i.e. no changes are made unless something is found to be incorrect. Hence they also work as a sort of design guide for the UE vendors.

In [1] and in its draft revision it was proposed to verify UE spurious emissions co-existence for certain protected bands only when NS values associated with the EN-DC combination are signaled (NS_35, NS_04 for EN-DC). In our understanding, the reason for this is to allow UEs to use A-MPR defined for EN-DC SEM to meet the also the UE protection limits. Note that the root reason for the definition of EN-DC A-MPR is to meet the EN-DC SEM which is significantly more difficult than meeting respective single RAT (LTE or NR) SEM. EN-DC A-MPR is allowed only when both NR and LTE NW signal an NS value. For instance in the case of DC_(n)41AA, both LTE and NR need to signal NS_04 in order to have A-MPR for EN-DC NS_04.

Our concern with proposal [1] and especially its draft revision is that there might be cases when the UE spurious emission co-existence limits are not defined. In this case the emissions at certain bands that should be protected can significantly exceed the required protection limit.

The NW can, unless otherwise specified, choose whether it signals the NS values for both NR and LTE or not. Thus the following can be concluded:

-As per current requirements, UE to UE emissions are verified regardless of whether NWs signal NS values or not.

-As per proposed modifications, UE emissions are verified only when associated NS values are signalled by the NWs

Observation 1: If the NWs do not signal the NS values associated with Intra-band EN-DC, there are no UE spurious emission co-existence requirements for certain currently protected bands which is a huge risk

In this case, the obvious question is if the NWs always signal the NS values or not. If yes, then this is less of an issue. However, if the either or both of NWs do not signal NS value, i.e are not mandated to signal NS value, then it is a huge issue because then protection is not required at all.

Observation 2: In no circumstances should usage of intra-band EN-DC lead to situations where other bands protection requirements are not always verified

Based on discussions with some FE vendors, protection of B66 in case of DC_(n)41AA and DC_41A_(n)41A and protection of B71 and B29 in case of DC_(n)71B with the current limits are feasible even without A-MPR provided by the EN-DC NS value if filters are chosed properly.

Observation 3: At least protection of some bands proposed to be verified with NS-values only could be verified also without NS value if reasonable filters are chosen

Basically, we see three alternatives going forward:

Alternative 1: Use current protection limits and verify them without NS values (current practice)

Alternative 2: Use current protection limits, verify them with NS values, and guarantee that LTE/NR NWs always signal the NS values associated with A-MPR

Alternative 3: Use current protection limits, verify them with NS values, and guarantee 2 concurrent UL is not used if both LTE and NR NS values are not signalled.

Please note that we did not even list relaxing the protection limits of bands as an alternative.

Naturally alternative 1 would be the most straightforward. Actually, our view is that when something new is built, such as Intra-band EN-DC devices, the legacy systems and bands protection requirements should not be relaxed, especially if the relaxation is justified by poor filters.

Alternative 2 would do the job, but might be problematic because if both LTE and NR NW always signal NS value, also devices operating in single RAT mode, i.e LTE or NR will receive the same NS value. It would need to be defined in the specification that both LTE and NR always signal the associated NS value.

Alternative 3 is kind of complement of alternative 2. In option 3, it is assumed that single UL UE always meets the emission limits. It would need to be defined in the specification that only single UL is allowed if both LTE and NR NW do not signal NS value.

Our preference is alternative 1 but we are open to discuss other options.

# 3	Conclusions

UE-UE co-existence for Intra-band EN-DC was considered. The following observations and alternatives were made.

Observation 1: If the NW’s do not signal the NS values associated with Intra-band EN-DC, there are no UE spurious emission co-existence requirements for certain currently protected bands which is a huge risk

Observation 2: In no circumstances usage of intra-band EN-DC should led into situation where other bands protection requirements are not always verified

Observation 3: At least protection of some band proposed to be verified with NS-values only could be verified also without NS value if reasonable filters are chosen

Alternative 1: Use current protection limits and verify them without NS values (current practice)

Alternative 2: Use current protection limits, verify them with NS values, and guarantee that LTE/NR NW’s always signal the NS values associated with A-MPR

Alternative 3: Use current protection limits, verify them with NS values, and guarantee 2 concurrent UL is not used if both LTE and NR NS values are not signalled.

Our preference is alternative 1 but we are open to discuss other options.

# 4	References

[1] R4-1810457 “Draft CR for UE-to-UE coexistence requirements for intra-band B41/n41 EN-DC with NS_04 in TS38.101-3”, LG Electronics