3GPP TSG-RAN WG4#88bis	R4-1813095

Chengdu, P.R of China, 8 – 12 October 2018

Agenda Item:	6.4.4.1

Source:	Ericsson

Title:	Simulation results for NB-IoT WUS receptions

Document for:	Discussion

# Introduction

The current release 15 NB-IoT WUS reception requirements defined in section 4.6.2.9 in [1] contains numerous TBDs. In this contribution, the simulation results for WUS reception performance for different DRX cycle lengths and coverage levels are presented with the aim to resolve the TBDs. Our companion paper in [2] contains the proposed changes to the specification based on these results.

# Simulation assumption for WUS for feNB-IoT

At last meeting, following simulation assumptions were agreed [3]

Table 1 WUS simulation assumptions for release 15 NB-IoT [3]

| Parameter | Value |
| --- | --- |
| BS TX antenna configuration | 1 Tx, 2 Tx with transmit diversity, Note: Tx diversity scheme will depend on RAN1 agreements |
| UE Rx antenna configuration | 1 Rx |
| WUS BW | 180 kHz |
| Channel model | EPA 1Hz, ETU 1 Hz |
| SNR | -6 dB, -15 dB |
| False alarm rate | 1% |
| Detection probability | 99% |

Based on the simulation study, RAN4 is to resolve the TBD values as agreed in [3]:

Table 2 Conditions for WUS reception for UE normal coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| 1.28 | TBD | TBD |
| 2.56 | TBD | TBD |
| 5.12 | TBD | TBD |
| 10.24 | TBD | TBD |

Table 3 Conditions for WUS reception for UE enhanced coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| 1.28 | TBD | TBD |
| 2.56 | TBD | TBD |
| 5.12 | TBD | TBD |
| 10.24 | TBD | TBD |

# Simulation results

## WUS transmissions

The two transmit antenna results are based on antennas switching every second subframe.

### EPA channel model

Figure 1: WUS reception performance for DRX cycle of 1.28 s with 1 Tx (left) and 2 Tx (right)

Figure 2: WUS reception performance for DRX cycle of 2.56 s with 1 Tx (left) and 2 Tx (right)

Figure 3: WUS reception performance for DRX cycle of 5.12 s with 1 Tx (left) and 2 Tx (right)

Figure 4: WUS reception performance for DRX cycle of 10.24 s with 1 Tx (left) and 2 Tx (right)

### ETU channel model

Figure 5: WUS reception performance for DRX cycle of 1.28 s with 1 Tx (left) and 2 Tx (right)

Figure 6: WUS reception performance for DRX cycle of 2.56 s with 1 Tx (left) and 2 Tx (right)

Figure 7: WUS reception performance for DRX cycle of 5.12 s with 1 Tx (left) and 2 Tx (right)

Figure 8: WUS reception performance for DRX cycle of 10.24 s with 1 Tx (left) and 2 Tx (right)

## Summary of results

The results are summarized in Table 4 based on worst case.

Table 4: Summary of results for receiving WUS in normal and enhanced coverage

| DRX cycle length [s] | Required no. of Rep. for normal coverage with Tx-Div | Required no. of Rep. for normal coverage w/o Tx-Div | Required no. of Rep. for enhanced coverage with Tx-Div | Required no. of Rep. for enhanced coverage w/o Tx-Div |
| --- | --- | --- | --- | --- |
| 1.28 | 4 | 64 | 128 | 256 |
| 2.56 | 4 | 64 | 128 | 256 |
| 5.12 | 8 | 64 | 128 | 512 |
| 10.24 | 32 | 128 | 512 | 1024 |

# Summary

In this contribution, the simulation results for minimum WUS reception performance for release 15 feNB-IoT are presented based on agreed simulation assumptions. We kindly ask the companies to consider these results to resolve the TBDs in the current tables for minimum reception requirements for WUS as agreed in [3]. Based on the results, following observations are made:

- Proposal: The TBDs in current minimum WUS reception requirements are replaced by the numbers in Table 4.

# Reference

TS 36.133, V15.3.0

R4-1813096, “Remaining work on minimum WUS reception requirements for NB-IoT”, Ericsson

R4-1811691, “WUS RRM requirements for Rel-15 NB-IoT”, Ericsson, Sony