3GPP TSG-RAN WG4 Meeting #89	R4-1815047

Spokane, US, 12th – 16th Nov, 2018

Agenda item:	7.13.1.2

Source: 		NTT DOCOMO, INC.

Title: 	Remaining issues on PDSCH demodulation requirements

Document for:	Discussion

1. Introduction

At the last meeting, the WF and simulation list were approved [1, 2]. In this contribution, we provide our views on remaining issues on PDSCH demodulation requirements.

2. Discussion

## 2.1	Type A performance requirements (soft combining verification)

In [1], the following is agreed:

- Requirement with 70% test point and 8 HARQ process for FDD and 16 HARQ process for TDD: (FFS whether needed or not) - Option 1: No tests - Option 2 (simulation purpose) - MCS #19 and rank 2 for FR1, FDD and TDD (7D1S2U) for both 2Rx and 4Rx - MCS #13 and rank 2 (2Rx) for TDD (DDDSU) FR2 - Other options not excluded - Interested companies are encourage to bring more analysis with simulation results to further decide the necessity of introducing such test cases in next RAN4 meeting.

As indicated in [3], the motivation to introduce above test cases is i) verification of normal PDSCH demodulation for higher rank/MCS with 16/8 HARQ processes for TDD/FDD, and ii) verification of soft combining for higher rank/MCS. The first motivation is very important to have enough test coverage for higher HARQ processes. The second motivation is also important since the current soft combining verification test is introduced only for low MCS/Rank, i.e. cell-edge environment. In the following, we show the impact of soft combining by link and system level evaluation. In Fig.1, we show the link evaluation results for MCS#13 and rank2 targeting for FR2. Note that channel bandwidth is 10MHz for simulation simplicity.

Fig. 1. MCS #13/Rank2 with 2Rx in FR2 (assuming 10MHz channel bandwidth)

Here, we compared the following three cases.

- Full combining: All retransmitted packets are correctly combined in retransmission case

- Limited combining:  Only the latest two packets are combined in retransmission case

- No combining: Any packet is not combined

From the results, we observed that PDSCH performance at 70%ile is degraded in No combining case compared with Full combining case, but it can be kept in Limited combining case. This means that 70% test point can distinguish No combining and others for high rank/MCS. Ideally, PDSCH performance requirement at 30%ile should be introduced to distinguish Full combining and others, i.e. to verify ideal soft combining behaviour. In reality, on the other hand, Limited combining can provide enough system performance as shown in Table 1. The detailed parameters are described in Annex. Considering the balance between performance and required soft buffer size, we propose to introduce 70%ile PDSCH demodulation requirements for higher MCS/Rank.

Table 1. System evaluation result for each combining scenario assuming dense urban@4GHz Macro scenario

Proposal 1. The following should be agreed as a package.

- Introduce FR1 and FR2 test case for 70% test point targeting for high rank and high modulation order and 16 HARQ processes for TDD and 8 HARQ processes for FDD

  - MCS #13 and rank 4 (4Rx) and MCS #19 and rank 2 (2Rx) for FDD with 8 HARQ process for FR1

  - MCS #13 and rank 4 (4Rx) and MCS #19 and rank 2 (2Rx) for TDD (7D1G2U) with 16 HARQ process for FR1

  - MCS #13 and rank 2 (2Rx) for TDD (DDDSU) with 16 HARQ process for FR2

- HARQ process for SDR requirements is 8 for TDD and 4 for FDD.

## 2.2.	ZP CSI-RS configuration

In the last meeting, following agreement was reached regarding test parameters on zero-power (ZP) CSI-RS [3].

Agreements:

  - ZP CSI-RS

      - Introducing ZP CSI-RS configuration in part of test cases FFS for demod or CSI.

Proposal 2: Apply the following ZP-CSI-RS for UE demodulation requirements.

| Parameters | Values |
| --- | --- |
| Row | 4 |
| First subcarrier index in the PRB used for CSI-RS (k0) | 0 |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 13 |
| Number of CSI-RS ports (X) | 4 |
| CDM Type | fd-CDM2 |
| Density (ρ) | 1 |
| CSI-RS periodicity | 5 slots |
| CSI-RS offset | 0 |

## 2.3	Rx beam selection and TCI signalling

In the last meeting, following agreements were reached regarding test parameters on TCI state configurations for normal demodulation test [3].

Agreements:

- FFS whether to configure CSI-RS for UE RX beam refinement

- NZP CSI-RS configuration for BM:

  - 2 ports

  - 4 resources

  - Periodicity 20 ms

  - Offset 0

  - Repetition ON

  - Subcarrier index: 0

  - OFDM symbol:

    - Resource 1: 8, Resource 2: 9, Resource 3: 10, Resource 4: 11

  - Density 1

  - CDM2

Agreements:

  - Only one TCI state configured for NR UE performance test cases in Rel-15( [QCL behavior Type A and  TypeD (FR2 only)])

      - Detailed parameters FFS

FR2 test is performed with OTA. Hence, it is necessary that UE has a DL reference to perform Rx beam selection. Corresponding TCI should be informed with DCI format 1_1.

Proposal 3: CSI-RS is configured for UE Rx beam management.

## 2.4	Dynamic UL/DL determination

RAN1 decided that the UE feature of “Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment” is mandatory without capability signalling (feature 5-1 in [3]). This means that the network assumes that ALL UEs employ this feature, and there is no way to distinguish UE with and without this capability. If RAN4 does not introduce a requirement for dynamic UL/DL determination, then UE is allowed not to employ this feature in the first release. This may not be so serious in the initial NR deployment, but will be serious in future deployment. Operator cannot configure dynamic UL/DL determination if they want to have such deployment in future since some legacy UEs not supporting this feature cannot be connected to the network. Thus, in such band, operator needs to configure UL/DL assignment via RRC/broadcast signalling to save legacy UEs. This seriously restricts future network deployment.

In addition, RAN1 performed system level analysis for dynamic UL/DL determination (called flexible duplexing) and they observed that flexible duplexing can significantly improve UL user throughput. One example in [4] is shown below. Unfortunately, inter-operator and inter-frequency interferences were not considered in this evaluation, but this feature would be still promising in future.

Proposal 4: Use dynamic UL/DL determination for some existing PDSCH demodulation test case(s).

Table A.3.1.2-1: Evaluation results for dense urban (source 1)

| Source 1 (R1-1702836), Dense urban |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ratio of DL/UL traffic | Feature | DL UPT (Mbps) |  |  |  |  |  | UL UPT (Mbps) |  |  |  |  |  |
|  |  | 5%-tile | 50%-tile | 95%-tile | Average | Served/offered packets | RU (%) | 5%-tile | 50%-tile | 95%-tile | Average | Served/offered packets | RU (%) |
| 4:1 Low load | Static TDD | 125 | 280 | 453 | 290 | 0.99 | 16.5 | 31 | 72.6 | 111 | 72.7 | 0.965 | 15.5 |
|  | Case 1-1 | 119 | 314 | 493 | 322 | 0.992 | 18.3 | 77.1 | 221 | 390 | 222 | 0.993 | 15.8 |
|  |  | -4.80% | 12.14% | 8.83% | 11.03% |  |  | 148.71% | 204.41% | 251.35% | 205.36% |  |  |
|  | Case 1-2 | 106 | 292 | 493 | 308 | 0.983 | 19.6 | 77 | 221 | 399 | 225 | 0.997 | 16.6 |
|  |  | -15.20% | 4.29% | 8.83% | 6.21% |  |  | 148.39% | 204.41% | 259.46% | 209.49% |  |  |
|  | Case 1-3 | 94.8 | 294 | 493 | 306 | 0.987 | 19.5 | 79.9 | 218 | 399 | 223 | 0.992 | 16 |
|  |  | -24.16% | 5.00% | 8.83% | 5.52% |  |  | 157.74% | 200.28% | 259.46% | 206.74% |  |  |
|  | Case 2-1 | 119 | 326 | 524 | 336 | 0.993 | 18.2 | 97 | 247 | 430 | 254 | 0.992 | 14.7 |
|  |  | -4.80% | 16.43% | 15.67% | 15.86% |  |  | 212.90% | 240.22% | 287.39% | 249.38% |  |  |
|  | Case 3-1 | 141 | 350 | 559 | 363 | 0.99 | 16.3 | 97.5 | 260 | 460 | 262 | 0.995 | 13.7 |
|  |  | 12.80% | 25.00% | 23.40% | 25.17% |  |  | 214.52% | 258.13% | 314.41% | 260.39% |  |  |
|  | Case 3-2 | 141 | 365 | 569 | 370 | 0.994 | 15.4 | 103 | 262 | 436 | 261 | 0.996 | 12.2 |
|  |  | 12.80% | 30.36% | 25.61% | 27.59% |  |  | 232.26% | 260.88% | 292.79% | 259.01% |  |  |
| 4:1 Medium load | Static TDD | 70.1 | 197 | 442 | 217 | 0.969 | 36 | 23.6 | 64 | 108 | 63.4 | 0.903 | 27.9 |
|  | Case 1-1 | 38.3 | 178 | 479 | 207 | 0.939 | 48.1 | 44.8 | 147 | 339 | 163 | 0.977 | 41.4 |
|  |  | -45.36% | -9.64% | 8.37% | -4.61% |  |  | 89.83% | 129.69% | 213.89% | 157.10% |  |  |
|  | Case 1-2 | 38.3 | 173 | 479 | 202 | 0.927 | 48.8 | 30.2 | 132 | 300 | 144 | 0.975 | 47.6 |
|  |  | -45.36% | -12.18% | 8.37% | -6.91% |  |  | 27.97% | 106.25% | 177.78% | 127.13% |  |  |
|  | Case 1-3 | 40.3 | 179 | 486 | 211 | 0.943 | 45.4 | 33.9 | 129 | 311 | 145 | 0.969 | 46 |
|  |  | -42.51% | -9.14% | 9.95% | -2.76% |  |  | 43.64% | 101.56% | 187.96% | 128.71% |  |  |
|  | Case 2-1 | 47.1 | 214 | 524 | 238 | 0.981 | 43.2 | 46.4 | 150 | 339 | 168 | 0.981 | 39.6 |
|  |  | -32.81% | 8.63% | 18.55% | 9.68% |  |  | 96.61% | 134.38% | 213.89% | 164.98% |  |  |
|  | Case 3-1 | 49.8 | 243 | 550 | 263 | 0.958 | 38 | 55 | 181 | 386 | 195 | 0.979 | 36.1 |
|  |  | -28.96% | 23.35% | 24.43% | 21.20% |  |  | 133.05% | 182.81% | 257.41% | 207.57% |  |  |
|  | Case 3-2 | 54.1 | 245 | 550 | 266 | 0.971 | 36.8 | 52.3 | 170 | 365 | 187 | 0.982 | 34.8 |
|  |  | -22.82% | 24.37% | 24.43% | 22.58% |  |  | 121.61% | 165.63% | 237.96% | 194.95% |  |  |
| Note (interference mitigation/cancellation schemes, evaluation assumption, etc): - Interference mitigation schemes - At the transmitter, fixed analog beamforming and SVD precoding is applied. - At the receiver, MMSE-IRC receiver is applied. - Ideal channel estimation - FTP model 1 with 0.5Mbytes |  |  |  |  |  |  |  |  |  |  |  |  |  |

3. Conclusion

In this contribution, we discuss the remaining issues on PDSCH demodulation requirement. Our proposals are summarized below.

Proposal 1: The following should be agreed as a package.

- Introduce FR1 and FR2 test case for 70% test point targeting for high rank and high modulation order and 16 HARQ processes for TDD and 8 HARQ processes for FDD

  - MCS #13 and rank 4 (4Rx) and MCS #19 and rank 2 (2Rx) for FDD with 8 HARQ process for FR1

  - MCS #13 and rank 4 (4Rx) and MCS #19 and rank 2 (2Rx) for TDD (7D1G2U) with 16 HARQ process for FR1

  - MCS #13 and rank 2 (2Rx) for TDD (DDDSU) with 16 HARQ process for FR2

- HARQ process for SDR requirements is 8 for TDD and 4 for FDD.

Proposal 2: Apply the following ZP-CSI-RS for UE demodulation requirements.

| Parameters | Values |
| --- | --- |
| Row | 4 |
| First subcarrier index in the PRB used for CSI-RS (k0) | 0 |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 13 |
| Number of CSI-RS ports (X) | 4 |
| CDM Type | fd-CDM2 |
| Density (ρ) | 1 |
| CSI-RS periodicity | 5 slots |
| CSI-RS offset | 0 |

Proposal 3: CSI-RS is configured for UE Rx beam management.

Proposal 4: Use dynamic UL/DL determination for some existing PDSCH demodulation test case(s).

References

- R4-1814238	Way forward on NR PDSCH demodulation performance requirements, Oct. 2018

- R4-1814239	Simulation assumptions for NR PDSCH, Oct. 2018

- R1-1812064	RAN1 NR UE features, Oct. 2018.

- TR38.802,	Study on new radio access technology Physical layer aspects, Sept. 2017.

Annex

System evaluation assumption in the section 2.1 (this is based on dense urban scenario in [4])