3GPP TSG-RAN WG4 Meeting #89	R4-1815783

Spokane, US, 12 - 16 November 2018

Agenda item:	7.13.1.4

Title: 	Discussion on NR performance of PMI test

Source: 	Huawei, Hisilicon

Document for:	Discussion

Introduction

Base on the previous agreements and the way forward of R4-1811693, companies are suggested to provide simulation results under the assumptions below.

In this contribution, we provide our simulation results of PMI test.

Initial Simulation assumption

For PMI (Wideband)

Table 1 lists the common parameters for the PMI (Wideband) test cases.

Table 1 PMI Wideband Test parameters

| Parameter | Unit | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | 7D1S2U | DDDSU |
| Propagation channel |  | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | TDL_A 30ns, 75Hz |
| Correlation and antenna configuration |  | 8*2 8*4 [XP spatially High] | 8*2 8*4 [XP spatially High] | 2*2 [ULA Low] |
| CSI-RS resource setting (aperiodic) |  |  |  |  |
| resourceConfigType |  | aperiodic | aperiodic | aperiodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | 5slot 1 slot offset | 5slot 1 slot offset | 5slot 1 slot offset |
| NZP CSI-RS ports |  | 8 port {3000…3007} | 8 port {3000…30007} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | [CDM4,density 1 (row 9)] | [CDM4,density 1 (row 9)] | [FD-CDM2/density 1] |
| CSI-IM configuration |  | FFS | FFS | FFS |
| CSI report setting (aperiodic) |  |  |  |  |
| reportConfigType |  | aperiodic | aperiodic | aperiodic |
| Reporting periodicity and slot offset |  | FFS | FFS | FFS |
| (N1,N2) |  | (2,2) | (2,2) | N.A |
| (O1,O2) |  | (4,4) | (4,4) | N.A |
| CodebookMode |  | 1 | 1 | N.A |
| Codebook Restriction |  | FFS | FFS | FFS |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Rank |  | 2 | 2 | 1 |
| FRC |  | MCS13 | MCS13 | MCS13 |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

For PMI (Sub-band)

Table 2 PMI Sub-band Test parameters

| Parameter | Unit | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) |  |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz |  | 40MHz/30kHz |
| Transmission Scheme |  | 1 |  | 1 |
| Duplex Mode |  | FDD |  | TDD |
| DLUL configuration |  | N.A |  | 7D1S2U |
| Propagation channel |  | TDL_C 100ns, 5Hz |  | TDL_C 100ns, 10Hz |
| Correlation and antenna configuration |  | 4*2 4*4 [XP spatially High] |  | 4*2 4*4 [XP spatially High] |
| CSI-RS resource setting (aperiodic) |  |  |  |  |
| resourceConfigType |  | aperiodic |  | aperiodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | 5slots, 1 slot offset |  | 5slotss, 1 slot offset |
| NZP CSI-RS ports |  | 4 port {3000…3003} |  | 4 port {3000…30003} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | [FD-CDM2 density 1 (row 4)] |  | [FD-CDM2 density 1 (row 4)] |
| CSI-IM configuration |  | FFS |  | FFS |
| CSI report setting (aperiodic) |  |  |  |  |
| reportConfigType |  | aperiodic |  | aperiodic |
| Reporting periodicity and slot offset |  | FFS |  | FFS |
| (N1,N2) |  | (2,1) |  | (2,1) |
| (O1,O2) |  | (4,-) |  | (4,-) |
| CodebookMode |  | 1 |  | 1 |
| Codebook Restriction |  | FFS |  | FFS |
| Reporting granularity |  | Sub-band |  | Sub-band |
| Rank |  | 1 |  | 1 |
| FRC |  | MCS13 |  | MCS13 |
| Physical channel for reporting |  | PUSCH |  | PUSCH |
| PUCCH reporting format |  | FFS |  | FFS |
| CQI table |  | Table 2 with 256QAM |  | Table 2 with 256QAM |

Simulation results

TBD

Conclusion

In this contribution, we provide our simulation results of PMI test.

Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018