3GPP TSG-RAN WG4 Meeting #90                 	R4-1900093

Athens, Greece, 25 Feb – 1 Mar 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PRACH simulation results

Agenda item:	6.14.2.4.1

Document for:	Discussion

Introduction

In the RAN4#88bis meeting, a way forward was agreed for NR PRACH demodulation in [1]. In this contribution we provide simulation results with the simulation assumptions agreed in the WF.
A procedure to arrive at performance requirement SNR numbers from the contributions of all companies was furthermore agreed in [3].
In this contribution we add the impaired counterparts to our previously delivered ideal results [4].

Simulation Results (ideal)

Simulation Parameters

In the following we will provide simulation results based on the simulation assumptions as agreed earlier for NR PRACH in [1].

Table 1: Simulation Parameter for NR PRACH

|  |  |  | Ncs | v | Frequency Offset (Hz) | Channel type |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 4 GHz | Format 0 | 1.25kHz |  | 32 | 400 | AWGN and TDL |
|  | Format A1 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format A2 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format A3 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format B4 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format C0 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format C2 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
| FR2 30 GHz | Format A1 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format A2 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format A3 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format B4 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format C0 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format C2 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |

Further parameters and details:

Propagation condition details:

FR1 PRACH: AWGN, TDL-C 300ns, 100 Hz

FR2 PRACH: AWGN, TDL-A 30ns, 300Hz

System Bandwidth [2]:

FR1: 10MHz/15kHz; 40MHz/30kHz; 40MHz/1,25kHz

FR2: 100MHz/60kHz; 100MHz/120kHz

Target false alarm probability = 0.05%

Simulation results PRACH FR1 1T2R

Simulation results for PRACH FR1 1T2R, based on parameters in Table 1 are shown below in Table 2.

Table 2: Simulation results (impaired) for PRACH FR1 1T2R

| PRACH FR1 1T2R |  | Propagation condition | Frequency offset (Hz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -16,35 |
|  |  | TDLC300-100 | 400 | -9,03 |
| Format A1 | 15kHz | AWGN | 0 | -11,73 |
|  |  | TDLC300-100 | 400 | -4,29 |
|  | 30kHz | AWGN | 0 | -11,51 |
|  |  | TDLC300-100 | 400 | -4,94 |
| Format A2 | 15kHz | AWGN | 0 | -14,72 |
|  |  | TDLC300-100 | 400 | -6,89 |
|  | 30kHz | AWGN | 0 | -14,44 |
|  |  | TDLC300-100 | 400 | -8,28 |
| Format A3 | 15kHz | AWGN | 0 | -16,14 |
|  |  | TDLC300-100 | 400 | -8,70 |
|  | 30kHz | AWGN | 0 | -15,73 |
|  |  | TDLC300-100 | 400 | -9,36 |
| Format B4 | 15kHz | AWGN | 0 | -18,74 |
|  |  | TDLC300-100 | 400 | -11,57 |
|  | 30kHz | AWGN | 0 | -18,50 |
|  |  | TDLC300-100 | 400 | -11,92 |
| Format C0 | 15kHz | AWGN | 0 | -8,70 |
|  |  | TDLC300-100 | 400 | -1,35 |
|  | 30kHz | AWGN | 0 | -8,44 |
|  |  | TDLC300-100 | 400 | -2,17 |
| Format C2 | 15kHz | AWGN | 0 | -14,80 |
|  |  | TDLC300-100 | 400 | -7,26 |
|  | 30kHz | AWGN | 0 | -14,38 |
|  |  | TDLC300-100 | 400 | -8,03 |

Simulation results PRACH FR1 1T4R

Simulation results for PRACH FR1 1T4R, based on parameters in Table 1 are shown below in Table 3.

Table 3: Simulation results (impaired) for PRACH FR1 1T4R

| PRACH FR1 1T4R |  | Propagation condition | Frequency offset (Hz) | SNR[dB] |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -18,77 |
|  |  | TDLC300-100 | 400 | -14,56 |
| Format A1 | 15kHz | AWGN | 0 | -14,27 |
|  |  | TDLC300-100 | 400 | -9,26 |
|  | 30kHz | AWGN | 0 | -14,22 |
|  |  | TDLC300-100 | 400 | -9,38 |
| Format A2 | 15kHz | AWGN | 0 | -17,40 |
|  |  | TDLC300-100 | 400 | -12,30 |
|  | 30kHz | AWGN | 0 | -16,93 |
|  |  | TDLC300-100 | 400 | -12,43 |
| Format A3 | 15kHz | AWGN | 0 | -18,52 |
|  |  | TDLC300-100 | 400 | -13,35 |
|  | 30kHz | AWGN | 0 | -18,17 |
|  |  | TDLC300-100 | 400 | -13,62 |
| Format B4 | 15kHz | AWGN | 0 | -21,15 |
|  |  | TDLC300-100 | 400 | -16,00 |
|  | 30kHz | AWGN | 0 | -20,71 |
|  |  | TDLC300-100 | 400 | -16,20 |
| Format C0 | 15kHz | AWGN | 0 | -11,35 |
|  |  | TDLC300-100 | 400 | -6,40 |
|  | 30kHz | AWGN | 0 | -11,01 |
|  |  | TDLC300-100 | 400 | -6,59 |
| Format C2 | 15kHz | AWGN | 0 | -17,32 |
|  |  | TDLC300-100 | 400 | -12,07 |
|  | 30kHz | AWGN | 0 | -17,00 |
|  |  | TDLC300-100 | 400 | -12,65 |

Simulation results PRACH FR1 1T8R

Simulation results for PRACH FR1 1T8R, based on parameters in Table 1 are shown below in Table 4.

Table 4: Simulation results (impaired) for PRACH FR1 1T8R

| PRACH FR1 1T8R |  | Propagation condition | Frequency offset (Hz) | SNR[dB] |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -21,09 |
|  |  | TDLC300-100 | 400 | -18,69 |
| Format A1 | 15kHz | AWGN | 0 | -16,78 |
|  |  | TDLC300-100 | 400 | -13,69 |
|  | 30kHz | AWGN | 0 | -16,57 |
|  |  | TDLC300-100 | 400 | -13,14 |
| Format A2 | 15kHz | AWGN | 0 | -19,75 |
|  |  | TDLC300-100 | 400 | -16,32 |
|  | 30kHz | AWGN | 0 | -19,44 |
|  |  | TDLC300-100 | 400 | -16,00 |
| Format A3 | 15kHz | AWGN | 0 | -20,82 |
|  |  | TDLC300-100 | 400 | -17,41 |
|  | 30kHz | AWGN | 0 | -20,45 |
|  |  | TDLC300-100 | 400 | -17,02 |
| Format B4 | 15kHz | AWGN | 0 | -23,27 |
|  |  | TDLC300-100 | 400 | -19,81 |
|  | 30kHz | AWGN | 0 | -22,90 |
|  |  | TDLC300-100 | 400 | -19,27 |
| Format C0 | 15kHz | AWGN | 0 | -13,77 |
|  |  | TDLC300-100 | 400 | -10,57 |
|  | 30kHz | AWGN | 0 | -13,54 |
|  |  | TDLC300-100 | 400 | -10,09 |
| Format C2 | 15kHz | AWGN | 0 | -19,75 |
|  |  | TDLC300-100 | 400 | -16,50 |
|  | 30kHz | AWGN | 0 | -19,47 |
|  |  | TDLC300-100 | 400 | -15,93 |

Simulation results PRACH FR2 1T2R

Simulation results for PRACH FR2 1T2R, based on parameters in Table 1 are shown below in Table 5.

Table 5: Simulation results (impaired) for PRACH FR2 1T2R

| PRACH FR2 1T2R |  | Propagation condition | Frequency offset (Hz) | SRN[dB] |
| --- | --- | --- | --- | --- |
| Format A1 | 60kHz | AWGN | 0 | -11,51 |
|  |  | TDLA30-300 | 4000 | -2,40 |
|  | 120kHz | AWGN | 0 | -11,39 |
|  |  | TDLA30-300 | 4000 | -3,04 |
| Format A2 | 60kHz | AWGN | 0 | -14,23 |
|  |  | TDLA30-300 | 4000 | -4,64 |
|  | 120kHz | AWGN | 0 | -14,19 |
|  |  | TDLA30-300 | 4000 | -5,92 |
| Format A3 | 60kHz | AWGN | 0 | -15,68 |
|  |  | TDLA30-300 | 4000 | -6,32 |
|  | 120kHz | AWGN | 0 | -15,63 |
|  |  | TDLA30-300 | 4000 | -7,28 |
| Format B4 | 60kHz | AWGN | 0 | -18,22 |
|  |  | TDLA30-300 | 4000 | -9,00 |
|  | 120kHz | AWGN | 0 | -18,29 |
|  |  | TDLA30-300 | 4000 | -9,89 |
| Format C0 | 60kHz | AWGN | 0 | -8,16 |
|  |  | TDLA30-300 | 4000 | 0,36 |
|  | 120kHz | AWGN | 0 | -8,23 |
|  |  | TDLA30-300 | 4000 | 0,14 |
| Format C2 | 60kHz | AWGN | 0 | -14,23 |
|  |  | TDLA30-300 | 4000 | -4,52 |
|  | 120kHz | AWGN | 0 | -14,26 |
|  |  | TDLA30-300 | 4000 | -5,95 |

Simulation Results (impaired)

Simulation Parameters

The simulation parameters are as before in the ideal case. To obtain the impaired result non-ideal effects such as, but not limited to, phase noise and fixed-point implementation, have been considered.

Simulation results PRACH FR1 1T2R

Simulation results for PRACH FR1 1T2R, based on parameters in Table 1 are shown below in Table 6.

Table 6: Simulation results (impaired) for PRACH FR1 1T2R

| PRACH FR1 1T2R |  | Propagation condition | Frequency offset (Hz) | SNR[dB] |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -13,85 |
|  |  | TDLC300-100 | 400 | -6,53 |
| Format A1 | 15kHz | AWGN | 0 | -9,27 |
|  |  | TDLC300-100 | 400 | -1,79 |
|  | 30kHz | AWGN | 0 | -8,89 |
|  |  | TDLC300-100 | 400 | -2,44 |
| Format A2 | 15kHz | AWGN | 0 | -12,22 |
|  |  | TDLC300-100 | 400 | -4,39 |
|  | 30kHz | AWGN | 0 | -11,94 |
|  |  | TDLC300-100 | 400 | -5,78 |
| Format A3 | 15kHz | AWGN | 0 | -13,64 |
|  |  | TDLC300-100 | 400 | -6,20 |
|  | 30kHz | AWGN | 0 | -13,23 |
|  |  | TDLC300-100 | 400 | -6,86 |
| Format B4 | 15kHz | AWGN | 0 | -16,24 |
|  |  | TDLC300-100 | 400 | -9,07 |
|  | 30kHz | AWGN | 0 | -16 |
|  |  | TDLC300-100 | 400 | -9,42 |
| Format C0 | 15kHz | AWGN | 0 | -6,2 |
|  |  | TDLC300-100 | 400 | 1,15 |
|  | 30kHz | AWGN | 0 | -5,94 |
|  |  | TDLC300-100 | 400 | 0,33 |
| Format C2 | 15kHz | AWGN | 0 | -12,3 |
|  |  | TDLC300-100 | 400 | -4,76 |
|  | 30kHz | AWGN | 0 | -11,88 |
|  |  | TDLC300-100 | 400 | -5,53 |

Simulation results PRACH FR1 1T4R

Simulation results for PRACH FR1 1T4R, based on parameters in Table 1 are shown below in Table 7.

Table 7: Simulation results (impaired) for PRACH FR1 1T4R

| PRACH FR1 1T4R |  | Propagation condition | Frequency offset (Hz) | SNR[dB] |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -16,27 |
|  |  | TDLC300-100 | 400 | -12,06 |
| Format A1 | 15kHz | AWGN | 0 | -11,77 |
|  |  | TDLC300-100 | 400 | -6,76 |
|  | 30kHz | AWGN | 0 | -11,72 |
|  |  | TDLC300-100 | 400 | -6,88 |
| Format A2 | 15kHz | AWGN | 0 | -14,90 |
|  |  | TDLC300-100 | 400 | -9,80 |
|  | 30kHz | AWGN | 0 | -14,43 |
|  |  | TDLC300-100 | 400 | -9,93 |
| Format A3 | 15kHz | AWGN | 0 | -16,02 |
|  |  | TDLC300-100 | 400 | -10,85 |
|  | 30kHz | AWGN | 0 | -15,67 |
|  |  | TDLC300-100 | 400 | -11,12 |
| Format B4 | 15kHz | AWGN | 0 | -18,65 |
|  |  | TDLC300-100 | 400 | -13,50 |
|  | 30kHz | AWGN | 0 | -18,21 |
|  |  | TDLC300-100 | 400 | -13,70 |
| Format C0 | 15kHz | AWGN | 0 | -8,85 |
|  |  | TDLC300-100 | 400 | -3,90 |
|  | 30kHz | AWGN | 0 | -8,51 |
|  |  | TDLC300-100 | 400 | -4,09 |
| Format C2 | 15kHz | AWGN | 0 | -14,82 |
|  |  | TDLC300-100 | 400 | -9,57 |
|  | 30kHz | AWGN | 0 | -14,50 |
|  |  | TDLC300-100 | 400 | -10,15 |

Simulation results PRACH FR1 1T8R

Simulation results for PRACH FR1 1T8R, based on parameters in Table 1 are shown below in Table 8.

Table 8: Simulation results (impaired) for PRACH FR1 1T8R

| PRACH FR1 1T8R |  | Propagation condition | Frequency offset (Hz) | SNR[dB] |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -18,59 |
|  |  | TDLC300-100 | 400 | -16,19 |
| Format A1 | 15kHz | AWGN | 0 | -14,28 |
|  |  | TDLC300-100 | 400 | -11,19 |
|  | 30kHz | AWGN | 0 | -14,07 |
|  |  | TDLC300-100 | 400 | -10,64 |
| Format A2 | 15kHz | AWGN | 0 | -17,25 |
|  |  | TDLC300-100 | 400 | -13,82 |
|  | 30kHz | AWGN | 0 | -16,94 |
|  |  | TDLC300-100 | 400 | -13,50 |
| Format A3 | 15kHz | AWGN | 0 | -18,32 |
|  |  | TDLC300-100 | 400 | -14,91 |
|  | 30kHz | AWGN | 0 | -17,95 |
|  |  | TDLC300-100 | 400 | -14,52 |
| Format B4 | 15kHz | AWGN | 0 | -20,77 |
|  |  | TDLC300-100 | 400 | -17,31 |
|  | 30kHz | AWGN | 0 | -20,40 |
|  |  | TDLC300-100 | 400 | -16,77 |
| Format C0 | 15kHz | AWGN | 0 | -11,27 |
|  |  | TDLC300-100 | 400 | -8,07 |
|  | 30kHz | AWGN | 0 | -11,04 |
|  |  | TDLC300-100 | 400 | -7,59 |
| Format C2 | 15kHz | AWGN | 0 | -17,25 |
|  |  | TDLC300-100 | 400 | -14,00 |
|  | 30kHz | AWGN | 0 | -16,97 |
|  |  | TDLC300-100 | 400 | -13,43 |

Simulation results PRACH FR2 1T2R

Simulation results for PRACH FR2 1T2R, based on parameters in Table 1 are shown below in Table 9.

Table 9: Simulation results (impaired) for PRACH FR2 1T2R

| PRACH FR2 1T2R |  | Propagation condition | Frequency offset (Hz) | SRN[dB] |
| --- | --- | --- | --- | --- |
| Format A1 | 60kHz | AWGN | 0 | -7,66 |
|  |  | TDLA30-300 | 4000 | 0,70 |
|  | 120kHz | AWGN | 0 | -7,68 |
|  |  | TDLA30-300 | 4000 | 0,06 |
| Format A2 | 60kHz | AWGN | 0 | -11,13 |
|  |  | TDLA30-300 | 4000 | -1,54 |
|  | 120kHz | AWGN | 0 | -11,09 |
|  |  | TDLA30-300 | 4000 | -2,82 |
| Format A3 | 60kHz | AWGN | 0 | -12,58 |
|  |  | TDLA30-300 | 4000 | -3,22 |
|  | 120kHz | AWGN | 0 | -12,53 |
|  |  | TDLA30-300 | 4000 | -4,18 |
| Format B4 | 60kHz | AWGN | 0 | -15,12 |
|  |  | TDLA30-300 | 4000 | -5,90 |
|  | 120kHz | AWGN | 0 | -15,19 |
|  |  | TDLA30-300 | 4000 | -6,79 |
| Format C0 | 60kHz | AWGN | 0 | -5,06 |
|  |  | TDLA30-300 | 4000 | 3,46 |
|  | 120kHz | AWGN | 0 | -5,13 |
|  |  | TDLA30-300 | 4000 | 3,24 |
| Format C2 | 60kHz | AWGN | 0 | -11,13 |
|  |  | TDLA30-300 | 4000 | -1,42 |
|  | 120kHz | AWGN | 0 | -11,16 |
|  |  | TDLA30-300 | 4000 | -2,85 |

Conclusion

In this contribution we have provided simulation results for NR PRACH.

- References

- R4-1813757, Way forward on NR PRACH demodulation requirements, Ericsson, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018

- R4-1813758, Work plan for NR BS demodulation performance requirements, China Telecom, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018

- R4-1816594, WF for test case SNR derivation procedure, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#89, Spokane, US, 12 – 16 November 2018.

- R4-1815351 NR PRACH simulation results, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#89, Spokane, US, 12 – 16 November 2018.