3GPP TSG-RAN WG4#90 Meeting	R4-1900106

Athens, Greece, 25 Feb. – 1 Mar. 2018

Agenda item:	5.11.2.

Title: 	PDSCH simulation results and discussion for 8Rx UEs

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN Plenary Meeting #78, the Rel.15 WI “UE requirements for LTE DL 8Rx antenna ports” has been approved and its WID is included in [1]. In the scope of the WID [1], there is part related to RAN4 UE demodulation/CSI performance tests as shown below:

- For UE demodulation/CSI requirements

  - Define channel model for downlink 8Rx antennas

    - Specify the antenna configuration and MIMO channel correlation matrices for 8Rx antennas;

    - Specify the static channel model;

  - Define test cases for the rank lower than or equal to 4.

    - Adopt the test cases studied in SI to define requirements.

  - Define test cases for the rank higher than 4 in fading channel.

    - Base on combinations of rank and MCS that can achieve the maximum configured throughput.

  - Define SDR test for 8Rx in WI.

  - Define CSI test for 8Rx.

  - No PDCCH/PCFICH demodulation requirement is expected for 8Rx.

- Considering the test coverage of 8Rx, test applicability rule is needed to define

  - Define applicability rule of existing performance requirements for 8Rx capable UEs.

In RAN4#86bis Meeting, a WF [2] was agreed on the PDSCH demodulation tests for 8Rx UE as follows:

  - For test cases of ranks lower than or equal to 4, introduce:

TM2: Rank 1, 16QAM 1/2, EVA5, 2x8 medium correlation B

TM3: Rank 2, 16QAM 1/2, EVA70, 2x8 low

  - FFS: Whether to introduce rank 5~8 demodulation performance tests

In RAN4#88 Meeting, a WF [3] was agreed on the PDSCH demodulation tests for 8Rx UE as follows:

  - Introduce 8Rx demodulation test(s)

TM9

Rank higher than four, including at least one rank from {6, 8}

16QAM

EPA5

In RAN4#89 Meeting, a WF [4] was agreed on the PDSCH demodulation tests for 8Rx UE in TDD modes. In this contribution, we provide our simulation results for TDD TM2, TM3 and TM9 PDSCH demodulation performance.

# Discussion

## Simulation results and requirement proposals for PDSCH test cases with ranks lower than or equal to 4

Table 1 lists the simulation parameters for TM2 and TM3 test cases.

Table 1 Simulation parameters for TM2 and TM3 test cases

| Test number | Transmission mode | Bandwidth and MCS | Reference channel | Propagation condition | Correlation matrix and antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | TM2 | 10 MHz 16QAM,1/2 | R.11 TDD | EVA5 | 2x8 Low Medium correlation B, ULA |
| 2 | TM3 | 10 MHz 16QAM,1/2 | R.11 TDD | EVA70 | 2x8 Low |

Figure 1 and 2 illustrate the simulation results for the above two test cases without considering any impairment margins.

Figure 1 Simulation results for TM2

Observation 1: For TM2 test case, the 70% of maximum configured throughput is reached at -2.2dB.

Proposal 1: For TM2 test case, the PDSCH demodulation requirement is set to -2.2dB without considering impairment margins.

Figure 2 Simulation results for TM3

Observation 2: For TM3 test case, the 70% of maximum configured throughput is reached at 1.8dB.

Proposal 2: For TM3 test case, the PDSCH demodulation requirement is set to 1.8dB without considering impairment margins.

## Simulation results and views on PDSCH demodulation performance requirements for ranks higher than 4

Table 2 lists the simulation parameters for TM9 test cases with 16QAM. More simulation assumption details can be found in [4].

Table 2 Simulation parameters for TM9 test cases

| TM9 | 10 MHz 16QAM, 1/2 | R.xx TDD | EPA5 | 8x8 Low |
| --- | --- | --- | --- | --- |

Figure 3 illustrate the simulation results for the above TM9 test cases with 8×8 antenna configuration and 16QAM modulation, and without considering any impairment margins.

Figure 3 PDSCH TM9 simulation results with 8×8 antenna configuration, 16QAM and Rank8

Observation 3: For TM9 test case, the maximum configured throughput is reached at 22dB, and 70% of maximum configured throughput is reached at 17.3dB.

Proposal 3: For TM9 test case, the PDSCH demodulation requirement is set to 17.3dB without considering impairment margins.

# Conclusion

In this contribution, we first provide our updated simulation results for PDSCH demodulation performance and then share our views on the 8Rx capable UE demodulation requirement.

Observation 1: For TM2 test case, the 70% of maximum configured throughput is reached at -2.2dB.

Proposal 1: For TM2 test case, the PDSCH demodulation requirement is set to -2.2dB without considering impairment margins.

Observation 2: For TM3 test case, the 70% of maximum configured throughput is reached at 1.8dB.

Proposal 2: For TM3 test case, the PDSCH demodulation requirement is set to 1.8dB without considering impairment margins.

Observation 3: For TM9 test case, the maximum configured throughput is reached at 22dB, and 70% of maximum configured throughput is reached at 17.3dB.

Proposal 3: For TM9 test case, the PDSCH demodulation requirement is set to 17.3dB without considering impairment margins.

# Reference

[1] R4-172842, “New WI proposal: UE requirements for LTE DL 8Rx antenna ports”, Huawei, HiSilicon, Dec. 2017

[2] R4-1805552, “Way forward on 8Rx demodulation requirements”, Huawei, April 2018

[3] R4-1811868, “Way forward on 8Rx demodulation and CSI tests”, Huawei, August 2018

[4] R4-1816383, “Way forward on 8Rx demodulation and CSI tests”, Huawei, November 2018