3GPP TSG-RAN WG4 Meeting #90	 	R4-1900119

Athens, GR, 25th Feb – 1st March, 2019

Agenda item:       6.14.1.3

Source:                Intel Corporation

Title:	                     NR PBCH UE Performance Requirements

Document for:     Discussion

# Introduction

In RAN4#89 NR PBCH requirements were discussed and way forward [1] was approved. Companies were encouraged to provide alignment and impairment results on the agreed test cases in order to define NR PBCH performance requirements.

The open items and proposed way forward related to NR PBCH requirements are captured below.

Necessity of 4Rx PBCH test cases - Option 1: introducing 4Rx PBCH test cases - Option 2: not introducing 4Rx PBCH test cases Proposal: - Further discuss whether to introduce 4Rx PBCH test cases after collecting results from more companies in next meeting - If 4Rx PBCH requirements is introduced, - whether to apply 4Rx PBCH requirements is up to UE declaration - Note the PBCH demodulation requirements do not need to be tested in RAN5 as same as LTE PBCH demodulation requirements. (i.e. no conformance requirements) UE assumption on DMRS SSB index acquisition for PBCH demodulation - Option 1: Derive requirements assuming without knowledge of SSB index - Option 2: Derive two kinds of requirements with and without knowledge of SSB index Proposal: - Introduce PBCH requirements assuming without knowledge of SSB index. - PBCH requirements is derived with the combination of SSB index detection probability and MIB decoding probability - Whether to introduce requirements with knowledge of SSB index will be further discussed after collecting results from more companies in next meeting

In this contribution we provide simulation results for NR PBCH test cases and views on PBCH demodulation requirements.

# Simulation Results

## Simulation Assumptions

The simulation parameters for PBCH test cases agreed in [1] are summarized below.

Table 1: Common parameters for PBCH test cases

| Parameters | Unit | Value |
| --- | --- | --- |
| Cell ID |  | 0 |
| CP length |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index |  | 0 |
| SS burst periodicity | ms | 20 |
| MIB size | bits | 24 |
| PBCH payload size (including 24 bits CRC and 8 bits of SS/PBCH location information) | bits | 56 |
| Receiver assumption |  | LMMSE |

Table 2: Test cases for PBCH Demodulation

| Parameters | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Frequency Range | FR1 | FR1 | FR2 | FR2 |
| Channel Bandwidth | 10MHz | 40MHz | 100MHz | 100MHz |
| SS/PBCH SCS | 15kHz | 30kHz | 120kHz | 240kHz |
| SS Block pattern (Note 1) | Case A | Case B Case C | Case D | Case E |
| Antenna Configuration | 1x2 1x4 | 1x2 1x4 | 1x2 | 1x2 |
| Propagation channel | TDLC300-100 | TDLA30-10 | TDLA30-300 | TDLA30-75 |
| Assumption on DMRS SSB index acquisition | a: SS/PBCH block index is known b: SS/PBCH block index is not known |  |  |  |

The UE combines PBCH symbols for the same SSB index within the MIB TTI.

## FR1 Results

In this section we present alignment results for FR1 test cases with 2Rx and 4Rx.

| 2Rx | 4Rx |
| --- | --- |

Figure 1: PBCH Simulation results in FR1

The tables capture the SNR (dB) @ 1% Pm-bch. The simulations are run with and without PBCH-DMRS sequence detection.

Table 3: Alignment results for PBCH in FR1 with 2Rx

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | With DMRS Seq Det | Without DMRS Seq Det |
| Test 1 | 10MHz | 15kHz | Case A | 1x2 | TDL-C-300; 100Hz | -8.9 | -9.8 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x2 | TDL-A-30; 10Hz | -7.1 | -7.6 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x2 | TDL-A-30; 10Hz | -7.1 | -7.7 |

Table 4: Alignment results for PBCH in FR1 with 4Rx

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | With DMRS Seq Det | Without DMRS Seq Det |
| Test 1 | 10MHz | 15kHz | Case A | 1x4 | TDL-C-300; 100Hz | -11.7 | -12.2 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x4 | TDL-A-30; 10Hz | -10.4 | -10.8 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x4 | TDL-A-30; 10Hz | -10.5 | -10.8 |

## FR2 Results

In this section we present alignment results for FR2 test cases with 2Rx.

Figure 2: PBCH Simulation Results in FR2

The tables capture the SNR (dB) @ 1% Pm-bch. The simulations are run with and without PBCH-DMRS sequence detection.

Table 5: Alignment results for PBCH in FR2 with 2Rx

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | With DMRS Seq Det | Without DMRS Seq Det |
| Test 3 | 100MHz | 120kHz | Case D | 1x2 | TDL-A-30; 300Hz | -8.5 | -9.4 |
| Test 4 | 100MHz | 240kHz | Case E | 1x2 | TDL-A-30; 75Hz | -8.9 | -10.0 |

## Impairment Results

Taking into account implementation margin, we present the SNR (dB) @ 1% Pm-bch test cases in table below.

Table 6: Impairment results for NR PBCH tests

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | With DMRS Seq Det | Without DMRS Seq Det |
| Test 1 | 10MHz | 15kHz | Case A | 1x2 | TDL-C-300; 100Hz | -7.4 | -8.3 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x2 | TDL-A-30; 10Hz | -5.6 | -6.1 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x2 | TDL-A-30; 10Hz | -5.6 | -6.2 |
| Test 1 | 10MHz | 15kHz | Case A | 1x4 | TDL-C-300; 100Hz | -10.2 | -10.7 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x4 | TDL-A-30; 10Hz | -8.9 | -9.3 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x4 | TDL-A-30; 10Hz | -9.0 | -9.3 |
| Test 3 | 100MHz | 120kHz | Case D | 1x2 | TDL-A-30; 300Hz | -7.0 | -7.9 |
| Test 4 | 100MHz | 240kHz | Case E | 1x2 | TDL-A-30; 75Hz | -7.4 | -8.5 |

# PBCH Demodulation Requirement

In the Section 2 we present PBCH results with and without PBCH-DMRS sequence detection. In RAN4#89 it was agreed to introduce PBCH tests with PBCH-DMRS sequence detection. Test cases without PBCH-DMRS sequence detection were FFS. Based on the results presented, we observe that the operating SNR for PBCH test cases is already low with PBCH-DMRS sequence detection. Without PBCH-DMRS sequence detection in some cases the operating SNR is even further reduced. Testing the UE at such low operating SNRs may not be feasible and practical.

Observation #1: Operating SNR for some test cases is even further reduced without PBCH-DMRS sequence detection

Based on the observations above we recommend not to introduce tests without PBCH-DMRS sequence detection.

Proposal #1: For PBCH demodulation do not introduce requirements without PBCH-DMRS sequence detection

# Conclusion

In this paper we present alignment and impairment results for PBCH test cases and also provide our views on NR PBCH demodulation requirements. Our observations and proposals are summarized below.

Observation #1: Operating SNR for some test cases is even further reduced without PBCH-DMRS sequence detection

Proposal #1: For PBCH demodulation do not introduce requirements without PBCH-DMRS sequence detection

# Reference

- R4-1816490 , “Way forward on NR PBCH demodulation requirements”, CMCC