3GPP TSG-RAN WG4 Meeting #90	R4-1900329

Athens, Greece, 25th Feb – 1st March, 2019

Agenda item:	5.10.3

Source: 	Qualcomm Incorporated

Title: 	NW CRS-IM demod simulation results for TDD

Document for:	Discussion

# Introduction

In the last RAN4#88bis meeting, the Way Forward for network based CRS interference mitigation demodulation requirement was agreed [1]. Two FRC options and two muting pattern options were agreed for simulations. In RAN4#89 meeting, option 1 for FRC and option 1 for muting pattern were agreed to be used for defining the demodulation performance requirements [2]. FDD performance requirement was defined and TDD performance requirement was left as TBD [2]. In this paper, we present the TDD performance results based on settings and configurations of [2].

# TDD Simulation results

Figures 1 shows TDD performance based on configuration settings in [2] with muting pattern of [10UU11111110UU111000] where U indicates uplink subframes, 1 indicates DL subframe with full system BW CRS configured and 0 indicates DL subframes including special subframes with CRS only on the center 6 PRBs configured.

Figure 1 TDD simulation results based on configuration settings in [2]

For TDD, the SNR point corresponding to 70% of max throughput is achieved at 12.5.

Observation 1. In TDD, the SNR point corresponding to 70% of max throughput is achieved at 12.5 without implementation margin.

It is also noted that the FRC Table A.3.4.2.1-11 for TDD introduced in [2] and reproduced here has an error in max throughput averaged over 1 radio frame. As shown in Figure 1, this value is double of what is tabulated below.

Table A.3.4.2.1-11: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.11-x TDD |  |
| Channel bandwidth | MHz | 10 |  |
| Allocated resource blocks (Note 5) |  | 50 |  |
| Uplink-Downlink Configuration (Note 3) |  | 4 |  |
| Special subframe configuration |  | 4 |  |
| Allocated subframes per Radio Frame (D+S) |  | 7+1 |  |
| Modulation |  | 16QAM |  |
| Target Coding Rate |  | 1/2 |  |
| Information Bit Payload (Note 5) |  |  |  |
| For Sub-Frames 0,4,6,7,8,9 | Bits | 10680 |  |
| For Sub-Frames 1 |  | 7736 |  |
| For Sub-Frames 5 |  | NA |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |
| For Sub-Frames 0,4,5,6,7,8,9 |  | 2 |  |
| For Sub-Frames 1 |  | 2 |  |
| For Sub-Frames 5 |  | NA |  |
| Binary Channel Bits (Note 5) |  |  |  |
| For Sub-Frames 0,4,5,6,7,8,9 | Bits | 21648 |  |
| For Sub-Frames 1 |  | 17424 |  |
| For Sub-Frames 5 |  | NA |  |
| Max. Throughput averaged over 1 frame (Note 5) | Mbps | 7.1816 14.363 |  |
| UE Category |  | ≥ 2 |  |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Given per component carrier per codeword. Note 6: 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0,1,4,5,6,7,8,9. |  |  |  |

Proposal 1. Correct the value of max throughput averaged over 1 frame in Table A.3.4.2.1-11 to 14.363.

# Conclusions

Observation 1. In TDD, the SNR point corresponding to 70% of max throughput is achieved at 12.5 without implementation margin.

Proposal 1. Correct the value of max throughput averaged over 1 frame in Table A.3.4.2.1-11 to 14.363.

# References

[1] R4-1813714

[2] R4-1816152