3GPP TSG-WG RAN4 Meeting #90	R4-1900591

# Athens, Greece, 25st Feb – 1st Mar, 2019

Source: 	Nokia, Nokia Shanghai Bell, Verizon

Title: 					TP to TR 36.716-03-01: CA_2-48-66

Agenda Item: 			7.3.2

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA for 3 bands DL with 1 band UL for the CA bands CA_2-48-66.

- DL_2A-48E-66A_BCS0

**************************** Start of changes ********************************************

## 5.X	CA_2-48-66

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 3-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-48E-66A | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	∆TIB and ∆RIB values

The ∆TIB and ∆RIB values of CA_2-48-66 is already specified in TS36.101.

### 5.X.3	REFSENS requirements

The refsens exception can be specified in the same way as the fallback CA_2A-48D-66A.

Table 5.X.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-48E-66A8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48E-66A10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42C, CA_3A-42D, CA_3A-3A-42C, CA_3A-42A-42C, CA_3A-42C-42C, CA_1A-3A-19A-42C, CA_1A-3A-21A-42C, CA_1A-3A-42C, CA_3A-28A-42C, CA_3A-28A-42D, CA_3A-19A-42C, CA_3A-19A-42D, CA_3A-21A-42D, CA_3A-28A-40C, CA_3A-28A-40D, CA_3A-28A-41A-42C, CA_3A-28A-41C-42A, CA_2A-48C-48C, CA_2A-48A-48D, CA_48A-48C-66C, CA_48A-48C-66B, CA_48A-48D-66A, CA_48C-48C-66A, CA_48D-66A, CA_48A-48C-66A, CA_2A-13A-48D, CA_2A-13A-48A-48C, CA_2A-13A-48C-66A, CA_2A-13A-48A-48A-66A, CA_2A-48D-66A, CA_2A-48E-66A, CA_2A-48A-48C-66A and CA_2A-48E. NOTE 9: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

Table 5.X.3-2: Uplink configuration for the low band (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-48E-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |

**************************** End of changes ********************************************