3GPP TSG-RAN WG4 Meeting #90	R4-1900790

# Athens, GR, 25th  Feb – 1st March, 2019

Agenda item:	          	6.12.8.1.2

Source:		MediaTek Inc.

Title:		Simulation result of L1-RSRP measurement accuracy

Document for:	Discussion

# Introduction

In RAN4#89, the some agreements are made on the measurements for beam management, e.g. the side condition at SNR of -3dB, the measurement period (M values) will be based 1 or 3 samples [1].

In this paper, we update the link level simulation result for the measurement accuracy of beam management/L1-RSRP reporting.

# Link-level simulation results of L1-RSRP measurement accuracy

The measurement accuracy of SSB and CSI-RS are provided for reference. Note it is not capturing RF impairment. The simulation parameters can refer to [2].

Table 1: SSB, 90% RSRP accuracy (dB)

| SSB 90% RSRP accuracy (dB) |  |  |  |  |
| --- | --- | --- | --- | --- |
| SNR (dB) | FR1, SCS=15kHz |  |  | FR2, SCS=120kHz |
|  | AWGN |  | TDL-C 300ns 100Hz | TDL-A 30ns 75Hz |
|  | 1 tap | 3 taps | 3 taps | 3 taps |
| -3 | 1.43 | 1.02 | 1.14 | 1.16 |

Table 2: CSI-RS, 90% RSRP accuracy (dB), SCS=15 kHz, 4GHz

| CSI-RS 90% RSRP accuracy (dB) |  |  |  |
| --- | --- | --- | --- |
| SNR (dB) | BW (PRBs) | AWGN |  |
|  |  | 1 tap | 3 taps |
| -3 | 24 | 2 .20 | 1.82 |
|  | 48 | 1.83 | 1.49 |
|  | 96 | 1.44 | 1.24 |

# Conclusion

The link level simulation result for L1-RSRP measurement accuracy has been updated in this paper.

# Reference

- R4-1816120, Ad hoc minutes for NR RRM capability, gap related requirement and beam management, Intel Corporation

- R4-1811426, “Simulation assumption for evaluation of L1-RSRP measurement accuracy”, Huawei.