3GPP TSG-RAN WG4 Meeting #90	R4-1900962

Athens, Greece, 25 February-1 March, 2019

Title: 					Side conditions for NR Es/Iot in FR2

Source: 	Ericsson

Agenda item:	6.13.1

Document for:	Discussion

# Introduction

Currently, side conditions in NR FR2 are mostly TBD, for instance according to the following example for SS-RSRP intra-frequency absolute accuracy. Other measurement requirements and measurement quantities have a similar situation

Table 10.1.3.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [6] | [9] | TBD | NR_TDD_FR2_A | TBD | TBD | N/A | -70 |
|  |  |  | NR_TDD_FR2_B | TBD | TBD | N/A | -70 |
|  |  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| [8] | [11] | TBD | TNR_TDD_FR2_A, NR_TDD_FR2_B, NR_TDD_FR2_F, NR_TDD_FR2_G, NR_TDD_FR2_T, NR_TDD_FR2_Y | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

One discussion has been on the meaning of Ês/Iot in an OTA requirement. In this contribution, we provide our views on the necessity and way to formulate an Ês/Iot requirement.

# Discussion

It has been discussed that FR2 may not be an interference limited system, and hence the most important aspect is to specify minimum Io. While this is true in many scenarios, this aspect is very much deployment related, and there is no guarantee that a certain SSB is not strongly interfered, especially due to the impact of TX beamforming. Without specifying an Ês/Iot based side condition, the UE would be expected to detect and report SSB with any level of interference, which is clearly not reasonable nor practically feasible. For this reason, our view is that Ês/Iot requirements are necessary, even though they are also more difficult to specify for OTA than for conducted requirements.

Fundamentally, the issue is that RAN4 has simulated measurement accuracy with different SINR, but the actual SINR that will be experienced by a UE in a scenario depends totally on the UE receive beamforming gain which is a function of antenna design, codebook design and algorithms which select the RX beam to use.

To solve this dilemma, we propose that RAN4 defines an equivalent isotropic signal to noise and interferer ratio, which for the purposes of discussion we will abbreviate to EISINR. The EISINR is the Ês/Iot which would be observed with an ideal RF and baseband receiver if the beamforming antenna system were replaced by a hypothetical isotropic antenna.

Since the UE performs RX beamsweeping to determine a favourable RX beam to make measurements, we can assume that typically the practical UE will experience positive beamforming gain for wanted signals, especially in directions where spherical coverage requirements, and hence RRM requirements apply. Moreover, unwanted interference should typically arrive from a different direction, and the expectation should be that the interference is to some extent suppressed compared with the wanted signal, at least if it is spatially separate. Of course, if the wanted signal and interferer arrive from the same AoA, they are subject to the same antenna gain.

Since RX beamforming can be expected to improve the SINR, if we set a side condition of e.g. -6dB Ês/Iot for EISINR, the real UE should also be able to work in such a condition.

Based on this we propose

Proposal 1: RAN4 defines an equivalent isotropic signal to noise and interferer ratio, which for the purposes of discussion we will abbreviate to EISINR. The EISINR is the Ês/Iot which would be observed with an ideal RF and baseband receiver if the beamforming antenna system were replaced by a hypothetical isotropic antenna

Proposal 2: The UE is expected to be able to measure in conditions where EISINR≥-[6]dB

# Conclusion

Proposal 1: RAN4 defines an equivalent isotropic signal to noise and interferer ratio, which for the purposes of discussion we will abbreviate to EISINR. The EISINR is the Ês/Iot which would be observed with an ideal RF and baseband receiver if the beamforming antenna system were replaced by a hypothetical isotropic antenna

Proposal 2: The UE is expected to be able to measure in conditions where EISINR≥-[6]dB

# References

- RP-181482, “New Work Item on Further performance enhancement for LTE in high speed scenario”, Softbank(moderator)