3GPP TSG-RAN WG4 Meeting #90	R4-1901021

# Athens, Greece,  25th Feb - 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.171 | CR | 0031 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defining the NR requirements for WLAN and Bluetooth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 10/02/19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NR requirements for WLAN and Bluetooth are not defined. It has been agreed that separate clauses are required for these requirements, but no such clauses or requirements have been defined yet. In the absence of any other proposal it is proposed that the same same requirements as for E-UTRA are used for NR. Note that this CR only proposes the requirements, the test conditions remain FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. NR WLAN Access Point Measurements added which copy those for E-UTRA. 2. NR Bluetooth Access Point Measurements added which copy those for E-UTRA.. 3. Minor clarifications added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NR requirements for WLAN and Bluetooth will remain undefined and no further progress will be possible in this area. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 4.3.2.3, 4.4.2.1, 4.4.2.2, 4.4.2.3, new 4.4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 4.3	WLAN Measurements

### 4.3.1	General

Clause 4.3 defines the measurement requirements for the measurements performed by the UE for WLAN based positioning.

### 4.3.2	WLAN Access Point Measurements

Editor’s note:	In the WLAN requirements for NR, the NR clauses are separate from LTE, but it is FFS whether separate clauses are needed for SA NR and non-SA NR.

#### 4.3.2.1	E-UTRAN FDD-WLAN Access Point Measurements

##### 4.3.2.1.1	Introduction

The requirements defined in section 4.3.2.1 shall apply provided the E-UTRA FDD UE has received WLAN-RequestLocationInformation message from E-SMLC via LPP requesting the UE to report WLAN measurement for one or more WLAN Access Points [12].

##### 4.3.2.1.2	Measurement Requirements

The measurement delay reporting requirements for WLAN are defined in section 4.3.2.1.3. The WLAN Access Point identification minimum performance requirements are defined in clause 7.

##### 4.3.2.1.3	Measurement Reporting Delay

For LTE, WLAN measurement time is defined as the time starting from the moment that the UE has received the LPP message of type REQUEST LOCATION INFORMATION, and ending when the UE starts sending the LPP message of type PROVIDE LOCATION INFORMATION on the Uu interface. The response times specified for all test cases are based on new measurements unless otherwise stated, i.e. the UE shall not re use any information on measurements or other aiding data that was previously acquired or calculated and stored internally in the UE. A dedicated test message 'RESET UE POSITIONING STORED INFORMATION' has been defined in TS 36.509 [4] clause 6.9 for the purpose of deleting this information. No WLAN assistance data is provided to the UE.

The signals from the WLAN APs shall be available at the UE for the duration of the measurement time. Each WLAN AP transmits a beacon signal with a beacon interval smaller or equal to 102.4 ms. The beacon frames from different access points shall be transmitted in different time slots or non-overlapping frequency channels. The beacon frames have variable time duration of ~1ms.

The WLAN Measurement Reporting Delay is given as:

sec

where

is the total time for detecting and measuring the WLAN Access Points

is the elapsed time from the trigger of the measurement to the start of the first WLAN transmission period and is shown in Figure 4.3.2.1.3-1.

Figure 4.3.2.1.3-1: WLAN Measurement Time

#### 4.3.2.2	E-UTRAN TDD-WLAN Access Point Measurements

##### 4.3.2.2.1	Introduction

The requirements defined in section 4.3.2.2 shall apply provided the E-UTRA TDD UE has received WLAN-ProvideLocationInformation message from E-SMLC via LPP requesting the UE to report WLAN measurement for one or more WLAN Access Points [12].

##### 4.3.2.2.2	Measurement Requirements

The measurement reporting delay requirements for WLAN are defined in section 4.3.2.2.3. The WLAN Access Point identification minimum performance requirements are defined in clause 7.

##### 4.3.2.2.3	Measurement Reporting Delay

Same as 4.3.2.1.3.

#### 4.3.2.3	NR WLAN Access Point Measurements

##### 4.3.2.3.1	Introduction

The requirements defined in section 4.3.2.3 shall apply provided the NR UE has received WLAN-RequestLocationInformation message from LMF via LPP requesting the UE to report WLAN measurement for one or more WLAN Access Points [12].

##### 4.3.2.3.2	Measurement Requirements

The measurement reporting delay requirements for WLAN are defined in section 4.3.2.3.3. The WLAN Access Point identification minimum performance requirements are defined in clause 7.

##### 4.3.2.3.3	Measurement Reporting Delay

Same as 4.3.2.1.3.

## 4.4	Bluetooth Measurements

### 4.4.1	General

Clause 4.4 defines the measurement requirements for the measurements performed by the UE for Bluetooth based positioning.

### 4.4.2	Bluetooth Access Point Measurements

#### 4.4.2.1	Introduction

The requirements defined in sections 4.4.2 for E-UTRA shall apply provided the UE has received BT-RequestLocationInformation message from E-SMLC via LPP requesting the UE to report Bluetooth measurements for one or more Bluetooth Access Points [12].

The requirements defined in sections 4.4.2 for NR shall apply provided the UE has received BT-RequestLocationInformation message from LMF via LPP requesting the UE to report Bluetooth measurements for one or more Bluetooth Access Points [12].

Editor’s note:	In the Bluetooth access point measurement requirements for NR, the NR clauses are separate from LTE, but it is FFS whether separate clauses are needed for SA NR and non-SA NR.

#### 4.4.2.2	E-UTRA Measurement Requirements

In the RRC_CONNECTED state the measurement period for Bluetooth Access Point identification shall be TBT_meas. The value of TBT_meas is 10.24 s, and can be extended to 40.96 s if extended inquiry is allowed, provided that the following conditions are met [13]:

-	At least one Bluetooth beacon signal is transmitted on one of the Bluetooth advertising channels with a broadcast interval of 100 ms.

TBT_meas defined in this section shall apply when no DRX cycle is configured or when any DRX cycle defined in [14] is configured.

The UE physical layer shall be capable of reporting Bluetooth Access Point(s) measurements to higher layers within the measurement period of TBT_meas.

The Bluetooth RSSI measurement accuracy for all measured access points shall be fulfilled according to the accuracy as specified in the clause 6.

#### 4.4.2.3	E-UTRA Measurement Reporting Delay

This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

#### 4.4.2.4	NR Measurement Requirements

Same as 4.4.2.2.

#### 4.4.2.5	NR Measurement Reporting Delay

Same as 4.4.2.3.