3GPP TSG RAN WG4 Meeting #90	R4-1901495

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	6.10.2.3

Source:	Huawei, HiSilicon

Title:	TP to TS38.141-2 subclause 4.9.2 filling remaining RBs in PDCCH symbols

Document for:	Approval

# Introduction

During the drafting of the TPs for the test models in subclause 4.9.2.2 of 38.141-1 [2] and 38.141-2 [3], there were implementation concerns for filling RBs in the first part of the slot. A way forward was drafted in the last meeting to discuss some options [1]. This contribution captures a text proposal to fill the PDCCH for 38.141-2 [3] based on the discussion in [4]. In addition to the main text proposal, the appendix provides additional proposals for consideration for other options.

# Text proposal option 2

This option is the introduction of an independent shared channel transmitted in the same symbols as the PDCCH.

************************ BEGIN CHANGE ******************************

#### 4.9.2.2	NR FR2 test models

[…]

Common physical channel parameters for all NR FR2 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 for PDSCH, and table 4.9.2.2-3 for the PDSCH transmitted on the same symbols as the PDCCH. Specific physical channel parameters for NR FR2 test models are described in sections 4.9.2.2.1 to 4.9.2.2.3.

Table 4.9.2.2-2: Common physical channel parameters for BS type 2-O PDCCH

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 2 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 12 |
| Aggregation level | 2 |
| # of RBs not allocated for PDCCH in the each symbol | NRB – 6 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control region | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for BS type 2-O PDSCH

| Parameter | Value |
| --- | --- |
| mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | ‘pos2’ |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 0 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Starting symbol number | 2 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |
| PTRS configuration and density | LPT-RS 4 KPT-RS 2 = 00 |
| Ratio of PT-RS EPRE to DM-RS EPRE | 0 dB |

Table 4.9.2.2-5: Physical channel parameters for BS type 2-O PDSCH transmitted on symbols 0 and 1,

| Parameter | Value |
| --- | --- |
| mapping type | PDSCH mapping type B |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 0 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | Not configured |
| Modulation | QPSK |
| PDSCH duration ld | 2 |
| Starting symbol number | 0 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

************************ END CHANGE ******************************

************************ BEGIN CHANGE ******************************

##### 4.9.2.2.2	NR FR2 test model 2 (NR-FR2-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in section 4.9.2.2. The RBs occupied by PDSCH during PDCCH transmission are not filled. Specific physical channel parameters for NR-FR2-TM2 are defined in table 4.9.2.2.2-1.

[…]

************************ END CHANGE ******************************

************************ BEGIN CHANGE ******************************

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], subclause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: (# of PDCCH) * (# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in subclause 4.4.9.2.2.

-	Generate this amount of bits according to ‘all 0’ data.

-	Each CCE shall be according to TS 38.211 [20], subclause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH should occupy first two symbol for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], subclause 7.3.2.3.

-	NID=NIDcellin DM-RS sequence generation in TS 38.211 [20], subclause 7.4.1.3.

-	nRNTI = 0 in scrambling sequence generation in TS 38.211 [20], subclause 7.3.2.3.

-	Perform mapping to REs according to TS 38.211 [20], subclause 7.3.2.5.

************************ END CHANGE ******************************

************************ BEGIN CHANGE ******************************

##### 4.9.2.3.3	PDSCH during PDCCH transmission

-	For each slot generate the required amount of bits for all PRBs according to ‘all 0’ data.

-	1 user PDSCH transmissions distinguished by nRNTI=2.

-	Perform user specific scrambling according to TS 38.211 [20], subclause 7.3.1.1.

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [20], subclause7.3.1.2.

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [20], subclause 7.3.1.3.   Complex-valued modulation symbols  for codeword q shall be mapped onto the layers ,  where υ is equal to 1.

-	Perform PDSCH mapping type according to TS 38.211 [20] using parameters listed in table 4.9.2.2-4.

-	DM-RS sequence generation according to TS 38.211 [20], subclause 7.4.1.1.1 where l is the OFDM symbol number within the slot with symbols indicated by table 4.9.2.2-4.

-	DM-RS mapping according to TS 38.211 [20], subclause 7.4.1.1.2 with parameters listed in table 4.9.2.2-4.

************************ END CHANGE ******************************

# Conclusion

The text proposal for the filler PDSCH is provided for filling the control region. Additional text proposals in the appendix are provided for comparison purposes.

# References

R4-1816736, “WF on filling remaining RBs of PDCCH symbols”, Huawei, RAN4#89, Nov. 11-15, 2018.

TS38.141-1

TS38.141-2

R4-1901493, “Discussion of filling remaining RBs in PDCCH symbols of test models”, Huawei, HiSilicon RAN4#90, Feb. 25 – Mar. 1, 2019.

# Appendix A: Text proposal option 1

The proposal updates the PDCCH configuration table by introducing 3 sets which are a function of the bandwidth. In addition, the set needed for TM2.1 is specified as well as the number of bits needed.

************************ BEGIN CHANGE ******************************

#### 4.9.2.2	NR FR2 test models

[…]

Table 4.9.2.2-2: Common physical channel parameters for BS type 2-O PDCCH

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | AL=8: NRB64 AL=4: 2NRB6mod84 AL=2: 2NRB6mod42 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 12NRB6 |
| Aggregation level | AL=8: NRB64 AL=4: 2NRB6mod84 AL=2: 2NRB6mod42 |
| # of RBs not allocated for PDCCH in the each symbol | 12NRB6 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control region | 10log10NRB6NRB6 dB |

************************ END CHANGE ******************************

************************ BEGIN CHANGE ******************************

##### 4.9.2.2.2	NR FR2 test model 2 (NR-FR2-TM2)

[…]

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| PDCCH Configuration | Use NRB=11 in table 4.9.2.2-2 and Boosting level of control region to [0] dB |

************************ END CHANGE ******************************

************************ BEGIN CHANGE ******************************

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], subclause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: (# of PDCCH) * (# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in subclause 4.4.9.2.2.

-	Generate this amount of bits according to ‘all 0’ data.

-	Each PDCCH shall be according to TS 38.211 [20], subclause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH should occupy first two symbol for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], subclause 7.3.2.3.

-	NID=NIDcellin DM-RS sequence generation in TS 38.211 [20], subclause 7.4.1.3.

-	nRNTI = 0 in scrambling sequence generation in TS 38.211 [20], subclause 7.3.2.3.

-	Perform mapping to REs according to TS 38.211 [20], subclause 7.3.2.5. Highest aggregation levels are mapped to the lowest CCE index. Lowest aggregation level is mapped to the highest CCE available index.

************************ END CHANGE ******************************

# Appendix B: Text proposal option 1 (variant)

The proposal updates the PDCCH configuration table by introducing 3 sets which are a function of the bandwidth. In addition, the set needed for TM2.1 is specified as well as the number of bits needed.

************************ BEGIN CHANGE ******************************

#### 4.9.2.2	NR FR2 test models

[…]

Table 4.9.2.2-2: Common physical channel parameters for BS type 2-O PDCCH

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Starting RB location for PDCCH | 0 |
| Set 1: NRB < 24 |  |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 12 |
| Aggregation level | 2 |
| # of RBs not allocated for PDCCH in each symbol | NRB – 6 |
| Boosting level of control region | 10 log10(NRB/6) |
| Set 2: 24 ≤ NRB < 96 |  |
| # of CCEs allocated to PDCCH | 1 |
| # of available REGs | 48 |
| Aggregation level | 8 |
| # of RBs not allocated for PDCCH in each symbol | NRB – 24 |
| Boosting level of control region | 10 log10(NRB/24) |
| Set 3: 96 ≤ NRB |  |
| # of CCEs allocated to PDCCH | 4 |
| # of available REGs | 192 |
| Aggregation level | 8 |
| # of RBs not allocated for PDCCH in each symbol | NRB – 96 |
| Boosting level of control region | 10 log10(NRB/96) |

[…]

************************ END CHANGE ******************************

************************ BEGIN CHANGE ******************************

##### 4.9.2.2.2	NR FR2 test model 2 (NR-FR2-TM2)

[…]

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| PDCCH configuration | Set 1 in table 4.9.2.2-2 with boosting level of control region set to [0 dB] |

************************ END CHANGE ******************************

************************ BEGIN CHANGE ******************************

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], subclause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: (# of PDCCH) * (# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in subclause 4.4.9.2.2.

-	Generate this amount of bits according to ‘all 0’ data.

-	Each PDCCH shall be according to TS 38.211 [20], subclause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH should occupy first two symbol for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], subclause 7.3.2.3.

-	NID=NIDcellin DM-RS sequence generation in TS 38.211 [20], subclause 7.4.1.3.

-	nRNTI = 0 in scrambling sequence generation in TS 38.211 [20], subclause 7.3.2.3.

-	Perform mapping to REs according to TS 38.211 [20], subclause 7.3.2.5. Highest aggregation levels are mapped to the lowest CCE index. Lowest aggregation level is mapped to the highest CCE available index.

************************ END CHANGE ******************************

# Appendix C: Text proposal option 3

************************ BEGIN CHANGE ******************************

#### 4.9.2.2	NR FR2 test models

[…]

Table 4.9.2.2-2: Common physical channel parameters for BS type 2-O PDCCH

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 2 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 12 |
| Aggregation level | 2 |
| # of RBs not allocated for PDCCH in the each symbol | NRB – 6 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control region | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for BS type 2-O PDSCH

| Parameter | Value |
| --- | --- |
| mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | ‘pos2’ |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 0 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |
| PTRS configuration and density | LPT-RS 4 KPT-RS 2 = 00 |
| Ratio of PT-RS EPRE to DM-RS EPRE | 0 dB |
| Starting symbol number | 0 |

************************ END CHANGE ******************************

************************ BEGIN CHANGE ******************************

##### 4.9.2.2.2	NR FR2 test model 2 (NR-FR2-TM2)

[…]

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| Starting symbol number | 2 in table 4.9.2.2-3 |

************************ END CHANGE ******************************

************************ BEGIN CHANGE ******************************

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], subclause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: (# of PDCCH) * (# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in subclause 4.4.9.2.2.

-	Generate this amount of bits according to ‘all 0’ data.

-	Each CCE shall be according to TS 38.211 [20], subclause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH should occupy first two symbol for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], subclause 7.3.2.3.

-	NID=NIDcellin DM-RS sequence generation in TS 38.211 [20], subclause 7.4.1.3.

-	nRNTI = 0 in scrambling sequence generation in TS 38.211 [20], subclause 7.3.2.3.

-	Perform mapping to REs according to TS 38.211 [20], subclause 7.3.2.5.

************************ END CHANGE ******************************