3GPP TSG-RAN WG4#890	R4-1901584

Athens, Greece, 25th February – 1st March 2018

Agenda Item:	5.3.4

Source:	Ericsson

Title:	Updated simulation results for WUS reception performance for NB-IoT

Document for:	Discussion

# Introduction

At last RAN4 meeting, RAN4 agreed to make some further updates to the WUS simulation assumptions. In particular, the frequency and timing drifts were updated and use of orthogonal precoders in the case of 2 Tx based WUS transmissions were clarified in [1].

In this contribution, we present simulation results based on the updated simulation assumptions with the aim to resolve the TBDs in the specification.

# Simulation assumption for WUS for feNB-IoT

Table 1: Simulation assumption for WUS [1]

| Parameter | Value |
| --- | --- |
| BS TX antenna configuration | 1 Tx, 2 Tx with transmit diversity, Note: Tx diversity scheme will depend on RAN1 agreements |
| UE Rx antenna configuration | 1 Rx |
| WUS BW | 180 kHz |
| Channel model | EPA 1Hz, ETU 1 Hz |
| SNR | -6 dB, -12 dB |
| False alarm rate | 1% |
| Detection probability | 99% |
| Combining | Option 1: No coherent combining over multiple subframes; Option 2: Coherent combining of 2 consecutive WUS subframes |
| Precoders | Orthogonal precoders alternate every 2 WUS subframes for 2 Tx based WUS transmissions |

Table 2 Simulation assumption for WUS [1]

| DRX cycle (s) | Frequency drift [Hz] | Timing drift [s] |
| --- | --- | --- |
| 1.28 | 170.8 | 4.0960e-08 |
| 2.56 | 311.6 | 1.6348e-07 |
| 5.12 | 593.2 | 6.5536e-07 |
| 10.24 | 1156.4 | 2.6212e-06 |

# Simulation results

Table 1: Conditions for NWUS reception for at 1 % false alarm (FA) and 99% detection probability at SNR = -6 dB and -12 dB with EPA channel model

| DRX cycle length [s] | Required no. of Rep. with 2 Tx (no coh. comb.) |  | Required no. of Rep. with 1 Tx (no coh. comb.) |  | Required no. of Rep. with 1 Tx and coh. comb. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | -6 dB | -12 dB | -6 dB | -12 dB | -6 dB | -12 dB |
| 1.28 | 8 | 64 | 64 | 256 | 64 | 256 |
| 2.56 | 8 | 64 | 64 | 256 | 128 | 512 |
| 5.12 | 16 | 128 | 128 | 256 | 256 | 1024 |
| 10.24 | - | - | - | - | - | - |

Table 2: Conditions for NWUS reception for at 1 % false alarm (FA) and 99% detection probability at SNR = -6 dB and -12 dB with ETU channel model

| DRX cycle length [s] | Required no. of Rep. with 2 Tx (no coh. comb.) |  | Required no. of Rep. with 1 Tx (no coh. comb.) |  | Required no. of Rep. with 1 Tx and coh. comb. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | -6 dB | -12 dB | -6 dB | -12 dB | -6 dB | -12 dB |
| 1.28 | 8 | 64 | 64 | 256 | 64 | 256 |
| 2.56 | 8 | 64 | 64 | 256 | 128 | 512 |
| 5.12 | 16 | 128 | 128 | 512 | 256 | 1024 |
| 10.24 | - | - | - | - | - | - |

# Summary

In this contribution, the simulation results for minimum WUS reception performance for release 15 NB-IoT are presented based on updated agreed simulation assumptions in [1]. We kindly ask the companies to consider these results to resolve the TBDs in the current specification. A companion CR to resolve TBDs based on the above results can be found in [2].

# Reference

R4-1816332, “Updated simulation assumptions for Rel-15 NB-IoT WUS”, Ericsson

R4-1901585, “Maintenance CR on WUS requirements for Rel-15 NB-IoT”, Ericsson