3GPP TSG-RAN WG3 Meeting #90	R4-1901592

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	7.11.1

Source:	Ericsson

Title:	WI work plan for Rel-16 LTE-MTC for core part

Document for:	Discussion

# Introduction

The objective of the Rel-16 WI on “Additional MTC enhancements for LTE” [1][2] is to specify a set of improvements for machine-type communications for BL/CE UEs (i.e. BL UEs and UEs in CE). In this contribution, we go through each objective and identify the potential RAN4 work and provide a time plan.

# Discussion

In this section we discuss the expected RAN4 work with regard to the different WI objectives listed in [1].

Improved DL transmission efficiency and/or UE power consumption

- Specify support for mobile-terminated (MT) early data transmission (EDT) [RAN2, RAN3]

- Specify quality report in MSG3 at least for EDT [RAN1, RAN2]

- Specify aperiodic quality report in connected mode using same quality definition as in Msg3 [RAN1, RAN2, RAN4]

- Specify MPDCCH performance improvement by using CRS at least for connected mode [RAN1, RAN2, RAN4]

- Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

The above objective requires following RAN4 work:

- RRM requirements needed to estimate the repetition levels needed for quality report in MSG3 in RRC_CONNECTED mode.

- RRM work needed for MPDCCH performance improvement using CRS. This may affect the RLM procedure and RAN4 needs to evaluate the potential RLM improvement.

- RAN4 needs to study, and if needed, specify RRM requirements for release 16 group WUS based on the physical layer design.

Improved UL transmission efficiency and/or UE power consumption

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance [RAN1, RAN2, RAN4]

  - Both shared resources and dedicated resources can be discussed

  - Note: This is limited to orthogonal (multi) access schemes

The above objective requires RAN4 work which includes the following:

- Transmission using preconfigured uplink resources may require corresponding RRM requirements.

Scheduling enhancement

- Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast [RAN1, RAN2]

  - Enhancement of SPS can be discussed.

The above objective requires following RAN4 work:

- No RRM work expected.

Mobility enhancement

- Consider improving the DL RSRP and, if needed, RSRQ measurement accuracy, through use of RSS [RAN1, RAN4, RAN2]

- Specify relaxation of RRM measurements for serving cell for UEs using WUS for at least low mobility UEs [RAN4, RAN2]

The above objective requires RAN4 work which includes the following:

- RRM work is needed to study and specify RSS based RRM requirements.

- RRM work is needed to specify requirements for serving cell RRM measurement relaxation.

Extreme coverage for non-BL UEs

- Specify CE mode A and B improvements for non-BL UEs from among the following list [RAN1, RAN2, RAN4]

  - Enhancements to idle mode mobility

  - UE demodulation performance requirements for 2 RX antennas and full duplex FDD

  - Dual layer DL reception

  - Feedback based on CSI-RS

  - ETWS/CMAS in connected mode

The above objective requires RAN4 work which includes the following:

RAN4 shall follow the RAN2 outcome on enhancements to IDLE mode mobility work and define RRM requirements if needed.

Stand-alone deployment

- Enable the use of LTE control channel region for DL transmission (MPDCCH/PDSCH) to BL/CE UEs [RAN1, RAN2, RAN4]

  - This deployment mode should support legacy operation for legacy BL/CE UEs.

The above objective requires RAN4 work which includes the following:

- RAN4 shall follow the RAN1 outcome and if needed, define corresponding requirements.

Coexistence with NR:

- Study NR and LTE specifications to identify possible issues related to coexistence of LTE-MTC with NR [RAN4, RAN1, RAN2]

The above objective requires RAN4 work which includes the following:

- This objective requires at least UE and BS RF work, and RRM is FFS.

# Time plan

In this section we present the WI rapporteur’s view of the time plan for the WI.

February 2019

RAN4 core part:

- Initial discussions/agreements

  - Discussions on transmissions using PUR based on incoming LS in April 2019 [3]

  - RSS based measurements based on incoming LS in [4]

  - Initial RRM discussions on other objectives based on progress in other working groups

- Discuss simulation assumptions

May 2019

RAN4 core part:

- Agree simulation assumptions

August 2019

RAN4 core part:

- Align simulation results

October 2019

RAN4 core part:

- Agree requirements

November 2019

RAN4 core part:

- Agree final CRs

December 2020

RAN:

- Approve CRs for core part

# Summary

In this contribution, we have presented the WI rapporteur’s view of the expected RAN4 work and time plan for the Rel-16 WI on “Additional MTC enhancements for LTE”.

# References

RP-182891, “Revised WID: Additional MTC enhancements for LTE”

RP-182904, “TU overview after RAN#82”

R1-1813778, “LS on preconfigured uplink resources in NB-IoT and MTC”, RAN WG1

R1-1811934, “LS on RSRP measurements using RSS for Rel-16 LTE-MTC”, RAN WG1