3GPP RAN WG4 Meeting #90	R4-1901855

Athens, Greece, February 25 – March 1, 2019

Agenda item:	6.7.7.1

Source:	Apple Inc.

Title:	Temperature Condition for Testing REFSENS and Peak EIRP in FR2

Document for:	Approval

# Introduction

In RAN4 #89 meeting, it was agreed to test EIRP and EIS spherical coverage requirements in normal temperature condition (NTC) for FR2 UEs. The discussions on temperature condition for testing REFSENS and peak EIRP requirements are still ongoing. This contribution provides some insight on the impact of temperature condition on these requirements.

# Discussion

The peak EIRP and EIS requirements as well as EIRP and EIS special coverage requirements for power class 3 UEs are available in of TS38.101-2 [1].

The UE in FR2 is required to work in extreme temperature conditions (ETC) per Annex E.2.1 in TS 38.101-2 [1].  There were however some discussions in RAN4#88bis on the applicability of extreme thermal conditions to some RF requirements in FR2 and it was agreed that companies to propose a list of RF requirements which need be tested in NTC due to expected different performance in extreme temperature conditions [2].

In RAN4 #89, it was agreed to test EIRP and EIS spherical coverage requirements in NTC [3] and RAN4 informed RAN5 about the decision in an LS [4].

As we mentioned in [5], the current REFSENS requirement is derived at 20C as it is using -174 dBm(kT) [14]. Therefore, the difference in the noise floor between 20C and either of two extreme temperatures i.e. -10C and +55C is about 0.5 dB using the equation below.

∆NF=10log10273.15+55273.15+20≈0.5dB

Observation 1: The REFSENS requirement is derived under normal temperature condition (NTC) assumption and testing it in extreme temperature condition (ETC) will result in 0.5dB difference in noise floor.

The extreme temperature not only changes the thermal noise floor, but also changes much more inside the ICs. For example, usually the gain changes, where a lower gain at higher temperatures results in a higher cascaded noise figure. Further, ETC will also increase the phase noise of the LO which decreases the SNR. Furthermore, ETC also changes the phase and amplitude balance of the IQ down-conversion thus reducing the performance. In total, it is expected to observe 1.5 dB total degradation due to ETC.

Observation 2: In addition to impact on thermal noise floor, ETC affects different modules in the receive chain resulting in lower performance. It is expected to observe 1.5dB total degradation due to ETC.

From testability perspective, we note that the peak EIRP and EIS beam directions are found and measured for testing the EIRP and EIS spherical coverage requirements respectively under NTC.

Observation 3: The peak EIRP and EIS beam directions are found and measured for testing the EIRP and EIS spherical coverage requirements respectively under NTC.

In order to find the beam peak direction in ETC, the test vendor should increase/decrease the temperature around the DUT by, for example, putting a bubble around DUT which increases the insertion loss. It is also worth noting that by putting the bubble around the DUT, the beam peak direction may change and it is thus needed to find the beam peak direction again in ETC. Therefore, testing peak EIRP and REFSENS requirements under ETC will significantly increase the testing time.

Observation 3: The beam peak direction for both peak EIRP and EIS requirements may change if the temperature is changed to ETC.

Observation 4: Testing peak EIRP and REFSENS requirements under ETC will significantly increase the testing time.

Based on observations given above, we think it makes sense to test the REFSENS and peak EIRP requirements under similar temperature conditions as EIS and EIRP spherical coverage requirements. We therefore propose to test the REFSENS and minimum peak EIRP requirements under normal temperature conditions.

Proposal 1: Test the REFSENS requirement under normal temperature condition.

Proposal 2: Test the minimum peak EIRP requirement under normal temperature condition.

In case RAN4 decides to test the REFSENS requirement under ETC, we propose a 1.5 dB relaxation in the REFSENS requirement to address the impact of extreme temperature conditions.

Proposal 3: In case RAN4 decides to test the REFSENS under ETC, consider a 1.5 dB relaxation in the REFSENS requirement to address the impact of extreme temperature conditions.

# Proposals (for approval)

In this paper, we explained some of the possible impacts of testing RESENS and peak EIRP requirements in extreme temperature conditions.

Based on the discussions, the following observations and proposal are made:

Observation 1: The REFSENS requirement is derived under normal temperature condition (NTC) assumption and testing it in extreme temperature condition (ETC) will result in 0.5dB difference in noise floor.

Observation 2: In addition to impact on thermal noise floor, ETC affects different modules in the receive chain resulting in lower performance. It is expected to observe 1.5 dB total degradation due to ETC.

Observation 3: The peak EIRP and EIS beam directions are found and measured for testing the EIRP and EIS spherical coverage requirements respectively under NTC.

Observation 4: Testing peak EIRP and REFSENS requirements under ETC will significantly increase the testing time.

Proposal 1: Test the REFSENS requirement under normal temperature condition.

Proposal 2: Test the minimum peak EIRP requirement under normal temperature condition.

Proposal 3: In case RAN4 decides to test the REFSENS under ETC, consider a 1.5 dB relaxation in the REFSENS requirement to address the impact of extreme temperature conditions.

# References

- TS38.101-2 v15.4.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” 3GPP, December 2018

- R4-1814173, “WF on FR2 Temperature Conditions for UE RF Requirements Verification”, MediaTek, 3GPP RAN4 #88bis, October 2018

- R4-1900002, “RAN4#89 Meeting Report”, ETSI MCC, RAN4#90, February 2019

- R4-1816748, “LS on the temperature conditions for testing some FR2 requirements”, RAN4#89, November 2018

- R4-1812388, “Further Views On EIS Spherical Coverage for FR2 UEs”, Apple Inc, 3GPP RAN4 #88bis, October 2018