3GPP TSG-RAN WG4 Meeting #90	R4-1901900

# Athens, Greece, 25th Feb – 1st Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.133 MTTD and MRTD requirements in inter-band sync EN-DC (7.5.2, 7.6.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To remove MTTD and MRTD requirements in inter-band sync EN-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | To remove the MTTD and MRTD requirements in inter-band sync EN-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for MTTD and MRTD in inter-band sync EN-DC is still open. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2, 7.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

### 7.5.2	Minimum Requirements for inter-band EN-DC

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1.

Table 7.5.2-1 Maximum uplink transmission timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note1 | 62.5 |
| NOTE 1: For E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC, for which the requirement is defined in Section 7.5.3 and this Table 7.5.2-1 is also applicable, the scenario with 120kHz PSCell does not exist. |  |  |

<< End of change 1>>

<< Start of change 1>>

### 7.6.2	Minimum Requirements for inter-band EN-DC

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-1.

Table 7.6.2-1: Maximum receive timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) (Note 1) | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note2 | 62.5 |
| NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. NOTE 2: For E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC, for which the requirement is defined in Section 7.6.3 and this Table 7.6.2-1 is also applicable, the scenario with 120kHz does not exit. |  |  |

<< End of change 2>>