3GPP TSG-RAN WG4 Meeting #90 	R4-1901943

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interruptions requirements with SRS carrier based switching in 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No interruptions requirements are defined for SRS carrier based switching |  |  |  |  |  |  |  |  |
| Summary of change: |  | Interruptions requirements for SRS carrier based switching are introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No interruptions requirements are defined for SRS carrier based switching |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 8.2.2, 8.2.3, 8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

### 8.2.1	NSA: Interruptions with EN-DC

#### 8.2.1.1	Introduction

This section contains the requirements related to the interruptions on PSCell and SCell, when

E-UTRA PCell transitions between active and non-active during DRX, or

E-UTRA PCell transitions from non-DRX to DRX, or

E-UTRA SCell in MCG or SCell in SCG is added or released, or

E-UTRA SCell in MCG or SCell in SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

SRS carrier based switching is performed between NR carriers, or

SRS carrier based switching is performed between E-UTRAN carriers, or

SRS carrier based switching is performed between NR carriers and between E-UTRAN carriers.

This section also contains the requirements related to the interruptions on other active serving cells in the same frequency range wherein the UE is performing BWP switching.

The requirements shall apply for E-UTRA-NR DC with an E-UTRA PCell.

This section contains interruptions where victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PCell or E-UTRA SCell belonging to MCG are specified in [15].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or active SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or active SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

#### 8.2.1.2	Requirements

--- unchanged sections ---

##### 8.2.1.2.8	Interruptions at SRS carrier based switching between NR carriers

A PUSCH-less SCell is a TDD SCell without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [6] and/or non-contention based PRACH on a PUSCH-less SCell, the UE can perform carrier based switching to one or more CCs with PUSCH-less SCells from a CC with PUSCH or from another CC with PUSCH-less SCell prior to transmitting SRS and/or PRACH, provided that:

-	SRS switching is performed between NR carriers: from a configured CC to another CC with activated TDD CC;

-	the CCs with PUSCH-less SCells to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [2] for periodic SRS transmission or indicated by PDCCH for PRACH;

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [2];

-	other conditions are FFS.

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

When performing SRS carrier based switching the UE shall meet the following interruptions requirements:

The interruption on PSCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 slots including the first slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PSCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within [TBD],

The interruption on PSCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 slots + TBD including the first slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PSCell or the activated SCell)  immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within more than [TBD],

The interruption on PSCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 slots including the last slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PSCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within [TBD].

The interruption on PSCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 slots + TBD including the last slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PSCell or the activated SCell) immediately before the switching, provided that the victim victim serving cell and the PUSCH-less SCell are aligned within more than [TBD].

NOTE: interruptions requirements on E-UTRAN PCell and E-UTRAN SCells in EN-DC due to the SRS switching between the NR carriers are defined in TS 36.133 [15].

##### 8.2.1.2.9	Interruptions at SRS carrier based switching between E-UTRAN carriers

Editor’s note: requirements are TBD.

NOTE: interruptions requirements on E-UTRAN PCell and E-UTRAN SCells in EN-DC due to the SRS switching between the E-UTRAN carriers are defined in TS 36.133 [15].

##### 8.2.1.2.10	Interruptions at SRS carrier based switching between NR carriers and between E-UTRAN carriers

Editor’s note: requirements are TBD.

### NOTE: interruptions requirements on E-UTRAN PCell and E-UTRAN SCells in EN-DC due to the SRS switching between the NR carriers and between the E-UTRAN carriers are defined in TS 36.133 [15].

### 8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation

#### 8.2.2.1	Introduction

This section contains the requirements related to the interruptions on PCell and activated SCell if configured, when up to TBD SCells are configured, deconfigured, activated or deactivated.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

This section also contains the requirements related to the interruptions on PCell and activated SCell when SRS carrier based switching is performed between NR carriers.

This section also contains the requirements related to the interruptions on other active serving cells in the same frequency range wherein the UE is performing BWP switching.

For a UE which does not support per-FR measurement gaps, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For UE which support per-FR gaps, interruptions to PCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

#### 8.2.2.2	Requirements

--- unchanged sections ---

##### 8.2.2.2.6	Interruptions at SRS carrier based switching between NR carriers

A PUSCH-less SCell is a TDD SCell without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [6] and/or non-contention based PRACH on a PUSCH-less SCell, the UE can perform carrier based switching to one or more CCs with PUSCH-less SCells from a CC with PUSCH or from another CC with PUSCH-less SCell prior to transmitting SRS and/or PRACH, provided that:

-	SRS switching is performed between NR carriers: from a configured CC to another CC with activated TDD CC;

-	the CCs with PUSCH-less SCells to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [2] for periodic SRS transmission or indicated by PDCCH for PRACH;

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [2];

-	other conditions are FFS.

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

When performing SRS carrier based switching the UE shall meet the following interruptions requirements:

The interruption on PCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 slots including the first slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within [TBD],

The interruption on PCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 slots + TBD including the first slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within more than [TBD],

The interruption on PCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 slots including the last slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within [TBD].

The interruption on PCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 slots + TBD including the last slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within more than [TBD].

### 8.2.3	Interruptions with NR-DC

#### 8.2.3.1	Introduction

This section contains the requirements related to the interruptions on PCell, PSCell and SCell, when

SRS carrier based switching is performed between NR carriers, or

[TBD].

The requirements shall apply for NR-DC.

#### 8.2.3.2	Requirements

#### 8.2.3.2.1	Interruptions at SRS carrier based switching between NR carriers

A PUSCH-less SCell is a TDD SCell without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [6] and/or non-contention based PRACH on a PUSCH-less SCell, the UE can perform carrier based switching to one or more CCs with PUSCH-less SCells from a CC with PUSCH or from another CC with PUSCH-less SCell prior to transmitting SRS and/or PRACH, provided that:

-	SRS switching is performed between NR carriers: from a configured CC to another CC with activated TDD CC;

-	the CCs with PUSCH-less SCells to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [2] for periodic SRS transmission or indicated by PDCCH for PRACH;

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [2];

-	other conditions are FFS.

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

When performing SRS carrier based switching the UE shall meet the following interruptions requirements:

The interruption on PCell, PSCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 slots including the first slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell, PSCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within [TBD],

The interruption on PCell, PSCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 slots + TBD including the first slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell, PSCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within more than [TBD],

The interruption on PCell, PSCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 slots including the last slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell, PSCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within [TBD].

The interruption on PCell, PSCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 slots + TBD including the last slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell, PSCell or the activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within more than [TBD].

### 8.2.4	Interruptions with NE-DC

#### 8.2.4.1	Introduction

This section contains the requirements related to the interruptions on PCell and SCell, when

SRS carrier based switching is performed between NR carriers, or

SRS carrier based switching is performed between E-UTRAN carriers, or

SRS carrier based switching is performed between NR carriers and between E-UTRAN carriers, or

[TBD].

The requirements shall apply for NE-DC.

#### 8.2.4.2	Requirements

##### 8.2.4.2.1	Interruptions at SRS carrier based switching between NR carriers

A PUSCH-less SCell is a TDD SCell without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [6] and/or non-contention based PRACH on a PUSCH-less SCell, the UE can perform carrier based switching to one or more CCs with PUSCH-less SCells from a CC with PUSCH or from another CC with PUSCH-less SCell prior to transmitting SRS and/or PRACH, provided that:

-	SRS switching is performed between NR carriers: from a configured CC to another CC with activated TDD CC;

-	the CCs with PUSCH-less SCells to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [2] for periodic SRS transmission or indicated by PDCCH for PRACH;

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [2];

-	other conditions are FFS.

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

When performing SRS carrier based switching the UE shall meet the following interruptions requirements:

The interruption on PCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 slots including the first slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell or activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within [TBD],

The interruption on PCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 slots + TBD including the first slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell or activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within more than [TBD],

The interruption on PCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 slots including the last slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell or activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within [TBD].

The interruption on PCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 slots + TBD including the last slot where SRS transmission is configured on the PUSCH-less SCell, where μ is defined in TS 38.211 [6, Section 4.2] and determined by the SCS of the victim serving cell (PCell or activated SCell) immediately before the switching, provided that the victim serving cell and the PUSCH-less SCell are aligned within more than [TBD].

NOTE: interruptions requirements on E-UTRAN PSCell and E-UTRAN SCells in NE-DC due to the SRS switching between the NR carriers are defined in TS 36.133 [15].

##### 8.2.4.2.2	Interruptions at SRS carrier based switching between E-UTRAN carriers

Editor’s note: requirements are TBD.

NOTE: interruptions requirements on E-UTRAN PSCell and E-UTRAN SCells in NE-DC due to the SRS switching between the E-UTRAN carriers are defined in TS 36.133 [15].

##### 8.2.4.2.3	Interruptions at SRS carrier based switching between NR carriers and between E-UTRAN carriers

Editor’s note: requirements are TBD.

NOTE: interruptions requirements on E-UTRAN PSCell and E-UTRAN SCells in NE-DC due to the SRS switching between the E-UTRAN carriers and between the NR carriers are defined in TS 36.133 [15].

--- End of change 1 ---