3GPP TSG-RAN WG4 Meeting #90                 	R4-1902433

Athens, Greece, 25 Feb – 1 Mar 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PUSCH simulation results

Agenda item:	6.14.2.2.1

Document for:	Discussion

Introduction

In the RAN4#88-bis and RAN4#89 meetings, the way forward for contributing the input for NR PUSCH demodulation performance requirements was agreed [1, 2]. A procedure to arrive at performance requirement SNR numbers from the contributions of all companies was furthermore agreed in [3].
In this contribution we deliver corrections of our previously delivered results for the agreed basic simulation cases. The corresponding fields in the advance test cases are currently empty but will be delivered via a revision of this contribution at the beginning of the meeting.

Simulation Results (ideal)

Transform precoding disabled

For transform precoding disabled case, the following simulation assumptions were agreed in [1] and [2]:

Table 1: Simulation parameters – Transform precoding disabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1, 2 | 1, 2 |
| Number of Rx | 2, 4, 8 | 2 |
| Number of layers | 1, 2 | 1, 2 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1, 1+1 | 1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type B |
| Frequency domain resource | full RB allocation of the applicable BW | full RB allocation of the applicable BW |
| MCS index | 2, 16, 20 (20 only for 1Tx) | 2, 16, 20 (20 only for 1Tx) |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz MCS 20: TDL-A 30ns, 10Hz | MCS 2: TDL-A 30ns, 300Hz MCS 16: TDL-A 30ns, 300Hz MCS 20: TDL-A 30ns, 75Hz |
| SCS and BW | 15kHz: 5MHz, 10MHz, 20MHz 30kHz: 10MHz, 20MHz, 40MHz, 100MHz 60kHz(FR2): 50MHz, 100MHz 120kHz: 50MHz, 100MHz, 200MHz |  |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS =1 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Remark: Following the agreement in [2], the DMRS configuration 1+0 is not applicable in FR1 under 2T2R.

Simulation results with these parameters are shown in Table 2 to Table 7.

Table 2: Simulation results for Transform precoding disabled case 1T2R

| PUSCH 1T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -0,62 |
|  |  |  |  |  | DMRS 1+1 | -4,13 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 11,18 |
|  |  |  |  |  | DMRS 1+1 | 8,68 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 11,65 |
|  |  |  |  |  | DMRS 1+1 | 11,55 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,34 |
|  |  |  |  |  | DMRS 1+1 | -4,33 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,73 |
|  |  |  |  |  | DMRS 1+1 | 8,41 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 11,8 |
|  |  |  |  |  | DMRS 1+1 | 11,54 |
|  | FR2 | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -3,74 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 10,08 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 12,21 |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -4,11 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 8,95 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 11,77 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | 0,13 |
|  |  |  |  |  | DMRS 1+1 | -3,69 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 10,56 |
|  |  |  |  |  | DMRS 1+1 | 8,81 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 11,63 |
|  |  |  |  |  | DMRS 1+1 | 11,57 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | -3,28 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 11,68 |
|  |  |  |  |  | DMRS 1+1 | 8,58 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 11,81 |
|  |  |  |  |  | DMRS 1+1 | 11,5 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,66 |
|  |  |  |  |  | DMRS 1+1 | -4,26 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,64 |
|  |  |  |  |  | DMRS 1+1 | 8,63 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 11,42 |
|  |  |  |  |  | DMRS 1+1 | 11,6 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,88 |
|  |  |  |  |  | DMRS 1+1 | -4,42 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,82 |
|  |  |  |  |  | DMRS 1+1 | 8,58 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 11,66 |
|  |  |  |  |  | DMRS 1+1 | 11,61 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,79 |
|  |  |  |  |  | DMRS 1+1 | -4,46 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,69 |
|  |  |  |  |  | DMRS 1+1 | 8,25 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 12,44 |
|  |  |  |  |  | DMRS 1+1 | 12,29 |
|  | FR2 | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -3,49 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 9,92 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 12,05 |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -3,62 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 9,18 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 12,09 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | -4,01 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 9,09 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 11,85 |

Table 3: Simulation results for Transform precoding disabled case 1T4R

| PUSCH 1T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,88 |
|  |  |  |  |  | DMRS 1+1 | -7,49 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,3 |
|  |  |  |  |  | DMRS 1+1 | 4,39 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 7,79 |
|  |  |  |  |  | DMRS 1+1 | 7,74 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,83 |
|  |  |  |  |  | DMRS 1+1 | -7,65 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,29 |
|  |  |  |  |  | DMRS 1+1 | 4,15 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 7,66 |
|  |  |  |  |  | DMRS 1+1 | 7,44 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,43 |
|  |  |  |  |  | DMRS 1+1 | -7,21 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,02 |
|  |  |  |  |  | DMRS 1+1 | 4,4 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 7,76 |
|  |  |  |  |  | DMRS 1+1 | 7,71 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,02 |
|  |  |  |  |  | DMRS 1+1 | -6,96 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,34 |
|  |  |  |  |  | DMRS 1+1 | 4,25 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 7,72 |
|  |  |  |  |  | DMRS 1+1 | 7,39 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,12 |
|  |  |  |  |  | DMRS 1+1 | -7,59 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,24 |
|  |  |  |  |  | DMRS 1+1 | 4,38 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 7,6 |
|  |  |  |  |  | DMRS 1+1 | 7,75 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,31 |
|  |  |  |  |  | DMRS 1+1 | -7,73 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,44 |
|  |  |  |  |  | DMRS 1+1 | 4,28 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 7,8 |
|  |  |  |  |  | DMRS 1+1 | 7,71 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,33 |
|  |  |  |  |  | DMRS 1+1 | -7,75 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,3 |
|  |  |  |  |  | DMRS 1+1 | 4,09 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 8,2 |
|  |  |  |  |  | DMRS 1+1 | 8,01 |

Table 4: Simulation results for Transform precoding disabled case 1T8R

| PUSCH 1T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -8,21 |
|  |  |  |  |  | DMRS 1+1 | -10,42 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,44 |
|  |  |  |  |  | DMRS 1+1 | 0,82 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,08 |
|  |  |  |  |  | DMRS 1+1 | 4,01 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -9,8 |
|  |  |  |  |  | DMRS 1+1 | -10,56 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,79 |
|  |  |  |  |  | DMRS 1+1 | 0,73 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,02 |
|  |  |  |  |  | DMRS 1+1 | 3,75 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,97 |
|  |  |  |  |  | DMRS 1+1 | -10,25 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,25 |
|  |  |  |  |  | DMRS 1+1 | 0,8 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,04 |
|  |  |  |  |  | DMRS 1+1 | 4,01 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,27 |
|  |  |  |  |  | DMRS 1+1 | -10,07 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,34 |
|  |  |  |  |  | DMRS 1+1 | 0,71 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,05 |
|  |  |  |  |  | DMRS 1+1 | 3,7 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -10,12 |
|  |  |  |  |  | DMRS 1+1 | -10,55 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,75 |
|  |  |  |  |  | DMRS 1+1 | 0,89 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 3,82 |
|  |  |  |  |  | DMRS 1+1 | 4,07 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -10,28 |
|  |  |  |  |  | DMRS 1+1 | -10,66 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,9 |
|  |  |  |  |  | DMRS 1+1 | 0,83 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 3,99 |
|  |  |  |  |  | DMRS 1+1 | 3,9 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -9,34 |
|  |  |  |  |  | DMRS 1+1 | -10,58 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,79 |
|  |  |  |  |  | DMRS 1+1 | 0,74 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,35 |
|  |  |  |  |  | DMRS 1+1 | 4,19 |

Table 5: Simulation results for Transform precoding disabled case 2T2R

| PUSCH 2T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 0,46 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 18,23 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,58 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,47 |
|  | FR2 | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -0,06 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | Na |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -0,19 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 19,95 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,1 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 17,66 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 0,91 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 18,06 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,77 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,86 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,45 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,67 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,58 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,92 |
|  | FR2 | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | 0,23 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NA |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -0,2 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 20,26 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | -0,16 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 19,68 |

Table 6: Simulation results for Transform precoding disabled case 2T4R

| PUSCH 2T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | -3,66 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 9,15 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,53 |
|  |  |  |  |  | DMRS 1+1 | -4,36 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,63 |
|  |  |  |  |  | DMRS 1+1 | 8,83 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -0,23 |
|  |  |  |  |  | DMRS 1+1 | -3,78 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 9,06 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | -3,37 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 9,03 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,76 |
|  |  |  |  |  | DMRS 1+1 | -4,46 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,59 |
|  |  |  |  |  | DMRS 1+1 | 8,97 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,89 |
|  |  |  |  |  | DMRS 1+1 | -4,27 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,57 |
|  |  |  |  |  | DMRS 1+1 | 8,97 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,17 |
|  |  |  |  |  | DMRS 1+1 | -4,35 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,9 |
|  |  |  |  |  | DMRS 1+1 | 9,06 |

Table 7: Simulation results for Transform precoding disabled case 2T8R

| PUSCH 2T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | -7,02 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 6,07 |
|  |  |  |  |  | DMRS 1+1 | 4,65 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,07 |
|  |  |  |  |  | DMRS 1+1 | -7,39 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,71 |
|  |  |  |  |  | DMRS 1+1 | 4,55 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,53 |
|  |  |  |  |  | DMRS 1+1 | -7,17 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,77 |
|  |  |  |  |  | DMRS 1+1 | 4,53 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | -6,74 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 6,14 |
|  |  |  |  |  | DMRS 1+1 | 4,51 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -7 |
|  |  |  |  |  | DMRS 1+1 | -7,47 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,88 |
|  |  |  |  |  | DMRS 1+1 | 4,7 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,44 |
|  |  |  |  |  | DMRS 1+1 | -7,37 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,8 |
|  |  |  |  |  | DMRS 1+1 | 4,64 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,73 |
|  |  |  |  |  | DMRS 1+1 | -7,38 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,93 |
|  |  |  |  |  | DMRS 1+1 | 4,73 |

Transform precoding enabled

For transform precoding enabled case, the following simulation assumptions were agreed in [1] and refined in [2]:

Table 8: Simulation parameters – Transform precoding enabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | enabled | enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2, 4, 8 | 2 |
| Number of layers | 1 | 1 |
| DMRS type | type 1 | Type 1 |
| Number of DMRS symbols | 1+1, 1 | 1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type B |
| Frequency domain resource | 15kHz: 25 PRB (contig., middle of test BW) 30kHz: 24 PRB (contig., middle of test BW) | 60kHz: 30 PRB (contig., middle of test BW) 120kHz: 30 PRB (contig., middle of test BW) |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz MCS 20: TDL-A 30ns, 10Hz | MCS 2: TDL-A 30ns, 300Hz MCS 16: TDL-A 30ns, 300Hz MCS 20: TDL-A 30ns, 75Hz |
| SCS and BW | 15kHz: 5 MHz 30kHz: 10 MHz | 60kHz: 50 MHz 120kHz: 50 MHz |
| PTRS | Not configured | NRB0=0, NRB1=8, NRB2=NRB3=32, and NRB4=108 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Simulation results with these parameters are shown in Table 9 to Table 11.

Table 9: Simulation results for Transform precoding enabled case 1T2R

| PUSCH 1T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | FR1 | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | 0 |
|  |  |  |  |  | DMRS 1+1 | -4,1 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -3,83 |
|  |  |  |  |  | DMRS 1+1 | -4,47 |
|  | FR2 | 60kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -3,78 |
|  |  | 120kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -3,9 |

Table 10: Simulation results for Transform precoding enabled case 1T4R

| PUSCH 1T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | FR1 | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -4,63 |
|  |  |  |  |  | DMRS 1+1 | -7,48 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -7,3 |
|  |  |  |  |  | DMRS 1+1 | -7,88 |

Table 11: Simulation results for Transform precoding enabled case 1T8R

| PUSCH 1T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | FR1 | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -8,21 |
|  |  |  |  |  | DMRS 1+1 | -10,47 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -10,26 |
|  |  |  |  |  | DMRS 1+1 | -10,75 |

Simulation Results (impaired)

Transform precoding disabled

The simulation parameters are as before in the ideal case (see Table 1). To obtain the impaired result non-ideal effects such as, but not limited to, phase noise and fixed-point implementation, have been considered.

Simulation results with these parameters are shown in Table 12 to Table 17.

Table 12: Simulation results (impaired) for Transform precoding disabled case 1T2R

| PUSCH 1T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | 1,88 |
|  |  |  |  |  | DMRS 1+1 | -1,63 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 13,68 |
|  |  |  |  |  | DMRS 1+1 | 11,18 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 14,15 |
|  |  |  |  |  | DMRS 1+1 | 14,05 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -0,84 |
|  |  |  |  |  | DMRS 1+1 | -1,83 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 11,23 |
|  |  |  |  |  | DMRS 1+1 | 10,91 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 14,3 |
|  |  |  |  |  | DMRS 1+1 | 14,04 |
|  | FR2 | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -0,64 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 13,18 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 15,31 |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,01 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 12,05 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 14,87 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | 2,63 |
|  |  |  |  |  | DMRS 1+1 | -1,19 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 13,06 |
|  |  |  |  |  | DMRS 1+1 | 11,31 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 14,13 |
|  |  |  |  |  | DMRS 1+1 | 14,07 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | 2,5 |
|  |  |  |  |  | DMRS 1+1 | -0,78 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 14,18 |
|  |  |  |  |  | DMRS 1+1 | 11,08 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 14,31 |
|  |  |  |  |  | DMRS 1+1 | 14 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,16 |
|  |  |  |  |  | DMRS 1+1 | -1,76 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 11,14 |
|  |  |  |  |  | DMRS 1+1 | 11,13 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 13,92 |
|  |  |  |  |  | DMRS 1+1 | 14,1 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,38 |
|  |  |  |  |  | DMRS 1+1 | -1,92 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 11,32 |
|  |  |  |  |  | DMRS 1+1 | 11,08 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 14,16 |
|  |  |  |  |  | DMRS 1+1 | 14,11 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -0,29 |
|  |  |  |  |  | DMRS 1+1 | -1,96 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 11,19 |
|  |  |  |  |  | DMRS 1+1 | 10,75 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 14,94 |
|  |  |  |  |  | DMRS 1+1 | 14,79 |
|  | FR2 | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -0,39 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 13,02 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 15,15 |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -0,52 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 12,28 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 15,19 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | -0,91 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 12,19 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 14,95 |

Table 13: Simulation results (impaired) for Transform precoding disabled case 1T4R

| PUSCH 1T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,38 |
|  |  |  |  |  | DMRS 1+1 | -4,99 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,8 |
|  |  |  |  |  | DMRS 1+1 | 6,89 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 10,29 |
|  |  |  |  |  | DMRS 1+1 | 10,24 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,33 |
|  |  |  |  |  | DMRS 1+1 | -5,15 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 6,79 |
|  |  |  |  |  | DMRS 1+1 | 6,65 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 10,16 |
|  |  |  |  |  | DMRS 1+1 | 9,94 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,93 |
|  |  |  |  |  | DMRS 1+1 | -4,71 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,52 |
|  |  |  |  |  | DMRS 1+1 | 6,9 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 10,26 |
|  |  |  |  |  | DMRS 1+1 | 10,21 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | 1,48 |
|  |  |  |  |  | DMRS 1+1 | -4,46 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,84 |
|  |  |  |  |  | DMRS 1+1 | 6,75 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 10,22 |
|  |  |  |  |  | DMRS 1+1 | 9,89 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,62 |
|  |  |  |  |  | DMRS 1+1 | -5,09 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 6,74 |
|  |  |  |  |  | DMRS 1+1 | 6,88 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 10,1 |
|  |  |  |  |  | DMRS 1+1 | 10,25 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,81 |
|  |  |  |  |  | DMRS 1+1 | -5,23 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 6,94 |
|  |  |  |  |  | DMRS 1+1 | 6,78 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 10,3 |
|  |  |  |  |  | DMRS 1+1 | 10,21 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,83 |
|  |  |  |  |  | DMRS 1+1 | -5,25 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 6,8 |
|  |  |  |  |  | DMRS 1+1 | 6,59 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 10,7 |
|  |  |  |  |  | DMRS 1+1 | 10,51 |

Table 14: Simulation results (impaired) for Transform precoding disabled case 1T8R

| PUSCH 1T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,71 |
|  |  |  |  |  | DMRS 1+1 | -7,92 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,94 |
|  |  |  |  |  | DMRS 1+1 | 3,32 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,58 |
|  |  |  |  |  | DMRS 1+1 | 6,51 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,3 |
|  |  |  |  |  | DMRS 1+1 | -8,06 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,29 |
|  |  |  |  |  | DMRS 1+1 | 3,23 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,52 |
|  |  |  |  |  | DMRS 1+1 | 6,25 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,47 |
|  |  |  |  |  | DMRS 1+1 | -7,75 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,75 |
|  |  |  |  |  | DMRS 1+1 | 3,3 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,54 |
|  |  |  |  |  | DMRS 1+1 | 6,51 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,77 |
|  |  |  |  |  | DMRS 1+1 | -7,57 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,84 |
|  |  |  |  |  | DMRS 1+1 | 3,21 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,55 |
|  |  |  |  |  | DMRS 1+1 | 6,2 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,62 |
|  |  |  |  |  | DMRS 1+1 | -8,05 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,25 |
|  |  |  |  |  | DMRS 1+1 | 3,39 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,32 |
|  |  |  |  |  | DMRS 1+1 | 6,57 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,78 |
|  |  |  |  |  | DMRS 1+1 | -8,16 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,4 |
|  |  |  |  |  | DMRS 1+1 | 3,33 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,49 |
|  |  |  |  |  | DMRS 1+1 | 6,4 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,84 |
|  |  |  |  |  | DMRS 1+1 | -8,08 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,29 |
|  |  |  |  |  | DMRS 1+1 | 3,24 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,85 |
|  |  |  |  |  | DMRS 1+1 | 6,69 |

Table 15: Simulation results (impaired) for Transform precoding disabled case 2T2R

| PUSCH 2T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 2,96 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 20,73 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 1,92 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 18,97 |
|  | FR2 | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | 3,04 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NA |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | 2,91 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 23,05 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 2,4 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 20,16 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 3,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 20,56 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 1,73 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 19,36 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 2,05 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 19,17 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 1,92 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 19,42 |
|  | FR2 | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | 3,33 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NA |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | 2,9 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 23,36 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | 2,94 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 22,78 |

Table 16: Simulation results (impaired) for Transform precoding disabled case 2T4R

| PUSCH 2T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | -1,16 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 11,65 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -0,03 |
|  |  |  |  |  | DMRS 1+1 | -1,86 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 12,13 |
|  |  |  |  |  | DMRS 1+1 | 11,33 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | 2,27 |
|  |  |  |  |  | DMRS 1+1 | -1,28 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 11,56 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | 2,5 |
|  |  |  |  |  | DMRS 1+1 | -0,87 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 11,53 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,26 |
|  |  |  |  |  | DMRS 1+1 | -1,96 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 12,09 |
|  |  |  |  |  | DMRS 1+1 | 11,47 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -0,39 |
|  |  |  |  |  | DMRS 1+1 | -1,77 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 12,07 |
|  |  |  |  |  | DMRS 1+1 | 11,47 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | 0,33 |
|  |  |  |  |  | DMRS 1+1 | -1,85 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 12,4 |
|  |  |  |  |  | DMRS 1+1 | 11,56 |

Table 17: Simulation results (impaired) for Transform precoding disabled case 2T8R

| PUSCH 2T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | -4,52 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,57 |
|  |  |  |  |  | DMRS 1+1 | 7,15 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,57 |
|  |  |  |  |  | DMRS 1+1 | -4,89 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,21 |
|  |  |  |  |  | DMRS 1+1 | 7,05 |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,03 |
|  |  |  |  |  | DMRS 1+1 | -4,67 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,27 |
|  |  |  |  |  | DMRS 1+1 | 7,03 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | -4,24 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,64 |
|  |  |  |  |  | DMRS 1+1 | 7,01 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,5 |
|  |  |  |  |  | DMRS 1+1 | -4,97 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,38 |
|  |  |  |  |  | DMRS 1+1 | 7,2 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,94 |
|  |  |  |  |  | DMRS 1+1 | -4,87 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,3 |
|  |  |  |  |  | DMRS 1+1 | 7,14 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,23 |
|  |  |  |  |  | DMRS 1+1 | -4,88 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,43 |
|  |  |  |  |  | DMRS 1+1 | 7,23 |

Transform precoding enabled

The simulation parameters are as before in the ideal case (see Table 8). To obtain the impaired result non-ideal effects such as, but not limited to, phase noise and fixed-point implementation, have been considered.

Simulation results with these parameters are shown in Table 18 to Table 20.

Table 18: Simulation results (impaired) for Transform precoding enabled case 1T2R

| PUSCH 1T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | FR1 | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | 2,5 |
|  |  |  |  |  | DMRS 1+1 | -1,6 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -1,33 |
|  |  |  |  |  | DMRS 1+1 | -1,97 |
|  | FR2 | 60kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -0,68 |
|  |  | 120kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -0,8 |

Table 19: Simulation results (impaired) for Transform precoding enabled case 1T4R

| PUSCH 1T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | FR1 | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -2,13 |
|  |  |  |  |  | DMRS 1+1 | -4,98 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -4,8 |
|  |  |  |  |  | DMRS 1+1 | -5,38 |

Table 20: Simulation results (impaired) for Transform precoding enabled case 1T8R

| PUSCH 1T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | FR1 | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -5,71 |
|  |  |  |  |  | DMRS 1+1 | -7,97 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -7,76 |
|  |  |  |  |  | DMRS 1+1 | -8,25 |

Conclusion

In this contribution we have provided simulation results for NR PUSCH test cases.

- References

- R4-1813942, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018

- R4-1816347, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#89, Spokane, US, 12 – 16 November 2018

- R4-1816594, WF for test case SNR derivation procedure, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#89, Spokane, US, 12 – 16 November 2018.