3GPP TSG-RAN4 Meeting #90	R4-1902598

# Athens, Greece, Feb 25 – Mar 1, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on test case for handover in FR2 (section A.6.3.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2019-01-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TRS configuraiton is TBD in current FR2 handover test case. Handover delay and interruption length in test requirement is TBD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce TRS configuration - Remove TBD in test requirements - Introduce TCI configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TRS configuration is missing in FR2 handover test case. There would be still many TBDs in teest requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

### A.7.3.1	Handover

#### A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell

##### A.7.3.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR2 inter frequency handover requirements specified in clause 6.1.1.5 [TS38.133 v15.2.1].

##### A.7.3.1.1.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.2.2-1. Both handover delay and interruption lengthare tested by using the parameters in table A.7.3.1.1.2-2, and A.7.3.1.1.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.7.3.1.1.2-1: Inter-frequency handover from FR1 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.3.1.1.2-2: General test parameters Inter-frequency handover from FR1 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Offset |  | dB | [TBD] |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.7.3.1.1.2-3: Cell specific test parameters for NR FR1-FR2 Inter frequency handover test case

| Parameter |  |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  | TDD |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  | TDDConf.3.1 |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  | TDDConf.3.1 |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  | 100: NRB,c = 66 |  |  |  |
| BWP BW |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  | 100: NRB,c = 66 |  |  |  |
| DRx Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |  |  | SR3.1 TDD |  |  |  |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |  | SR3.1 TDD |  |  |  |
|  |  |  | Config 3 |  | SR2.1 TDD |  |  | SR3.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  |  | CR3.1 TDD |  |  |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |  | CR3.1 TDD |  |  |  |
|  |  |  | Config 3 |  | CR2.1 TDD |  |  | CR3.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC configuration |  |  | Config 1,2 |  | SMTC.1 FR1 |  |  | SMTC.1 FR2 |  |  |  |
|  |  |  | Config 3 |  | SMTC.2 FR1 |  |  | SMTC.1 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 kHz |  |  | 120 kHz |  |  |  |
|  |  |  | Config 3 |  | 30 kHz |  |  | 120 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 kHz |  |  | 120 kHz |  |  |  |
|  |  |  | Config 3 |  | 30 kHz |  |  | 120 kHz |  |  |  |
| PRACH configuration |  |  |  |  | FR1 PRACH configuration 1 |  |  | FR2 PRACH configuration 1 |  |  |  |
| TRS configuration |  | Config 1 |  |  | N/A |  |  | TRS.2.1 TDD |  |  |  |
|  |  | Config 2 |  |  | N/A |  |  | TRS.2.1 TDD |  |  |  |
|  |  | Config 3 |  |  | N/A |  |  | TRS.2.1 TDD |  |  |  |
| TCI configuration |  |  |  |  | N/A |  |  | CSI-RS.Config.0 |  |  |  |
| BWP configuraiton |  |  | Initial DL BWP |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |  |
|  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |  |
|  |  |  | Initial UL BWP |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |  |
|  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | dBm/15kHz | TBD |  |  | TBD |  |  |  |
| Note2 | Config 1,2 |  |  | dBm/SCS | TBD |  |  | TBD |  |  |  |
|  | Config 3 |  |  |  | TBD |  |  | TBD |  |  |  |
|  |  |  |  | dB | TBD | TBD |  | -Infinity |  | TBD |  |
|  |  |  |  | dB | TBD | TBD |  | -Infinity |  | TBD |  |
| IoNote3 | Config 1,2 |  |  | dBm/ BW | TBD | TBD |  | TBD |  | TBD |  |
|  | Config 3 |  |  | dBm/ BW | TBD | TBD |  | TBD |  | TBD |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone |  |  |  |  |  |  |  |  |  |  |  |

##### A.7.3.1.1.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [722] ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = [10] ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = [712] ms in the test. Tinterrupt is defined in clause 6.1.1.5.2 [TS38.133].

This gives a total of [722] ms.

#### A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell

##### A.7.3.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 intra frequency handover requirements specified in clause 6.1.1.4 [TS38.133 v15.2.1].

##### A.7.3.1.2.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.2.2-1. Both handover delay and interruption lengthare tested by using the parameters in table A.7.3.1.2.2-2, and A.7.3.1.2.2-3.

The test scenario comprises of carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.7.3.1.2.2-1: Intra-frequency handover from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.1.2.2-2: General test parameters Intra-frequency handover from FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Offset |  | dB | [TBD] |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.7.3.1.2.2-3: Cell specific test parameters for NR FR2-FR2 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| TCI configuration |  |  |  | CSI-RS.Config.0 |  |  |  |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | TBD |  |  | TBD |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | TBD |  |  | TBD |  |  |  |
|  | Config 3 |  |  | TBD |  |  | TBD |  |  |  |
|  |  |  | dB | TBD | TBD |  | TBD |  | TBD |  |
|  |  |  | dB | TBD | TBD |  | TBD |  | TBD |  |
| IoNote3 | Config 1,2 |  | dBm/ BW | TBD | TBD |  | TBD |  | TBD |  |
|  | Config 3 |  | dBm/ BW | TBD | TBD |  | TBD |  | TBD |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone |  |  |  |  |  |  |  |  |  |  |

##### A.7.3.1.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [382] ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = [10] ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = [372] ms in the test. Tinterrupt is defined in clause 6.1.1.4.2 [TS38.133].

This gives a total of [382] ms.

#### A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell

##### A.7.3.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 inter frequency handover requirements specified in clause 6.1.1.4 [TS38.133 v15.2.1].

##### A.7.3.1.3.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.3.2-1. Both handover delay and interruption lengthare tested by using the parameters in table A.7.3.1.3.2-2, and A.7.3.1.3.2-3.

The test scenario comprises of carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.7.3.1.3.2-1: Inter-frequency handover from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.1.3.2-2: General test parameters Inter-frequency handover from FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Offset |  | dB | [TBD] |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.7.3.1.3.2-3: Cell specific test parameters for NR FR2-FR2 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| TCI configuration |  |  |  | CSI-RS.Config.0 |  |  |  |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | TBD |  |  | TBD |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | TBD |  |  | TBD |  |  |  |
|  | Config 3 |  |  | TBD |  |  | TBD |  |  |  |
|  |  |  | dB | TBD | TBD |  | TBD |  | TBD |  |
|  |  |  | dB | TBD | TBD |  | TBD |  | TBD |  |
| IoNote3 | Config 1,2 |  | dBm/ BW | TBD | TBD |  | TBD |  | TBD |  |
|  | Config 3 |  | dBm/ BW | TBD | TBD |  | TBD |  | TBD |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone |  |  |  |  |  |  |  |  |  |  |

##### A.7.3.1.3.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [702] ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = [10] ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = [692] ms in the test. Tinterrupt is defined in clause 6.1.1.4.2 [TS38.133].

This gives a total of [702] ms.

End of Change 1