3GPP TSG-RAN WG4 RAN4-90	R4-1902658

# Athens, Greece, 25 February - 1 March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0034 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02 adding background of RX exclusion band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Important to have clarification on the Exclusion Bands definition for Radiated Immunity testing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of background elements on the definition of Exclusion Bands for Radiated Immunity testing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of background supporting the decisions made on Exclusion Bands size. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Modified section ------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R M.1036-5 (10/2015), "Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations (RR)".

[3]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[4]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[5]	ITU-R Recommendation SM.328: "Spectra and bandwidth of emissions".

[6]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[7]	3GPP TS 37.105: "Active Antenna System (AAS) Base Station (BS) transmission and reception".

[8]	3GPP TR 37.842: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)".

[9]	3GPP TR 37.843: "Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) radiated requirements".

[10]	R4-1700305, "LS on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz".

[11]	Code of Federal Regulations, Title 47, Part 30.203, Upper Microwave Flexible Use Service; Emission limits, Federal Communications Commission.

[12]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[13]	Void.

[14]	ETSI EN 301 489: "Electromagnetic compatibility and Radio spectrum Matters (ERM); ElectroMagnetic Compatibility (EMC) standard for radio equipment and services".

[15]	3GPP TS 38.113: "NR; Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)".

[16]	3GPP TS 37.114: "Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)".

[17]	3GPP TS 38.141-1: "NR; Base Station (BS) conformance testing; Part 1: Conducted conformance testing".

[18]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing".

[19]	3GPP TS 37.104: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception".

[20]	3GPP TS 38.817-01: "General aspects for User Equipment (UE) Radio Frequency (RF) for NR".

[21]	3GPP TR 36.815: "Further Advancements for E-UTRA; LTE-Advanced feasibility studies in RAN WG4".

[22]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[23]	3GPP TS 37.113: "Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)".

[24]	3GPP TR 38.803: "Study on new radio access technology: Radio Frequency (RF) and co-existence aspects".

[25]	3GPP TS 38.211: "NR; Physical channels and modulation".

[26]	3GPP TR 37.843: "Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) radiated requirements". v15.2.0

[27]	IEC 61000-4-3: 2006+AMD1:2007+AMD2:2010: “Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test”

[28]	ITU-T Recommendation K.114: “Electromagnetic compatibility requirements and measurement methods for digital cellular mobile communication base station equipment”

[29]	ITU-T Recommendation K.48: “EMC requirements for telecommunication equipment - Product family Recommendation”

- ------------------------------ Next Modified Section ------------------------------

## 11.4	Radiated immunity requirements

Radiated Immunity (RI) test assesses the ability of the EUT to operate as intended in the presence of a radio frequency electromagnetic field disturbance [14]. The theory behind the RI test is that the EUT will encounter many different types of electric field disturbances (e.g., motors, cell phones) in normal usage. Then, the RI test is intended to see that the susceptibility of the equipment is such that it can co-exist with other electronic equipment and operate without degradation across a range of frequencies.

RI testing requires a frequency scan at a certain fixed level of field strength specified by the standard. For the immunity requirements, 80 – 6000 MHz frequency range applies based on ETSI EN 301 489 [14]. The 'scan' will comprise a series of 'steps' in frequency. Each step is specified as a percentage of current frequency value. This percentage is variable from 0.2 % to 5 %. At each step, the frequency is held, the level adjusted to the required field strength (V/m) as measured by a field sensor, a prescribed modulation mode is initiated and then the conditions held for a 'dwell' time. The EUT should be monitored to detect faulty operation during the test. Typical values for field strength are 3 V/m for domestic and 10 V/m for industrial products.

### 11.4.1 	Measurement set-up for testing radiated immunity

In an EMC RI testing test setup for non-AAS BS products, the antenna connectors of the BS are terminated during test to prevent high power levels entering the RX of the EUT. For the AAS BS architectures such as BS type 1-O, due to lack of the antenna connectors (called TAB connectors in case of AAS BS products), it is not possible to terminate TAB connectors and prevent disturbances affecting the RX chain. This may lead to the receiver of the BS type 1-O being inoperable (in case the RF blocking requirements are not met). To avoid the BS type 1-O receiver being inoperable during the EMC RI test, protection mechanisms need to be employed for BS RX during the RI testing.

A high level example of EMC RI test setup for an OTA AAS BS is illustrated in figure 11.4.1-1. The Equipment Under Test (EUT) is placed in an anechoic chamber on top of a turn table. A range antenna or probe antenna generates the interfering RF electromagnetic field.

Figure 11.4.1-1: Example radiated immunity test setup for a BS type 1-O

An important difference of the EMC radiated immunity test setup for an BS type 1-O as compared with testing of the EUT equipped with the antenna connectors is the need for OTA performance parameter monitoring. During EMC RI test the EUT is rotated horizontally in order to expose it to the interfering RF electromagnetic field from all directions in horizontal plane.

According to IEC 61000-4-3 [25], all testing of equipment shall be performed in a configuration as close as possible to actual installation conditions. In order to allow electromagnetic susceptibility testing of the maximum surface for BS type 1-O, the EUT shall be mounted using rack (or pole) mount, if possible.

### 11.4.2 	Alternatives to protect BS type 1-O during RI test

3GPP has agreed on the use of two mechanisms to protect the BS during RI testing:

- Exclusion bands, or

- Spatial exclusion.

The combined use of these two alternatives is reflected in both AAS BS EMC specification TS 37.114 [16] and the NR BE EMC specification TS 38.113 [15].

# 11.4.2.1	Exclusion bands

Exclusion bands were considered as a mechanism to protect the receiver from potential damage/undue stress during the RI test of the non-AAS BS products so far, where the exclusion bands represent a range of frequencies over which no tests are made.

During the discussion of Release 15, the need for widening the size of the exclusion bands in TS 37.113 [12], TS 37.114 [2] and TS 38.113 [3] has been recognized. The parameter proposed to get this extension was the boundary of the in-band and out-of-band region, represented by the maximum offset of the out-of-band boundary from the uplink operating band edge (i.e. ∆fOOB). The value of ∆fOOB was derived considering the width of the NR operating band and was agreed to be:

- 20 MHz, for operating bands narrower then 100 MHz, or

- 60 MHz, for operating bands wider then 100 MHz.

The above ∆fOOB values considered the use of spatial exclusion during the EMC RI testing.

Before the significant risk to over-dimension the Rx band filter for BS type 1-O, the need for using a value higher than ∆fOOB to calculate the exclusion bands size was agreed for the test case with no spatial exclusion being considered. In this case the exclusion band takes into account the width of the operating band as follows:

- 60 MHz, for operating bands narrower then 100 MHz, or

- 200 MHz, for operating bands wider then 100 MHz.

# 11.4.2.2	Spatial exclusion

According to IEC 61000-4-3 [25], the EMC Radiated Immunity test are performed with the generating antenna facing each side of the EUT. When technically justified, some EUTs can be tested by exposing fewer faces to the generating antenna. For BS type 1-O the RI test need to be reduced to fewer angular incidences. This is to protect the EUT receiver from being blocked due to received input power levels exceeding the OOB blocking limits during the RI testing. Referring to the IEC 61000-4-3 [25] guidance above, the angular incidence where the front of the BS type 1-O is facing the interferer range antenna needs to be omitted, as depicted in figure 11.4.2.2-1.

Figure 11.4.2.2-1: RI testing directions for BS type 1-O (horizontal plane depicted)

------------------------------ End of Modified Section ------------------------------