3GPP TSG-RAN WG4 Meeting #90bis 	R4-1902943

Xi’an, China, 8 - 12 April, 2019

Agenda item:       6.10.8.2.1

Source:                Intel Corporation

Title:	                     Discussion on Scale factor N for Candidate Beam Detection Requirements

Document for:     Discussion

# Introduction

In RAN4#90 several open issues related to requirements for candidate beam detection were discussed and resolved. One open item for CBD requirements was evaluation period in FR2.

In this contribution we present our views on evaluation period for CSI-RS based candidate beam detection in FR2.

# Discussion

In TS 38.133, the evaluation period for CSI-RS based CBD in FR2 is defined in Table 8.5.6.2-2 as following:

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX | max([25], ceil(MCBD *P*N) * TCSI-RS) |
| DRX cycle ≤ 320ms | ceil(MCBD *P*N*1.5) * max(TDRX, TCSI-RS) |
| DRX cycle > 320ms | ceil(MCBD *P*N) *TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

The value of TEvaluate_CBD_CSI-RS is defined in Table 8.5.6.2-2 for FR2 with N=8.

Editor’s Note: FFS whether N=1 need to be applied for CSI-RS based candidate beam detection in FR2.

The Rx beam scaling factor in FR2 is defined as N=8. However there is an Editor’s note if N=1 is applicable for CSI-RS based BFD.

When the UE is making measurements to find suitable candidate beam following beam failure, the UE’s Rx beam information for any CBD resources based on previous measurements might be invalid, hence the UE is required to perform beam refinement and determine if suitable candidate beams are available for beam failure recovery.

Observation #1: After beam failure indication by UE, Rx beam information based on prior measurements might be invalid

For beam failure recovery, the UE needs to perform Rx beam refinement on resources configured for CBD and N=8 shall always apply in evaluation period for FR2.

Observation #2: For beam failure recovery UE needs to perform Rx beam refinement on CBD resources and N=8 applies for evaluation period in FR2

Hence we recommend removing the Editor’s note for evaluation period in FR2 for CSI-RS based CBD requirements.

Proposal #1: Remove Editor’s note: FFS whether N=1 need to be applied for CSI-RS based candidate beam detection in FR2.

# Conclusion

In this paper we present our views on evaluation period for for CSI-RS based candidate beam detection in FR2 and have the following observations and proposals:

Observation #1: After beam failure indication by UE, Rx beam information based on prior measurements might be invalid

Observation #2: For beam failure recovery UE needs to perform Rx beam refinement on CBD resources and N=8 applies for evaluation period in FR2

Proposal #1: Remove Editor’s note: FFS whether N=1 need to be applied for CSI-RS based candidate beam detection in FR2.

# Reference