3GPP TSG- RAN4 Meeting #90bis	R4-1902959

# Xi’an, China, 8 - 12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Method to derive Minimum SSB_RP side condition values in Annex B.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The method to derive SSB_RP side condition values for FR2 measurements is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add derivation of Minimum SSB_RP values for FR2, with separate subcluases for Rx Beam Peak direction and angles within the Spherical coverage directions. For power classes 1, 2 and 4 the value of Y is FFS. For all power classes the value of Z is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The method to derive SSB_RP side conditions over which the FR2 accuracy requirement applies would be unrecorded, and the technical basis for future updates would be difficult to find. The method used here is consistent with that already used for FR2 demodulation noise level in TS 38.101-4 clause 4.5.3.3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.1.1, B.2.1.2, B.2.1.3, B.2.1.3.1, B.2.1.3.2 (all new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

Annex B (normative):
Conditions for RRM requirements applicability for operating bands

# B.1	Conditions for NR RRC_IDLE state mobility

## B.1.1	Introduction

In Annex B.1, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurements procedures and requirements in Section 4,

-	UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in Section 4.

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

This section defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

Table B.1.2-2: Conditions for intra-frequency cell re-selection in FR2

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |
| Conditions | NR_TDD_FR2_A | TBD | TBD | TBD |
|  | NR_TDD_FR2_B | TBD | TBD |  |
|  | NR_TDD_FR2_F | TBD | TBD |  |
|  | NR_TDD_FR2_G | TBD | TBD |  |
|  | NR_TDD_FR2_T | TBD | TBD |  |
|  | NR_TDD_FR2_Y | TBD | TBD |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.3. |  |  |  |  |

## B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection

This section defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.1.2-1 for FR1 NR intra-frequency cell re-selection shall also apply for FR1 NR inter-frequency cells in this section.

The conditions defined in Table B.1.2-2 for FR2 NR intra-frequency cell re-selection shall also apply for FR2 NR inter-frequency cells in this section.

# B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state

## B.2.1	Introduction

#### B.2.1.1	General

In Annex B.2, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurements procedures and requirements in Section 9,

UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in Section 9,

-	UE conditions which shall apply for UE intra-frequency measurements performance requirements in Section 10,

-	UE conditions which shall apply for UE inter-frequency measurements performance requirements in Section 10.

- The conditions for RRC connection release with redirection to NR requirements in Section 6.2.3.2.1.

#### B.2.1.2	Derivation of Minimum SSB_RP values for FR1

[FFS]

#### B.2.1.3	Derivation of Minimum SSB_RP values for FR2

##### B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival

Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on Reference sensitivity for the Operating band and for the UE Power class, and taking a baseline of UE Power class 3 in Band n260 with 50MHz channel bandwidth.

Minimum SSB_RP = Reference sensitivity PC3, n260, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ΣMBP

where:

EIS Reference sensitivity PC3, n260, 50MHz is the Reference sensitivity value in dBm specified for Power Class 3 in Band n260 for 50MHz Channel bandwidth in TS 38.101-2 [19] Table 7.3.2.3-1.

Y is the gain difference between fine and rough beams, and is 7dB for a Power Class 3 UE. The value for other Power Classes is defined in Table B.2.1.3.1-1.

Table B.2.1.3.1-1: Gain difference Y between fine and rough beams, Rx beam peak direction

| Value “Y” in dB, for each UE Power class |  |  |  |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| FFS | FFS | 7.0 | FFS |

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-2 [19] Table 5.3.2-1, and is 32.

12 is the number of subcarriers in a PRB.

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1dB.

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6dB for intra-frequency measurements and -4dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise.

ΣMBP is the UE multi-band relaxation factor value in dB specified in TS 38.101-2 [19] clause 6.2.1.

The calculated Minimum SSB_RP value for the baseline of UE Power class 3 in Band n260 is (-109.5+ΣMBP) dBm/120kHz for intra-frequency measurements and (-107.5+ΣMBP) dBm/120kHz for inter-frequency measurements.

The following methodology to define the Minimum SSB_RP level for power class X (PC_X) and operating band Y (Band_Y) is used:

For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = -109.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +ΣMBP

For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = -107.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +ΣMBP

Editor’s notes:

- May need to change Power Class and Band terminology (to A, B?) to avoid confusion with Y, Z dB values

- If changed, would also need to align TS 38.101-4 clause 4.5.3.3, and also TR 38.810

##### B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage

Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on EIS Spherical coverage for the Operating band and for the UE Power class, and taking a baseline of UE Power class 3 in Band n260 with 50MHz channel bandwidth.

Minimum SSB_RP = EIS spherical coverage PC3, n260, 50MHz +Z -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ΣMBS

where:

EIS spherical coverage PC3, n260, 50MHz is the EIS spherical coverage value in dBm specified for Power Class 3 in Band n260 for 50MHz Channel bandwidth in TS 38.101-2 [19] Table 7.3.4.3-1.

Z is the gain difference between fine and rough beams, and is [FFS]dB for a Power Class 3 UE. The value for other Power Classes is defined in Table B.2.1.3.2-1.

Table B.2.1.3.2-1: Gain difference Z between fine and rough beams, Spherical coverage directions

| Value “Z” in dB, for each UE Power class |  |  |  |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| FFS | FFS | FFS | FFS |

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-2 [19] Table 5.3.2-1, and is 32.

12 is the number of subcarriers in a PRB.

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1dB.

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6dB for intra-frequency measurements and -4dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise.

ΣMBS is the UE multi-band relaxation factor value in dB specified in TS 38.101-2 [19] clause 6.2.1.

The calculated Minimum SSB_RP value for the baseline of UE Power class 3 in Band n260 is (-103.9+ΣMBS +Z) dBm/120kHz for intra-frequency measurements and is (-101.9+ΣMBS +Z) dBm/120kHz for inter-frequency measurements.

The following methodology to define the Minimum SSB_RP level for power class X (PC_X) and operating band Y (Band_Y) is used:

For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = (-103.9+ΣMBS +Z) dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Z PC_X – Z PC3 +ΣMBS

For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = (-101.9+ΣMBS +Z) dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Z PC_X – Z PC3 +ΣMBS

Editor’s notes:

- May need to change Power Class and Band terminology (to A, B?) to avoid confusion with Y, Z dB values

- If changed, would also need to align TS 38.101-4 clause 4.5.3.3, and also TR 38.810

## B.2.2	Conditions for NR intra-frequency measurements

This section defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |
| Conditions | NR_TDD_FR2_A | TBD | TBD | TBD |
|  | NR_TDD_FR2_B | TBD | TBD |  |
|  | NR_TDD_FR2_F | TBD | TBD |  |
|  | NR_TDD_FR2_G | TBD | TBD |  |
|  | NR_TDD_FR2_T | TBD | TBD |  |
|  | NR_TDD_FR2_Y | TBD | TBD |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.3. |  |  |  |  |

## B.2.3	Conditions for NR inter-frequency measurements

This section defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |
| Conditions | NR_TDD_FR2_A | TBD | TBD | TBD |
|  | NR_TDD_FR2_B | TBD | TBD |  |
|  | NR_TDD_FR2_F | TBD | TBD |  |
|  | NR_TDD_FR2_G | TBD | TBD |  |
|  | NR_TDD_FR2_T | TBD | TBD |  |
|  | NR_TDD_FR2_Y | TBD | TBD |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.3. |  |  |  |  |

<< End of changes>>