3GPP TSG RAN WG4 Meeting #90bis	R4-1902971

Xi'an, China, 8 - 12 April 2019

Agenda item:	9.4.3

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-11: EN-DC_1-11_n257

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_1-11_n257 was approved in RAN#83[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n257 | CA_1-11 | n257 | No |

Table 2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n257A | DC_1A_n257A DC_11A_n257A | CA_1A-11A | n257A |
| DC_1A-11A_n257D | DC_1A_n257A DC_11A_n257A | CA_1A-11A | n257D |

3.  Co-existence

Since the 2ULs supported are FR1+FR2, no IMD/harmonics needs to be addressed.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked from [2].

Table 3: ΔTIB of constituent 2 band combo.

| ΔTIB | B1+B11 | B1+n257 | B11+n257 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0.3 | 0 |  | 0.3 |
| B11 | 0.3 |  | 0 | 0.3 |
| n257 |  | 0 | 0 | 0 |

Table 4: ΔRIB of constituent 2 band combo.

| ΔRIB | B1+B11 | B1+n257 | B11+n257 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0 | 0 |  | 0 |
| B11 | 0 |  | 0 | 0 |
| n257 |  | 0 | 0 | 0 |

Then the proposal for this 2LTE-1NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n257 | 1 | 0.3 |
|  | 11 | 0.3 |
|  | n257 | 0 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n257 | 1 | 0 |
|  | 11 | 0 |
|  | n257 | 0 |

5.  REFSENS

As mentioned in section 3, for 1-11_n257 EN-DC combination, no new REFSENS relaxation is introduced as 2UL configurations are FR1+FR2.

6.  Conclusion

This paper proposes study results of EN-DC 1-11_n257.

TP is shown in Annex.

7.  Reference

[1]  RP-190379  WID revision: EN-DC for 3 bands DL with 2 bands UL (2 LTE bands + 1 NR band); rapporteur: Huawei, Huawei, HiSilicon

[2]  TR37.863-01-01

Annex.   Proposed text for TR37.716-21-11(v0.3.0): LTE 2B -NR 1B DC

[Unchanged Parts Skipped]

5.1.x	DC_1-11_n257

### 5.1.x.1	Operating bands for DC_1-11_n257

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n257 | CA_1-11 | n257 | No |

### 5.1.x.2	Configurations for DC_1-11_n257

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n257A | DC_1A_n257A DC_11A_n257A | CA_1A-11A | n257A |
| DC_1A-11A_n257D | DC_1A_n257A DC_11A_n257A | CA_1A-11A | n257D |

### 5.1.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n257 | 1 | 0.3 |
|  | 11 | 0.3 |
|  | n257 | 0 |

Table 5.1.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n257 | 1 | 0 |
|  | 11 | 0 |
|  | n257 | 0 |

[Unchanged Parts Skipped]