3GPP TSG-WG RAN4 Meeting #90bis	R4-1903004

# Xian, China, 8th – 12th April, 2019

Source: 	Nokia, Nokia Shanghai Bell, [AT&T]

Title: 					TP for TR 37.716-21-11 Introduction of DC_2A-30A_n5A and DC_2A-2A-30A_n5A

Agenda Item: 			9.4.2 [DC_R16_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# Introduction

This TP introduces two-band EN-DC configurations of DC_2A-30A_n5A and DC_2A-2A-30A_n5A to TR 37.716-21-11.

**************************** Start of changes ********************************************

## 5.1.X	DC_2-30_n5 and DC_2-2-30_n5

### 5.1.X.1	Operating bands for DC

Table 5.1.X.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-30_n5 | CA_2-30 | n5 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-30A_n5A | DC_2A_n5A DC_30A_n5A | CA_2A-30A | n5A |
| DC_2A-2A-30A_n5A | DC_2A_n5A DC_30A_n5A | CA_2A-2A-30A | n5A |

### 5.1.X.3	Coexistence studies

Based on co-existence studies of DC_2A_n5A and DC_30A_n5A in TR 37.863-01-01, there is no harmonic or IMD impact to any of three own Rx bands.

### 5.1.X.4	∆TIB and ∆RIB values

For DC_2-30_n5 and DC_2-2-30_n5, the TIB,c and RIB,c values are derived from CA_2-5-30 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-30_n5, DC_2-2-30_n5 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n5 | 0.3 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-30_n5, DC_2-2-30_n5 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n5 | 0 |

### 5.1.X.5	REFSENS requirements

There is no additional requirement for this band combination.

**************************** End of changes ********************************************