3GPP TSG-WG RAN4 Meeting #90bis	R4-1903082

# Xian, China, 8th – 12th April, 2019

Source: 	Nokia, Nokia Shanghai Bell

Title:	TP for TR 37.716-11-11 DC_1A_n258A and DC_1A_n258D

Agenda Item: 			9.3.3 [DC_R16_1BLTE_1BNR_2DL2UL-Core]

Document for:	Approval

# 1	Introduction

###### This contribution is a TP for TR 37.716-11-11 to introduce the option for DC_1A_n258A and DC_1A_n258D.

### DC band combination of LTE 1DL/1UL + one NR band: Specific Band Combination Part

| combo | EN-DC configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes and intra contiguous/non-contiguous CA (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1_n258 | DL_1A_n258A_UL_1A_n258A | Rel-15 | Nokia | hisashi.onozawa@nokia.com | Huawei, Ericsson, Qualcomm | new | NONE |
| 1_n258 | DL_1A_n258D_UL_1A_n258D | Rel-15 | Nokia | hisashi.onozawa@nokia.com | Huawei, Ericsson, Qualcomm | new | DL_1A_n258A_UL_1A_n258A (new) |

*********************** Start of the TP ***************************************************

## 6.x	DC_1A_n258A and DC_1A_n258D

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n258A, DC_1A-n258D | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6. x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n258A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n258 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |
| DC_1A_n258D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n258 | See CA_n258D BCS 0 in Table 5.5A.1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Table 6.x.3-1 lists Band 1 +Band n258 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.x.3-1: Band 1 and Band n258 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 24250 | 27500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 48500 | 55000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 72750 | 82500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 22270 | 25580 | 26170 | 29480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 23660 | 20290 | 46520 | 53080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28090 | 31460 | 50420 | 56980 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1520 | 2380 | 24230 | 27520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 21740 | 18310 | 70770 | 80580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 51160 | 44540 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 30010 | 33440 | 74670 | 84480 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 52340 | 58960 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 108080 | 95020 | 16330 | 19820 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 78660 | 68790 | 42560 | 49240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 98920 | 111980 | 31930 | 35420 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 76590 | 86460 | 54260 | 60940 |

Based on Table 6.x.3-1;

- 3rd order harmonic may fall into Rx frequencies of bands 46 and 47

- 2nd order IMD may fall into Rx frequencies of bands n257, n258 and n261

- 3rd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 10, 24, 25, 30, 33, 34, 35, 36, 37, 39, 40, 65, 66, 70, n257 and n258

It should be noted that 3rd order IMD may fall into own Rx of band 1 and that IMD will not be an issue for band n258 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band. When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.x.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.x.3-2: 2UL Band 1 +Band n258 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3 |

Table 6.x.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.x.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-n258A, DC_1A-n258D | E-UTRA Band 1, 2, 3, 4, 10, 24, 25, 30, 33, 34, 35, 36, 37, 39, 40, 46, 47, 65, 66 and 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257, n258 and n261 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |

### 6.x.4	∆TIB and ∆RIB values

For DC_1A_n258A and DC_1A_n258D, the TIB,c and RIB values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n258A, DC_1A-n258D | 1 | 0 |
|  | n258 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n258A, DC_1A-n258D | 1 | 0 |
|  | n258 | 0 |

### 6.x.5	MSD

There is no MSD issue for this DC configuration.

*********************** End of the TP ***************************************************