3GPP TSG-RAN WG4 Meeting #90bis	R4-1904226

Xi’an, China, 8th Apr. - 12 Apr. 2019

Agenda item:	6.12.1.4.4

Title: 	Discussion on simulation requirements of NR performance RI tests

Source: 	Huawei, Hisilicon

Document for:	Discussion

Introduction

Base on the previous agreements and agreed way forward of R4-1811693 and R4-1814235, companies are suggested to provide simulation results under the assumptions below.

After the RAN4 #89’s meeting in Spokane, some agreed and undetermined simulation assumptions are discussed by many companies, and based on the agreed way forward of RAN4 2018 November’s meeting we changed part of the simulation assumptions.

After the RAN4 #90’s meeting in Athens, many undetermined simulation assumptions and requirements have already aligned. For the left test cases, we provide our new simulation results for alignment in this meeting.

Simulation assumption for RI

The minimum performance requirement in Table 6.4.3.1-2 is defined as:

a) The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ γ1;

b) The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ γ2;

For the parameters specified in Table 6.4.3.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.3.1-2.

Table 1 lists the simulation parameters for the left RI test cases that need to determine the requirements.

Table 1 Test parameters(FDD)

| Parameter |  |  |  | Unit | Test 1 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 | 10 |
| Duplex Mode |  |  |  |  | FDD | FDD |
| DL BWP configuration #1 |  |  | First PRB |  | 0 | 0 |
|  |  |  | Number of contiguous PRB |  | 52 | 52 |
|  |  |  | Subcarrier spacing | kHz | 15 | 15 |
| SNR |  |  |  | dB | TBD | TBD |
| Propagation channel |  |  |  |  | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  |  | ULA Low 2x4 | ULA Low 4x4 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 4 (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 | 5/1 |
| ReportConfigType |  |  |  |  | Periodic | Periodic |
| CQI-table |  |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | [8] | [8] |
| csi-ReportingBand |  |  |  |  | [1111111] | [1111111] |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 | 5/1 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | (2,1) |
|  |  | CodebookSubsetRestriction |  |  | [010000 for fixed rank 2, 010011 for following rank] | 11111111 |
|  |  | RI Restriction |  |  | N/A | 00000010 for fixed Rank 2 and 00001111 for follow RI |
| Physical channel for CSI report |  |  |  |  | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  |  | ms | 8 | 8 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 2 and follow RI |

Simulation results

Table 3-1 TP Ratio of test 1 & test 4

|  | Ratio |  |
| --- | --- | --- |
| SNR | Test 1 | Test 4 |
| -6 | 1.41 | 1.12 |
| -4 | 0.98 | 1.08 |
| -2 | 1.04 | 1.14 |
| 0 | 1.01 | 1.05 |
| 2 | 1.05 | 1.01 |
| 4 | 1.05 | 1.01 |
| 6 | 1.01 | 1.03 |
| 8 | 1.02 | 1.01 |
| 10 | 1.00 | 1.05 |
| 12 | 0.99 | 1.02 |
| 14 | 1.00 | 0.99 |
| 16 | 1.00 | 1.03 |
| 18 | 1.01 | 0.98 |
| 20 | 1.02 | 1.02 |

Figure 3-1 FDD 4Rx Test 1 simulation result

Conclusion

In this contribution, we provide our new simulation results of rest of the undetermined RI test cases for discussion.

Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018

[6] R4-1814235, “Way forward on NR UE CSI requirements”, Samsung, October 2018

[7] R4-1902425, “DraftCR RI reporting”, Qualcomm Incorporated, Feb 2019