3GPP TSG-RAN WG4 Meeting # 90bis	R4-1904377

# Xi’an, Shaanxi, China,  8 - 12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.817-02: updating table 12.10.1-1 for the FR2 TX ON/OFF power requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2019-04-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the RAN4 #90 meeting, the WF on FR2 transient period (R4-1902286) was approved, capturing agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The column for the FR2 OTA transmit ON/OFF power requirement in Table 12.10.1-1 is updated according to the agreement captured in R4-1902286, i.e., “The conformance EIRP OFF level for the transient test will be Core TRP plus the ON antenna directviity”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The column for the FR2 OTA transmit ON/OFF power requirement in Table 12.10.1-1 can be ambiguous and is not aligned with the agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## 12.10	TRP measurements

### 12.10.1	General

Refer to subclause 10.8 in TR 37.843 [26] for further details.

A summary of TRP measurement procedures and their applicability to different OTA BS requirements is shown in Table 12.10.1-1.

Table 12.10.1-1 - Applicability of TRP measurement methods

|  | OTA base station output power | FR2 OTA transmit ON/OFF power | OTA unwanted emissions: Adjacent channel leakage ratio (ACLR) (Note 1) | OTA Operating band unwanted emission (OBUE) | OTA transmitter spurious emissions OTA (Note 2) |
| --- | --- | --- | --- | --- | --- |
| Full sphere using reference steps (accurate) | X |  | X | X | X |
| Full sphere using sparse sampling (overestimate) |  |  |  |  | X (Note 3) |
| Two cuts + Pattern multiplication (accurate) (Note 4) | X |  | X | X |  |
| Two/three cuts (overestimate) |  |  | X | X | X (Note 3) |
| Beam-based directions | X | X | X | X |  |
| Peak method |  | [X] (Note 5) |  | X | X |
| Equal sector with peak average |  |  |  | X | X |
| Pre-scan |  |  | X | X | X |
| NOTE 1: Two TRP measurements are needed. NOTE 2: Pre-scan is needed to identify the frequencies of interest. Pre-scan can also be applied to ACLR, OBUE and SEM. NOTE 3: At harmonic frequencies the use of this method is FFS due to risk of high beamforming gain NOTE 4: Pattern multiplication is conditional NOTE 5: If measured EIRPOFF level is less than the TX OFF power test requirement, then it is not necessary to estimate TRPOFF level using the beam-based directions method NOTE: If box is blank the method is not excluded but the methodology has not been described in subclause 10.8, TR 37.843; if a suitable analysis is shown the method may be applied. |  |  |  |  |  |

The summation error is based on a reasonable number of test directions and the minimum beam width, which is in turn dependent on antenna size and frequency, i.e., the reference steps see Figure 10.8.2.2-1 in TR 37.843 [26]. It has been agreed that a reasonable trade-off between accuracy and sampling is achieved when the SE is 0,75 dB for FR1 BS (which is adopted from OTA AAS BS) and 1,2 dB for FR2 BS. For FR2 BS, the beams are expected to be narrower than an FR1 BS and hence the SE is higher.

<End of modified section>