3GPP TSG-WG RAN4 Meeting #90bis	R4-1904899

# Xian, China, 8th – 12th April, 2019                                                            (Rev. of R4-1903079)

Source: 	Nokia, Nokia Shanghai Bell

Title:	TP for TR 37.716-21-21 DC_1A_n40A-n78A

Agenda Item: 			9.8.2 [DC_R16_xBLTE_2BNR_yDL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-21-21 to introduce the option for DC_1A_n40A-n78A

*********************** Start of the TP ***************************************************

## 6.X	DC_1A-n40A-n78A

### 6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-n40A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n40A-n78A | DC_1A_n40A DC_1A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.X.3	Co-existence studies

The result of co-existence studies for DC_1A-n40A-n78A are provided in TR 37.716-21-21, Table 5.3-2.

### 6.X.4	∆TIB and ∆RIB values

For DC_1A_n40A-n78A, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n40A-n78A | 1 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n40A-n78A | 1 | 0 |
|  | n40 | 0 |
|  | n78 | 0.5 |

### 6.X.5	MSD

Results of MSD study are provided in TR 37.716-21-21, Table 5.3-5.

**************************** End of changes ********************************************