3GPP TSG-RAN4 Meeting #92 	R4-1908784

# Ljubljana, Slovenia, 26 – 30 August, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Intra-frequency SS-RSRQ Accuarcy Test Cases for SA FR1 (section A.6.7.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2019-08-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Noc level is not appropriate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Noc level. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-frequency RSRQ could not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----- << Start of Change 1>> -----

A.6.7.2	SS-RSRQ

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

A.6.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.1.1.

A.6.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.6.7.2.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Cell 1 |  | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  |  |  |  |  | freq1 |  |  | freq1 |  | freq1 |  |
| Duplex mode |  |  |  |  | Config 1 |  |  | FDD |  |  |  |  |  |  |
|  |  |  |  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  |  |  |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP configuration |  |  |  |  | Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  |  |  |  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  |  |  |  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  |  |  |  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle |  |  |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  |  | Config 1 |  |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  |  |  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  |  |  | Config 3 |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  |  |  | Config 1 |  |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  |  |  |  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  |  |  | Config 3 |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Control Channel RMC |  |  |  |  | Config 1 |  |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  |  |  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  |  |  | Config 3 |  |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  |  |  |  |  | OP. 1 |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| SMTC configuration |  |  |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |
| SSB configuration |  |  |  |  | Config 1,2 |  |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  | 30kHz |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  | dB | 0 | 0 |  | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  | dBm/15kHz | -85 |  |  | -104 |  | [-120] |  |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | [-119.5] |  |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | [-119] |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | [-118.5] |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | [-118] |  |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | [-117] |  |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | [-116.5] |  |
|  |  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  |  | -91 |  |  | -110 |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
| Note2 |  | Config 1,2 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/SCS | -85 |  |  | -104 |  | Same as Noc for 15kHz |  |
|  |  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  | -88 |  |  | -107 |  | [-117] |  |
|  |  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  | [-116.5] |  |
|  |  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  | [-116] |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | [-115.5] |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | [-115] |  |
|  |  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  | [-114] |  |
|  |  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  | [-113.5] |  |
|  |  |  |  |  |  |  | dB | -1.76 |  |  | -4.7 |  | -5..46 | -5.46 |
|  |  |  |  |  |  |  | dB | 3 | 3 |  | -2.9 | -2.9 | -4 | -4 |
| SS-RSRPNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  |  | dBm/SCS | -82 | -82 |  | -106.9 | -106.9 | [-124] | [-124] |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | [-123.5] | [-123.5] |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | [-123] | [-123] |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | [-122.5] | [-122.5] |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | [-122] | [-122] |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | [-121] | [-121] |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | [-120.5] | [-120.5] |
|  | Config 3 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  |  | -85 | -85 |  | -109.9 | -109.9 | [-121] | [-121] |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | [-120.5] | [-120.5] |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | [-120] | [-120] |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | [-119.5] | [-119.5] |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | [-119] | [-119] |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | [-118] | [-118] |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | [-117.5] | [-117.5] |
| SS-RSRQ Note3 |  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  | dB | -14.77 | -14.77 |  | -16.76 | -16.76 | [-17.34 | [-17.34] |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
| IoNote3 |  |  | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  | dBm/ 9.36MHz | -50 |  |  | -73 |  | [-89.5] |  |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | [-89] |  |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | [-88.5] |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | [-88] |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | [-87.5] |  |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | [-86.5] |  |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | [-86] |  |
|  |  |  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  | dBm/ 38.16MHz | -50 |  |  | -72.89 |  | [-83.41] |  |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | [-82.91] |  |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | [-82.41] |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | [-81.91] |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | [-81.41] |  |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | [-80.41] |  |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | [-79.91] |  |
| Propagation condition |  |  |  |  |  |  | - | AWGN | AWGN |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  |  |  |  |  | 1x2 | 1x2 |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

A.6.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.7.1.1.

----- << End of Change 1>> -----