3GPP TSG-RAN WG4 Meeting #92	R4-1908882

# Ljubljana, Slovenia, August 26th-30th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | XXX | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104 – PUCCH format 0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update the requirements for PUCCH format 0 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the requirements for PUCCH format 0 tests based on simulation results |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No up to date requirements for PUCCH format 0. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2. 11.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-1 and TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<start of the change>

### 8.3.2	Performance requirements for PUCCH format 0

#### 8.3.2.1	General

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent.

Table 8.3.2.1-1: Test Parameters

| Parameter | Test |
| --- | --- |
| nrofBits | 1 |
| nrofPRBs | 1 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| pucch-GroupHopping | neither |
| hoppingId | 0 |
| initialCyclicShift | 0 |
| startingSymbolIndex | 13 for 1 symbol 12 for 2 symbols |

The transient period as specified in TS 38.101-1[X] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

#### 8.3.2.2	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.3.2.2-1 and in table 8.3.2.2-2.

Table 8.3.2.2-1: Minimum requirements for PUCCH format 0 and 15kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | TDLC300-100 Low | 1 | [8.7] | [8.8] | [9.2] |
|  |  |  | 2 | [2.8] | [3.5] | [3.3] |
| 1 | 4 | TDLC300-100 Low | 1 | [3.0] | [3.0] | [3.1] |
|  |  |  | 2 | [-1.0] | [-0.5] | [-0.8] |
| 1 | 8 | TDLC300-100 Low | 1 | [-1.1] | [-1.1] | [-1.0] |
|  |  |  | 2 | TBD | [-3.9] | [-4.2] |

Table 8.3.2.2-2: Minimum requirements for PUCCH format 0 and 30kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | TDLC300-100 Low | 1 | [9.8] | [9.7] | [9.5] | [9.4] |
|  |  |  | 2 | [4.2] | [3.7] | [3.6] | [3.1] |
| 1 | 4 | TDLC300-100 Low | 1 | [3.3] | [3.3] | [3.0] | [3.2] |
|  |  |  | 2 | [-0.3] | [-0.4] | [-0.4] | [-0.8] |
| 1 | 8 | TDLC300-100 Low | 1 | [-1.0] | [-0.9] | [-1.1] | [-1.0] |
|  |  |  | 2 | [-3.8] | [-3.9] | [-3.9] | [-4.2] |

<end of the change>

<start of the change>

#### 11.3.2.2	Performance requirements for PUCCH format 0

##### 11.3.2.2.1	General

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent.

Table 11.3.2.2.1-1: Test Parameters

| Parameter | Test |
| --- | --- |
| nrofBits | 1 |
| nrofPRBs | 1 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| pucch-GroupHopping | neither |
| hoppingId | 0 |
| initialCyclicShift | 0 |
| startingSymbolIndex | 13 for 1 symbol 12 for 2 symbols |

The transient period as specified in TS 38.101-1[17] subclause 6.3.3.1 and TS 38.101-2[18] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

##### 11.3.2.2.2	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 11.3.2.2.2-1 and in table 11.3.2.2.2-2.

Table 11.3.2.2.2-1: Minimum requirements for PUCCH format 0 and 60kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex G) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | TDLA30-300 Low | 1 | [9.3] | [9.0] |
|  |  |  | 2 | [4.2] | [4.0] |

Table 11.3.2.2.2-2: Minimum requirements for PUCCH format 0 and 120kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex G) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | TDLA30-300 Low | 1 | [9.5] | [9.2] | [9.7] |
|  |  |  | 2 | [4.1] | [3.8] | [4.0] |

<end of the change>