3GPP TSG-RAN WG4 Meeting #92	R4-1909151

Ljubljana, Slovenia, 26 – 30 August 2019

Agenda Item:	6.2.5

Source:	Ericsson

Title:	Discussion on NB-IoT NPRACH demodulation performance for TDD

Document for:	Discussion

# 1	Introduction

Addressing the issue on the huge TDD NPRACH performance gap between companies, RAN4 agreed with the way forward [1], where it asks the interested companies to provide the simulation results in RAN4#92 to identify the reason of performance gap. In this contribution we provide our ideal simulation results and discuss the way forward.

# 2	Discussion

## 2.1	Simulation results

Table 1 shows the simulation parameters for evaluation, and it is the same parameter set agreed in [1]. Figure 1 shows our ideal simulation results of the missed detection rate (MDR) of NPRACH TDD format 0 with false alarm rate < 0.1%. It is observed from our simulation that the required SNR to achieve MDR of 1% about 11.0dB.

Observation: For NPRACH TDD Format 0, the required SNR to achieve MDR of 1% about 11.0dB without repetition under static environment.

Table 1	TDD NPRACH simulation parameters.

| Parameters | Values |
| --- | --- |
| Preamble formats | TDD NPRACH format 0 |
| Cell ID | 0 |
| NPRACH signature | 0 |
| Uplink-downlink configuration | 1 |
| Special subframe configuration | 7 |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2 |
| Propagation condition | AWGN |
| Repetition number | 1 |
| Frequency offset | 0 Hz |
| Frequency hopping | 1. Enabled 2. Disabled (Optional) |
| Detection performance | SNR to achieve 1) missed detection probability of 1%, 2) false alarm rate of 0.1%, and 3) timing error probability with limits of 3.646us at the same time |

Figure 1	Missed detection probability simulation result of NPRACH TDD format 0 with false alarm rate of 0.1%.

## 2.2	Discussion

In Rel-13 RAN4 introduced NPRACH missed detection requirements for FDD format 0/1. Although FDD format 0 is different format from TDD format 0, we can refer to the results to estimate the achievable SNR levels with MDR=1%.

Figure 2 and Figure 3 compare the NPRACH preamble formats 0 for FDD and format 0 for TDD. It is important to note that the preamble sequence length is 8192Ts for both formats, which means the difference between FDD format 0 and TDD format 0 is the CP length and the number of repetitions.

Without the cyclic prefix, the FDD format 0 repeats the same sequence 20 times but TDD format 0 repeats the same sequence 4 times. Since FDD format 0 uses 5 times more repetitions compared with TDD, the effective performance gain (or coverage expansion) of FDD due to the repetition is 10*log10(5) ~ 7dB. If we look the existing NPRACH FDD format 0 (See Table 2 in Appendix), the required SNR with MDR=1% is -2.1dB with 8 repetitions for static condition. Since our simulation does not use repetition, the gain of FDD is about 10*log10(20*8/4) ~ 15dB, and therefore it is expected the TDD performance is around 12.9 dB (-2.1+15.0).

Figure 2	NPRACH preamble structure for FDD format 0.

Figure 3	NPRACH preamble structure for TDD format 0.

# 3	Conclusion

Observation: For NPRACH TDD Format 0, the required SNR to achieve MDR of 1% about 11.0dB without repetition under static environment.

This contribution provided our simulation results for NPRACH TDD format 0 and gave the analysis. We propose RAN4 considers our results and analysis.

# 4	References

- R4-1907388, “Way forward on Rel-15 NB-IOT NPRACH demodulation requirements”, Ericsson, Huawei, Nokia.

# Appendix

Table 2	NPRACH missed detection requirements for FDD (TS38.104, Table 8.5.3.2.1-1)

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 2 |
| 1 | 2 | 8 | AWGN | 0 | -2.1 | -2.1 | -1.0 |
|  |  |  | EPA1 Low | 200 Hz | 6.1 | 6.1 | 9.0 |
|  |  | 32 | AWGN | 0 | -6.8 | -6.8 | -5.1 |
|  |  |  | EPA1 Low | 200 Hz | 0.5 | 0.5 | 2.2 |