3GPP TSG-RAN4 Meeting #92	R4-1909330

# Ljubljana, Slovenia, 26th - 30th Aug, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on known TCI state definition (section 8.10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-06-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Regarding the definition of the known conditions for TCI state, “TCI state switch is within [X] ms of last transmission of the resource for beam reporting/ measurement for the target TCI state”. The “measurement” refers to one measurement sample and left to UE implementation. If the starting point of known TCI state definition is the moment that UE performs the measurement on the latest resource, network has no idea of the exact time when UE performs the measurement. Then network can not make correct judgement of known case and unknown case for TCI state switching and can not decide when start to scheduling. Thus the reporting of the latest resource is an appropriate starting point. - In order to guarantee the selected RX beam is still valid between the L1-RSRP measurement reported and the target TCI is applied, we suggest X is up to [1.28]ms. - It is agreed in RAN4#91 (R4-1907305 ad hoc minutes); Agreement: - The known TCI state is defined by the time from the latest measurement resource for the TCI to when TCI activation command is received, - If the period is less than X, the TCI state is considered as known. Otherwise the TCI state is considered as unknown. - Before when TCI activation command is received - L1 RSRP measurement for the RS to which the target TCI is QCLed with should be reported The QCLed condition shall be specified in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove “measurement” in the definition of the known conditions for TCI state. - X=[1.28] s - The reported RS for the L1-RSRP is the reported RS for the L1-RSRP measurement is the RS in the target TCI state or to which the RS in target TCI state is QCLed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.10	Active TCI state switching delay

8.10.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section.

### 8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

- TCI state switch is within [1.28] s of lastest transmission of the resource for beam reporting for the target TCI state

- The UE has sent at least 1L1-RSRP measurement report for the target TCI state, where the reported RS for the L1-RSRP measurement is the RS in the target TCI state or to which the RS in target TCI state is QCLed.

- The TCI state shall remain detectable during the TCI state switching period

  - SNR of the TCI state is > -3dB

Otherwise, the TCI state is unknown.

<End of Change 1>