3GPP TSG-RAN WG4 Meeting #92	R4-1909803

Ljubljana, Slovenia, 26 August – 30 August 2019

Source:	Ericsson, T-Mobile US

Title:	TP for TR 37.716-21-11: 2A-66A_n41A

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 to add 2A-66A_n41A.

# 2. Text Proposal

# ---Start of changes---

## 5.1.x	2A-66A_n41A

### 5.1.x.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-66_n41 | CA_2-66 | n41 | No |

### 5.1.x.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n41A | DC_2A_n41A DC_66A_n41A | CA_2A-66A | n41 |

### 5.1.x.3		∆TIB and ∆RIB values

For DC_2-66_n41, the TIB,c and RIB,c values are reused from the LTE combination CA_2-7-66, and are given in the tables below.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n41 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.1.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n41 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 5.1.x.4	REFSENS requirements

Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to Band 66 DL.

Co-existence analysis for DC_66_n41 shows that there is IMD4 impact from DC_66_n41 UL to Band 2 DL.

The IMD4 MSD value for the impact on band 2 DL are same as for band n25 DL in DC_66_n25-n41.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 | 1940 | 11.0 | FDD | IMD4 ¦2*fBn41-2*fB66¦ |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | TDD | N/A |

---End of changes---

# Reference

[1]	RP-191124, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei