3GPP TSG-RAN4 WG4 Meeting # 92	R4-1909862

# Ljubljana, Slovenia, 26 - 30 August, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | <CR#> | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR: Updates to FR2 PDSCH test configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1: The configuration for “position of the first DM-RS for PDSCH mapping type A” put in the “Common serving cell parameters” is confusing, because there is separate “PDSCH DMRS configuration” in the same table; 2: The tables for the test parameters configuration is not adapted to the page width and is inconsistent with FR1 part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: Moved the configuration of “position of the first DM-RS for PDSCH mapping type A” from “Common serving cell parameters” part to “PDSCH DMRS configuration” part in Table 7.2-1; 2: Ajusted the table format for test parameter’s configuration to align with FR1 part and make the specification look more consistent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion and inconsistence will still exist in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7	Demodulation performance requirements (Radiated requirements)

7.1	General

7.1.1	Applicability of requirements

7.1.1.1	General

The minimum performance requirements are applicable to the FR2 operating bands defined in TS 38.101-2 [7] with FDL_high not exceeding 40000 MHz.

The minimum performance requirements in Clause 7 are mandatary for UE supporting NR operation, except test cases listed in Clause 7.1.1.3.

7.1.1.2	Applicability of requirements for different number of RX antenna ports

UE shall support 2 RX ports for different RF operating bands. The UE requirements applicability is defined in Table 7.1.1.2-1.

Table 7.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports 2RX antenna ports | PDSCH | All tests in Clause 7.2.2 |
|  | PDCCH | All tests in Clause 7.3.2 |
|  | PBCH | All tests in Clause 7.4.2 |

7.1.1.3	Applicability of requirements for optional UE features

For UE which supports optional UE features the additional performance requirements from Table 7.1.1.3-1 should be applied.

Table 7.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Enhanced Type X receiver | FR2 TDD | PDSCH | 7.2.2.2.1 Minimum requirements for PDSCH Mapping Type-A (Test 3-1) |  |

#### 7.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

For UE which supports mandatory UE features with capability signalling the additional performance requirements from Table 7.1.1.4-1 should be applied.

Table 7.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR2 TDD | PDSCH | 7.2.2.2.1 Minimum requirements for PDSCH Mapping Type-A (Test 2-1 to 2-6) |  |
| Support of 1 port PTRS (onePortsPTRS) | FR2 TDD | PDSCH | All test cases |  |

7.2	PDSCH demodulation requirements

The parameters specified in Table 7.2-1 are valid for all PDSCH demodulation tests unless otherwise stated.

Table 7.2-1: Common Test Parameters

| Parameter |  |  |  | Unit |  |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  |  |  |  | Transmission scheme 1 |  |  |  |
| PTRS epre-Ratio |  |  |  |  |  |  | 0 |  |  |  |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  |  | RBs |  |  |  |  |  | 0 |
|  | Subcarrier spacing |  |  | kHz |  |  |  |  |  | 60 or 120 |
| DL BWP configuration #1 | Cyclic prefix |  |  |  |  |  |  |  |  | Normal |
|  | RB offset |  |  | RBs |  |  |  |  |  | 0 |
|  | Number of contiguous PRB |  |  | PRBs |  |  |  |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  |  |  |  |  |  |  | 0 |
|  | SSB position in burst |  |  |  |  |  |  |  |  | 1 |
|  | SSB periodicity |  |  | ms |  |  |  |  |  | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  |  |  |  |  |  |  | Each slot |
|  | Symbols with PDCCH |  |  |  |  |  |  |  |  | 0 |
|  | Number of PRBs in CORESET |  |  |  |  |  |  |  |  | Table 7.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  |  |  |  |  |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  |  |  |  |  |  |  | Non-interleaved |
|  | DCI format |  |  |  |  |  |  |  |  | 1_1 |
|  | TCI state |  |  |  |  |  |  |  |  | TCI state #1 |
| Cross carrier scheduling |  |  |  |  |  |  | Not configured |  |  |  |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  |  |  |  |  |  | 0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  |  |  |  |  |  | 6 for CSI-RS resource 1 and 3 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  |  |  |  |  |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  |  |  |  |  |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  |  |  |  |  |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  |  | Slots |  |  |  |  |  | 60 kHz SCS: 80 for CSI-RS resource 1,2,3,4 120 kHz SCS: 160 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  |  | Slots |  |  |  |  |  | 60 kHz SCS: 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  |  |  |  |  |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  |  |  |  |  |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  |  |  |  |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  |  |  |  |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  |  |  |  |  |  |  | 2 |
|  | CDM Type |  |  |  |  |  |  |  |  | FD-CDM2 |
|  | Density (ρ) |  |  |  |  |  |  |  |  | 1 |
|  | CSI-RS periodicity |  |  | Slots |  |  |  |  |  | 60 kHz SCS: 80 120 kHz SCS: 160 |
|  | CSI-RS offset |  |  |  |  |  |  |  |  | 0 |
|  | Frequency Occupation |  |  |  |  |  |  |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  |  |  |  |  |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  |  |  |  |  |  | 4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  |  |  |  |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  |  |  |  |  |  |  | 4 |
|  | CDM Type |  |  |  |  |  |  |  |  | FD-CDM2 |
|  | Density (ρ) |  |  |  |  |  |  |  |  | 1 |
|  | CSI-RS periodicity |  |  | Slots |  |  |  |  |  | 60 kHz SCS: 80 120 kHz SCS: 160 |
|  | CSI-RS offset |  |  |  |  |  |  |  |  | 0 |
|  | Frequency Occupation |  |  |  |  |  |  |  |  | Start PRB 0 Number of PRB = BWP size |
| CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS |  |  |  |  |  |  |  |  | k0=0 for CSI-RS resource 1,2 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  |  |  |  |  |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  | Number of CSI-RS ports (X) |  |  |  |  |  |  |  |  | 1 for CSI-RS resource 1,2 |
|  | CDM Type |  |  |  |  |  |  |  |  | ‘No CDM’ for CSI-RS resource 1,2 |
|  | Density (ρ) |  |  |  |  |  |  |  |  | 3 for CSI-RS resource 1,2 |
|  | CSI-RS periodicity |  |  | Slots |  |  |  |  |  | 60 kHz SCS: 80 for CSI-RS resource 1,2 120 kHz SCS: 160 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  |  | Slots |  |  |  |  |  | 0 for CSI-RS resource 1,2 |
|  | QCL info |  |  |  |  |  |  |  |  | TCI state #1 |
| PDSCH DMRS configuration | Antenna ports indexes |  |  |  |  |  |  |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests |
|  |  | Position of the first DMRS for PDSCH mapping type A |  |  |  |  | 2 |  |  |  |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  |  |  |  |  |  |  | 1 |
| TCI state #0 |  | Type 1 QCL information | SSB index |  |  |  |  | SSB #0 |  |  |
|  |  |  | QCL Type |  |  |  |  | Type C |  |  |
|  |  | Type 2 QCL information | SSB index |  |  |  |  | SSB #0 |  |  |
|  |  |  | QCL Type |  |  |  |  | Type D |  |  |
| TCI state #1 |  | Type 1 QCL information | CSI-RS resource |  |  |  |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |  |  |
|  |  |  | QCL Type |  |  |  |  | Type A |  |  |
|  |  | Type 2 QCL information | CSI-RS resource |  |  |  |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |  |  |
|  |  |  | QCL Type |  |  |  |  | Type D |  |  |
| PTRS configuration | Frequency density (KPT-RS) |  |  |  |  |  |  |  |  | 2 |
|  | Time density (LPT-RS) |  |  |  |  |  |  |  |  | 1 |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  |  |  |  | 1 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  |  | 4 |  |  |  |
| HARQ ACK/NACK bundling |  |  |  |  |  |  | Multiplexed |  |  |  |
| Redundancy version coding sequence |  |  |  |  |  |  | {0,2,3,1} |  |  |  |
| Precoding configuration |  |  |  |  |  |  | SP Type I, Random per slot with PRB bundling granularity |  |  |  |
| Symbols for all unused Res |  |  |  |  |  |  | OCNG in Annex A.5 |  |  |  |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |  |  |  |  |

Table 7.2-2: Number of PRBs in CORESET

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 30 | 66 | 132 | 264 |

7.2.1	1RX requirements

(Void)

7.2.2	2RX requirements

7.2.2.1	FDD

(Void)

7.2.2.2	TDD

7.2.2.2.1	Minimum requirements for PDSCH Mapping Type-A

For PDSCH Type-A scheduling, the requirements are specified in Table 7.2.2.2.1-3, 7.2.2.2.1-4 and 7.2.2.2.1-5, with the addition of the parameters in Table 7.2.2.2.1-2 and the downlink physical channel setup according to Annex C.5.1. The purpose is to verify the performance of PDSCH Type-A scheduling.

The test purposes are specified in Table 7.2.2.1.1-1.

Table 7.2.2.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs andnumber of MIMO layers | 1-1, 1-3, 2-1, 2-2, 2-3, 2-4, 2-5, 2-6 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 1-2 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 2 receive antenna conditions and with 2 MIMO layers. | 3-1 |

Table 7.2.2.2.1-2: Test Parameters

| Parameter |  | Unit |  | Value |  |
| --- | --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | For Test 1-1 and 1-2: 3 for CSI-RS resource 1 and 3 7 for CSI-RS resource 2 and 4 |
|  | CSI-RS offset | Slots | For Test 1-2: 82 for CSI-RS resource 1 and 2 83 for CSI-RS resource 3 and 4 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL4 for Test 2-3 1/AL8 for other tests |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | WB for Test 1-1, 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  |  | 8 for Test 1-1, 1-3, 2-2, 2-4 10 for Test 2-1, 2-3, 2-5, 2-6, 3-1 16 for Test 1-2 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | As defined in Annex A.1.3 |  |

Table 7.2.2.2.1-3: Minimum performance for Rank 1 (FRC)

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 1-1 | R.PDSCH.5-1.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-1A | TDLC60-300 | 2x2 ULA Low | 70 | -0.4 |
| 1-2 | R.PDSCH.5-2.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 30 | 1.7 |
| 1-3 | R.PDSCH.5-3.1 TDD | 100 / 120 | 64QAM, 0.46 | FR2.120-1 | TDLA30-300 | 2x2 XPL Medium-A | 70 | 12.4 |

Table 7.2.2.2.1-4: Minimum performance for Rank 2 (FRC)

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 2-1 | R.PDSCH.5-4.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 4.1 |
| 2-2 | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.4 |
| 2-3 | R.PDSCH.5-5.2 TDD | 50 / 120 | 16QAM, 0.48 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 14.0 |
| 2-4 | R.PDSCH.5-2.3 TDD | 200 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.2 |
| 2-5 | R.PDSCH.4-1.1 TDD | 50 / 60 | 16QAM, 0.48 | FR2.60-1 | TDLA30-75 | 2x2 ULA Low | 70 | 14.3 |
| 2-6 | R.PDSCH.5-6.1 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 18.6 |

Table 7.2.2.2.1-5: Minimum performance for Rank 2 (FRC) for Enhanced Type X Receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 3-1 | R.PDSCH.5-5.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-2 | TDLA30-75 | 2x2 ULA Medium | 70 | 19.0 |