3GPP TSG-RAN WG4 #92	R4-1910542

Ljubljana, Slovenia, 26th – 30th Aug., 2019

Title:	LS on sidelink BWP reconfiguration on ITS band

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Reply To:

Source:	RAN WG4

To:	RAN WG1

CC: 	RAN WG2

Contact Person:

Name:	Zhixun Tang

E-mail Address:	Zhixun.tang@mediatek.com

1. Overall Description:

RAN4 is discussing the need of requirement for sidelink BWP reconfiguration in ITS band. RAN4 noticed that there are some RAN1 agreements for sidelink BWP and SCS as follow.

- Only one SL BWP is configured in a carrier for a NR V2X UE.

- S-SSB has the same numerology, which includes SCS and CP length, as that of control and data channels for a given carrier.

- Only one combination of CP length and SCS is used in a carrier at a given time for NR V2X UEs communicating with each other using SL.

RAN4 would like to inform RAN1 that the RRC-based BWP reconfiguration (e.g., involving SCS, location and/or bandwidth BWP change) of NR sidelink on ITS band is not a practical use case in R16. RAN4 decided not define any requirement for this use case in R16.

2. Actions:

To RAN WG1

CC RAN WG2

ACTION: RAN WG4 respectfully asks RAN WG1 to taking into account the above information in future NR V2X sidelink discussion.

3. Date of Next RAN4 Meetings:

RAN4#92Bis	14th – 18th October 2019	Chongqing, China

RAN4#93	18th – 22th November 2019 	Reno, Nevada