3GPP TSG-RAN WG4 Meeting #92bis                    	   R4-1910826

Chongqing, China, 14th - 18th Oct, 2019

Agenda item:            8.17.2

Source:	MediaTek Inc.

Title:	Discussion on feasibility of NR HST with DPS transmission

Document for:          Discussion

# 1	Introduction

The WF on UE demodulation for NR HST was approved in RAN #92 meeting [1]. Companies were encouraged to study the feasibility and performance for HST-SFN with dynamic point selection (DPS) transmission. In this contribution, we elaborate three options for HST-SFN deployment and analyze corresponding channel models from view point of UE demodulation. Finally, the simulation results and analysis for HST SFN scenario are provided.

# 2	HST-SFN Deployments

2.1 Option 1: Four Paths Joint Transmission

As the figure 2.1, all RRHs/sectors share the same SSB index (so as TRS/PDCCH/PDSCH). UE receives signals from four RRHs/sectors with channels like the LTE 4-tap HST channel model. UE suffers positive and negative Doppler frequency at the same time. All RRHs transmit the same data to UE. Therefore, there is no need to do handover when moving through RRH. By this scheme, L1 RSRP report for tracking other SSB/TRS beam is not required and data interruption due to handover can be avoided.

Figure 2.1 HST-SFN deployment option 1: joint transmission

2.2 Option 2: Two Paths Unidirectional DPS Transmission

Each RRH have two SSB beams which are SSB0 and SSB1 transmitted from two sectors alternatively. UE receives data from two RRHs with the direction chosen by larger signal strength with one SSB index. UE suffers either positive or negative Doppler frequency at the same time. So that it may ease demodulation complexity. However, UE needs to monitor the SSB/TRS for the two beams, which increase UE’s complexity on time/frequency synchronization. Further, when UE travels across the boundary of different beams, data interruption or latency is inevitable since TCI change for PDCCH would require MAC-CE deactivation/activation. As the figure 2.2, UE switches beam (SSB index) per half distance of RRH spacing. From view of base station, there is possibility to increase capacity by SDMA.

Figure 2.2 HST-SFN deployment option 2: two paths unidirectional DPS transmission

2.3 Option 3: Two Paths Bidirectional DPS Transmission

Different from the option 2, UE receives both of toward and away beams from adjacent two RRHs with the same SSB index. UE suffers up to two paths with one positive and one negative Doppler frequency. Less frequent L1-RSRP report and TCI change happens for UE in this scenario. Beam switch is made per distance of RRH spacing. Same potential gain to increase capacity by SDMA can be obtained.

Figure 2.3 HST-SFN deployment option 3: two paths bidirectional DPS transmission

# 3	Channel Models and Analysis

To properly analyse the impact of HST deployment from different aspects, antenna radiation pattern of directional beam would be taken into account for practical concern. The antenna radiation pattern used in TR [3] as figure 3.1 is used for the study of the three deployment options in this paper. All options can be derived from HST-SFN 4 tap channel model with disabling paths of proper RRHs and applying directional antenna pattern. Modelling of path power is shown in Appendix B and modelling of tap delay and Doppler frequency is similar to HST-SFN 4 tap channel which can be referred to the technique report [3] and [4] and omitted in this paper. The channel models corresponding to option 1~3 can be derived from the illustration in the figure 2.1~2.3:

- Option 1: 4-tap bidirectional channel

- Option 2: 2-tap unidirectional channel

- Option 3: 2-tap bidirectional channel

Based on the channel models of different options, we provide following analysis from RSRP and demodulation performance perspective.

Figure 3.1 Antenna radiation pattern for directional beam

3.1 Relative Path Power Profiles Analysis

The path power profiles for three deployments are plotted based on the following parameters as table 3.2.

Table 3.2 Parameters for path power profile

| Configuration | Ds | Dmin | γ |
| --- | --- | --- | --- |
| Configuration 1 | 1000 m | 50 m | 45° |
| Configuration 2 | 750m | 150m | 50° |

First, we take Option 1 as an example to compare 4 taps bidirectional channel with different configurations. By Figure 3.2 and Figure 3.3, it is shown that parameters including beam direction, Ds, and Dmin affect power profile. Relative power difference for location B and C can be up to 35dB for configuration 1, which is much larger than that of configuration 2. Besides, antenna radiation pattern affects signal coverage of RRH. With proper radiation pattern design, signal coverage may be improved.

Observation #1: Beam direction, Ds, Dmin, and antenna radiation pattern affect signal coverage. Those parameters can be further studies for better UE experience in HST-SFN deployment.

Figure 3.2 Path power profile for 4 taps bidirectional channel for configuration 1

Figure 3.3 Path power profile for 4 Taps bidirectional channel for configuration 2

The path power profiles for Option 2 and Option 3 for Configuration 1 are illustrated in Figure 3.4 and 3.5, respectively. It can be shown that from RSRP point of view, under the same deployment (Ds, Dmin, and antenna radiation pattern, etc.) the three options will have similar coverage (in terms of total received power) as long proper SSB beam is selected.

Figure 3.4 Path power profile for 2 taps unidirectional channel for configuration 1

Figure 3.5 Path power profile for 2 taps bidirectional channel for configuration 1

3.2 DEMOD Performance Analysis

For better illustration and explanation, we mark location A as position which is closest to RRH, location B as position where the strongest path power occurs, and location C as middle position between two RRH, as shown in Figure 3.2.

- In location A:

  - There is large signal power drop due to angle of 45 degree within stopband of beam pattern. This may result in performance degradation. The loss may be improved by adding RRH nearby or adjusting deployment parameters such as beam direction, Ds, Dmin, and antenna radiation pattern, as our claim in Observation 1.

- In location B:

- The path from RRH closest to UE dominates UE total received power for all options, which is shown through Figure 3.2~ Figure 3.5. It is over 30dB stronger than paths from other RRHs, which can be almost negligible for UE demodulation. It is expected UE demodulation performance of this region is similar among three channel models.

- In location C:

  - It is observed signal paths from farer RRHs are weaker than those from nearby RRHs by about 20dB by comparing figure 3.2 and figure 3.5. It is expected that there is similar demodulation performance for 4-tap and 2-tap bidirectional schemes.

  - On the other hand, in Option 3, UE receives two paths with Doppler frequency of the same direction for 2 tap unidirectional channel. One path is stronger than another by about 20 dB or above. It results in UE experiences the channel like one tap HST scenario. Therefore, it is expected performance of 2 tap unidirectional channel is better than that of 2 tap bidirectional channel.

We compare PDSCH performance with different MCS and SNR in Table 3.2. It should be noted that the path power of all three models has been normalized to have same total power. Other settings can be referred in Appendix A.

Table 4.1 Required SNR (dB) @ 70% Tput for HST cases

| FDD 15kHz SCS |  |  |
| --- | --- | --- |
| Doppler |  | MCS | 4 tap bidirectional channel | 2 tap unidirectional channel | 2 tap bidirectional channel |
| 875 Hz |  | 4 | -0.35 | -0.32 | -0.36 |
| 875 Hz |  | 13 | 7.42 | 7.57 | 7.39 |
| 972 Hz |  | 4 | 0.54 | -0.31 | 0.51 |
| 972 Hz |  | 13 | 11 | 8.55 | 10.5 |
| 1250 Hz |  | 4 | 1.82 | 1.68 | 1.82 |
| 1250 Hz |  | 13 | Error flow | Error flow | Error flow |
| TDD 30kHz SCS |  |  |
| 1667 Hz |  | 4 | -0.21 | -0.23 | -0.24 |
| 1667 Hz |  | 13 | 8.48 | 8 | 8.42 |
| 1667 Hz |  | 17 | Error flow | 12.2 | Error flow |
| 1500 Hz |  | 4 | -0.19 | -0.24 | -0.21 |
| 1500 Hz |  | 13 | 8.8 | 7.96 | 8.66 |
| 1500 Hz |  | 17 | Error flow | 12.2 | Error flow |

The results conform to our analysis above, which lead us to following observations:

Observation #2: UE has similar demodulation performance for Option 3 (HST 2 taps bidirectional channel) and Option 1 (HST 4 taps bidirectional channel).

Observation #3: Demodulation performance for Option 2 (HST 2 taps unidirectional channel) is better than Option 3 (HST 2 taps bidirectional channel) or Option 1 (HST 4 taps bidirectional channel).

# 4	Comparison among HST-SFN deployment options

Besides the analysis from DEMOD performance, we also investigate the impact from beam management perspective in our companion Tdoc [5]. Following observations and proposals are made:

“Observation 4: Fail to trigger TCI-state switch when UE is much closed to RRH would results a poor decoding performance for an interval of 3 sec in every 7 sec. This is a high impact to user experience. “

“Proposal 1: RAN4 to consider other DPS transmission schemes to avoid TCI-state switch when UE is much closed to the RRH. ”

Though the analysis was done by comparing Option 1 and Option 2 in [5], the principle can be applied as well for Option 3. Difficulty of TCI-state switch also exists in option 3 due to the same reason: network has no idea about the L1-RSRP of the weaker beam when UE is much closed to the RRH.

Table 4.1 Comparison for HST-SFN deployment options

|  | Option 1: | Option 2: | Option 3: |
| --- | --- | --- | --- |
| Channel model | 4 tap bidirectional channel | 2 tap unidirectional channel | 2 tap bidirectional channel |
| Doppler frequency | Two positive tones and two negative tones | Either Two positive tones or two negative tones | One positive tone and one negative tone |
| Beam switch | Not required | Required | Required |
| Beam switch periodicity | N/A | Ds/2 | Ds |
| Beam switch robustness | N/A | Low | Low |
| Demodulation performance | Normal | High | Normal |

Besides above mentioned aspects, additional UE complexity is required for DPS schemes (both Option 2 and Option 3). For example, UE need to keep monitoring both SSB beams which may increase time/freq. tracking complexity. Further, to facilitate proper beam switching between different SSB beams, it required UE to perform frequent L1-RSRP measurement and report.

According to the analysis, DPS scheme based on Option 2 with 2-tap unidirectional channels does provide some benefits from DEMOD performance point of view. However, whether the gain can be realized still depends on robustness of beam switching. Therefore we suggest to further study any DPS scheme that may improve beam management robustness.

Proposal #1: Further study on any DPS scheme based on Option 2 with improved beam switching robustness.

# 5	Conclusions

In this contribution we provided channel power profile and simulation results for three HST deployments. Following observations and proposals were made:

Observation #1: Beam direction, Ds, Dmin, and antenna radiation pattern affect signal coverage. Those parameters can be further studies for better UE experience in HST-SFN deployment.

Observation #2: UE has similar demodulation performance for Option 3 (HST 2 taps bidirectional channel) and Option 1 (HST 4 taps bidirectional channel).

Observation #3: Demodulation performance for Option 2 (HST 2 taps unidirectional channel) is better than Option 3 (HST 2 taps bidirectional channel) or Option 1 (HST 4 taps bidirectional channel).

Proposal #1: Further study on any DPS scheme based on Option 2 with improved beam switching robustness.

# 6	Reference

[1] R4-1910050, “WF on UE demodulation for NR HST”, CMCC.

[2] R4-1902949, “LTE HST performance evaluation – Unidirectional HST SFN scenario”, Intel.

[3] 3GPP LTE 36.878, “Study on performance enhancements for high speed scenario in LTE”

[4] 3GPP LTE 36.101, “User Equipment (UE) radio transmission and reception”

[5] R4-1910825, “Beam management in HST with DPS transmission”, MediaTek.

Appendix A: Simulation Assumption

Table A.1 Simulation assumption for HST-SFN channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2x2; |  |
| Precoding scheme | Random precoding based on type1 single panel codebook, bundle size 2 |  |
| Front loaded DMRS symbol | L0=2 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | - DMRS 1+1+1 |  |
| MCS | MCS 4; MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | HST-SFN |  |
| TDD pattern | DDDDDDDSUU ,S(6D:4G:4U) |  |
| TRS periodicity | 10ms, 2 slot TRS |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds=1000m, Dmin=50m |  |
| Rank | Rank = 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | - For 500km/h - Option 1: 1250Hz - Option 2: 875Hz - Option 3: 972Hz | - For 500km/h - Option 1: 1667Hz - Option 2: 1500Hz |
| Testing metric | SNR @70% of maximum throughput |  |

Table A.2 Simulation Parameters for HST Channel Setting

| Ds | Dmin | γ |
| --- | --- | --- |
| 1000 m | 50 m | 45° |

Appendix B: Modeling of Signal Power

The signal power received by the UE from each respective path is given by

pit=e0∙G(θ0t∙H0(θ0t)-γ)2Dmin2Dmin2+(Ds+vt)2, i=0, 0<t≤Ds/ve1∙G(θ1t∙H1(θ1t)-γ)2Dmin2Dmin2+(vt)2, i=1, 0<t≤Ds/ve2∙G(θ2t∙H2(θ2t)-γ)2Dmin2Dmin2+(Ds-vt)2, i=2, 0<t≤Ds/ve3∙G(θ3t∙H3(θ3t)-γ)2Dmin2Dmin2+(2Ds-vt)2, i=3, 0<t≤Ds/vpit mod (Dsv), ∀ i,  t>Ds/v

where Dsis RRH spacing; Dminis distance of RRH and rail; γ is angle of beam direction as figure 3.2

ei=1, if i-th RRH exists0, otherwise

Hix=1 if i-th RRH transmit in only one directionsignx if i-th RRH transmit in both two direction

θit=Ds+vtDmin2+(Ds+vt)2, i=0vtDmin2+(vt)2, i=1Ds-vtDmin2+(Ds-vt)2 , i=22Ds-vtDmin2+(2Ds-vt)2,i=3

Table B.1 Chanel models for deployment options

| Deployment | Channel Model | ei | Hix |
| --- | --- | --- | --- |
| Option 1 | HST 4 taps bidirectional channel | ei=1 ∀ i=0~3 | Hix=signx ∀ i=0~3 |
| Option 2 | HST 2 taps unidirectional channel | ei=1, i=0,10, i=2,3 | Hix=1 ∀ i=0~3 |
| Option 3 | HST 2 taps bidirectional channel | ei=1, i=1,20, i=0,3 | Hix =signx ∀ i=0~3 |

Figure B.2 Beam Direction with Angle of γ