3GPP TSG-RAN WG4 #92Bis Meeting                                        	R4-1911289

Chongqing, China, 14th – 18th Oct., 2019

Source: 	LG Electronics, Verizon

Title: 	TP for TR 36.716-03-02 to include 3 bands DL with 2 bands UL band combinations

Agenda item:	7.6.2

Document for:	Approval

Introduction

The purpose of this document is to introduce co-existence studies and MSD analysis results [1] of LTE inter-band carrier aggregation 3 bands DL with 2 bands UL combinations for TR 36.716-03-02.

Text Proposal

<Start of TP>

----- Unchanged sections omitted -----

6.10 LTE-A inter-band CA: Band 2 and Band 13 and Band 46 DL with 2 bands UL

6.10.1	List of specific combination issues

6.10.1.1	Channel bandwidth per operating band for CA

Table 6.10.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-13A-46D | CA_2A-13A | 2 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 46 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A-46D | CA_2A-13A | 2 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | 110 | 0 |
|  |  | 13 |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46A-46D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A-46C | CA_2A-13A | 2 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | 90 | 0 |
|  |  | 13 |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46C | CA_2A-13A | 2 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | 70 | 0 |
|  |  | 13 |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A-46A | CA_2A-13A | 2 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | 70 | 0 |
|  |  | 13 |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46A-46A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A | CA_2A-13A | 2 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | 50 | 0 |
|  |  | 13 |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |
| CA_2A-13A-46E | CA_2A-13A | 2 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | 110 | 0 |
|  |  | 13 |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |

6.10.1.2	Co-existence studies for LTE-A inter-band UL CA band 2 and band 13and DL CA band 2 and band 13 and band 46

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.10.1.2-1.

Table 6.10.1.2-1: Co-existence study for UL CA band 2 and band 13 and DL CA band 2 and band 13 and band 46

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 777 | 787 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1554 | 1574 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2331 | 2361 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1133 | 1063 | 2627 | 2697 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2913 | 3043 | 356 | 276 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4477 | 4607 | 3404 | 3484 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4763 | 4953 | 421 | 511 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6327 | 6517 | 4181 | 4271 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2126 | 2266 | 5254 | 5394 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1298 | 1198 | 6863 | 6613 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4958 | 5058 | 8177 | 8427 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1339 | 1489 | 4176 | 3976 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6031 | 6181 | 7104 | 7304 |

In Table 6.10.1.2-1, 3rd harmonic from the band 2 can impact the band 46. Since the band 46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101, there is no need to study for this harmonic problem.

6.10.1.3	MSD

When uplink CA band 2 and band 13 is paired with downlink CA band 2 and band 13 and band 46, the 4th order IMD product by band 2 and band 13 falls into the own Rx frequency band 46.

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

6.10.1.3-1: IMD frequency range for UL CA band 2 and band 13 and DL CA band 2 and band 13 and band 46

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_2A-13A-46D CA_2A-13A-46A-46D CA_2A-13A-46A-46C CA_2A-13A-46C CA_2A-13A-46A-46A CA_2A-13A-46A CA_2A-13A-46E | CA_2A-13A | 3*fc_13A + 1*fc_2A | 3*BW_13A + 1*BW_2A |

6.10.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 46 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-46E-66A | CA_2A-66A | 2 | Yes | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | 120 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| CA_2A-46D-66A | CA_2A-66A | 2 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | 100 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| CA_2A-46C-66A | CA_2A-66A | 2 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | 80 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in the Table 5.6A.1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| CA_2A-46A-66A | CA_2A-66A | 2 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |
|  |  | 66 | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |

6.x.1.2	Co-existence studies for LTE-A inter-band UL CA band 2 and band 66 and DL CA band 2 and band 46 and band 66

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for UL CA band 2 and band 66 and DL CA band 2 and band 46 and band 66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 5130 | 5340 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 200 | 70 | 3560 | 3690 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 1920 | 2110 | 1510 | 1710 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 5410 | 5600 | 5270 | 5470 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 3770 | 4020 | 3220 | 3490 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 7260 | 7510 | 6980 | 7250 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limit (MHz) | 140 | 400 | 7120 | 7380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limit (MHz) | 5270 | 4930 | 5930 | 5620 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limit (MHz) | 8690 | 9030 | 9110 | 9420 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limit (MHz) | 1640 | 1310 | 2310 | 1990 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limit (MHz) | 8830 | 9160 | 8970 | 9290 |

In Table 6.x.1.2-1, 3rd harmonics from the band 2 and 66 can impact the band 46. Since the band 46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101, there is no need to study for this harmonic problem.

6.x.1.3	MSD

When uplink CA band 2 and band 66 is paired with downlink CA band 2 and band 46 and band 66, the 3rd and 5th order IMD products by band 2 and band 66 falls into the own Rx frequency band 46.

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

6.x.1.3-1: IMD frequency range for UL CA band 2 and band 66 and DL CA band 2 and band 46 and band 66

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_2A-46E-66A CA_2A-46D-66A CA_2A-46C-66A CA_2A-46A-66A | CA_2A-66A | 2*fc_2A + 1*fc_66A 2*fc_66A + 1*fc_2A 1*fc_2A – 4*fc_66A 1*fc_66A – 4*fc_2A | 2*BW_2A + 1*BW_66A 2*BW_66A + 1*BW_2A 1*BW_2A + 4*BW_66A 1*BW_66A + 4*BW_2A |

6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 can be applied.

6.16 LTE-A inter-band CA: Band 2 and Band 48 and Band 66 DL with 2 bands UL

6.16.1	List of specific combination issues

6.16.1.1	Channel bandwidth per operating band for CA

Table 6.16.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-48A-66A | CA_2A-48A CA_48A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-48C-66A | CA_2A-48A CA_48A-66A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

6.16.1.2	Co-existence studies for LTE-A inter-band UL CA band 48 and band 66 and DL CA band 2 and band 48 and band 66

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.16.1.2-1.

Table 6.16.1.2-1: Co-existence study for UL CA band 48 and band 66 and DL CA band 2 and band 48 and band 66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3550 | 3700 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 7100 | 7400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 10650 | 11100 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 1770 | 1990 | 5260 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 280 | 10 | 5320 | 5690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 6970 | 7260 | 8810 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 1430 | 1790 | 8870 | 9390 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 8680 | 9040 | 12360 | 12880 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limit (MHz) | 3980 | 3540 | 10520 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limit (MHz) | 13090 | 12420 | 3570 | 3140 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limit (MHz) | 15910 | 16580 | 10390 | 10820 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limit (MHz) | 7680 | 7090 | 1760 | 2270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limit (MHz) | 14070 | 14660 | 12230 | 12740 |

6.16.1.3	Co-existence studies for LTE-A inter-band UL CA band 2 and band 48 and DL CA band 2 and band 48 and band 66

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.16.1.3-1.

Table 6.16.1.3-1: Co-existence study for UL CA band 2 and band 48 and DL CA band 2 and band 48 and band 66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3550 | 3700 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 7100 | 7400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 10650 | 11100 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 1640 | 1850 | 5400 | 5610 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 0 | 270 | 5190 | 5550 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 7250 | 7520 | 8950 | 9310 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 1850 | 2180 | 8740 | 9250 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 9100 | 9430 | 12500 | 13010 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limit (MHz) | 3700 | 3280 | 10800 | 11220 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limit (MHz) | 12950 | 12290 | 4090 | 3700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limit (MHz) | 16050 | 16710 | 10950 | 11340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limit (MHz) | 7400 | 6830 | 1370 | 1850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limit (MHz) | 14350 | 14920 | 12650 | 13130 |

6.16.1.4	MSD

When uplink CA (band 48 and band 66) is paired with downlink CA (band 2 and band 48 and band 66), the 2nd and 5th order IMD products by band 48 and band 66 falls into the own Rx frequency band 2. In Table 6.16.1.4-1, evaluated MSD values of 3 bands (band 2 and band 48 and band 66) DL with 2 bands (band 48 and band 66) UL is shown.

Table 6.16.1.4-1: MSD summary for UL CA band 48 and band 66 and DL CA band 2 and band 48 and band 66

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-48A-66A CA_2A-48C-66A | CA_48A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 28.3 | FDD-TDD | IMD2 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  |  |
|  |  | 66 | 1735 | 5 | 25 | 2135 | 5 | N/A |  |  |
|  |  | 2 | 1895 | 5 | 25 | 1975 | 5 | 0 |  | IMD5 |
|  |  | 48 | 3620 | 5 | 25 | 3620 | 5 | N/A |  |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | N/A |  |  |

When uplink CA (band 2 and band 48) is paired with downlink CA (band 2 and band 48 and band 66), the 4th order IMD product by band 2 and band 48 falls into the own Rx frequency band 66. In Table 6.16.1.4-2, evaluated MSD values of 3 bands (band 2 and band 48 and band 66) DL with 2 bands (Band 2 and band 48) UL is shown.

Table 6.16.1.4-2: MSD summary for UL CA band 2 and band 48 and DL CA band 2 and band 48 and band 66

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-48A-66A CA_2A-48C-66A | CA_2A-48A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 48 | 3560 | 5 | 25 | 3560 | 5 | N/A |  |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | TBD |  |  |

6.16.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 can be applied.

6.x LTE-A inter-band CA: Band 13 and Band 48 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_13A-48A-66A | CA_13A-48A CA_13A-66A CA_48A-66A | 13 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

6.x.1.2	Co-existence studies for LTE-A inter-band UL CA band 13 and band 48 and DL CA band 13 and band 48 and band 66

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for UL CA band 13 and band 48 and DL CA band 13 and band 48 and band 66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 3550 | 3700 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 7100 | 7400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 10650 | 11100 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 2763 | 2923 | 4327 | 4487 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 2146 | 1976 | 6313 | 6623 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 5104 | 5274 | 7877 | 8187 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 1369 | 1189 | 9863 | 10323 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 5881 | 6061 | 11427 | 11887 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limit (MHz) | 5846 | 5526 | 8654 | 8974 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limit (MHz) | 14023 | 13413 | 402 | 592 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limit (MHz) | 14977 | 15587 | 6658 | 6848 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limit (MHz) | 9546 | 9076 | 4739 | 5069 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limit (MHz) | 12204 | 12674 | 9431 | 9761 |

According to Table 6.x.1.2.-1 above, no harmonic from the band 13 and 48 is found.

6.x.1.3	Co-existence studies for LTE-A inter-band UL CA band 13 and band 66 and DL CA band 13 and band 48 and band 66

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.3-1.

Table 6.x.1.3-1: Co-existence study for UL CA band 13 and band 66 and DL CA band 13 and band 48 and band 66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 5130 | 5340 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 923 | 1003 | 2487 | 2567 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 226 | 136 | 2633 | 2783 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 3264 | 3354 | 4197 | 4347 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 551 | 651 | 4343 | 4563 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 4041 | 4141 | 5907 | 6127 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limit (MHz) | 2006 | 1846 | 4974 | 5134 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limit (MHz) | 6343 | 6053 | 1438 | 1328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limit (MHz) | 7617 | 7907 | 4818 | 4928 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limit (MHz) | 3786 | 3556 | 1059 | 1229 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limit (MHz) | 6684 | 6914 | 5751 | 5921 |

In Table 6.x.1.3-1, 2nd harmonic from the band 66 can impact the band 48. Since harmonic issues have been already analysed in TS 36.101, there is no need to study for this harmonic problem.

6.x.1.4	Co-existence studies for LTE-A inter-band UL CA band 48 and band 66 and DL CA band 13 and band 48 and band 66

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.4-1.

Table 6.x.1.4-1: Co-existence study for UL CA band 48 and band 66 and DL CA band 13 and band 48 and band 66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3550 | 3700 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 7100 | 7400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 10650 | 11100 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 1770 | 1990 | 5260 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 280 | 10 | 5320 | 5690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 6970 | 7260 | 8810 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 1430 | 1790 | 8870 | 9390 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 8680 | 9040 | 12360 | 12880 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limit (MHz) | 3980 | 3540 | 10520 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limit (MHz) | 13090 | 12420 | 3570 | 3140 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limit (MHz) | 15910 | 16580 | 10390 | 10820 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limit (MHz) | 7680 | 7090 | 1760 | 2270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limit (MHz) | 14070 | 14660 | 12230 | 12740 |

6.x.1.5	MSD

When uplink CA (band 13 and band 48) is paired with downlink CA (band 13 and band 48 and band 66), 3rd order IMD product by band 13 and band 48 falls into the own Rx frequency band 66.

When uplink CA (band 13 and band 66) is paired with downlink CA (band 13 and band 48 and band 66), 5th order IMD product by band 13 and band 66 falls into the own Rx frequency band 48. In Table 6.x.1.x-1, evaluated MSD values of 3 bands (band 13 and band 48 and band 66) DL with 2 bands (band 13 and band 48 & band 14 and band 66) UL is shown.

When uplink CA (band 48 and band 66) is paired with downlink CA (band 13 and band 48 and band 66), there are no harmonic and IMD products falling into own Rx bands. Therefore, no MSD needs to be addressed for this uplink configuration.

Table 6.x.1.5-1: MSD summary for 2 bands UL CA and DL CA band 13 and band 48 and band 66

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_13A-48A-66A | CA_13A-48A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | IMD3 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  |  |
|  |  | 66 | 1731 | 5 | 25 | 2131 | 5 | TBD |  |  |
|  | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 48 | 3626 | 5 | 25 | 3626 | 5 | TBD |  |  |
|  |  | 66 | 1730 | 5 | 25 | 2130 | 5 | N/A |  |  |

6.x.1.6	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 can be applied.

6.15 LTE-A inter-band CA: Band 2 and Band 46 and Band 48 DL with 2 bands UL

6.15.1	List of specific combination issues

6.15.1.1	Channel bandwidth per operating band for CA

Table 6.15.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-46E-48A | CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  | Yes | Yes |  | 120 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  | Yes | Yes |  |  |  |
| CA_2A-46D-48A | CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  | Yes | Yes |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  | Yes | Yes |  |  |  |
| CA_2A-46D-48C | CA_2A-48A | 2 | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  | 120 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-48C | CA_2A-48A | 2 | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  | 100 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-48C | CA_2A-48A | 2 | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 46 |  |  |  |  | Yes |  | Yes | Yes |  | Yes |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-48A | CA_2A-48A | 2 | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  |  | Yes |  | Yes | Yes |  | Yes |  |  |
| CA_2A-46A-48A | CA_2A-48A | 2 | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  | Yes |  |  | Yes |  |  |
|  |  | 48 |  |  |  |  | Yes |  | Yes | Yes |  | Yes |  |  |

6.15.1.2	Co-existence studies for LTE-A inter-band UL CA band 2 and band 48 and DL CA band 2 and band 46 and band 48

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.15.1.2-1.

Table 6.15.1.2-1: Co-existence study for UL CA band 2 and band 48 and DL CA band 2 and band 46 and band 48

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3500 | 3700 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 7000 | 7400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 10500 | 11100 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 1590 | 1850 | 5350 | 5610 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 0 | 320 | 5090 | 5550 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 7200 | 7520 | 8850 | 9310 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 1850 | 2230 | 8590 | 9250 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 9050 | 9430 | 12350 | 13010 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limit (MHz) | 3700 | 3180 | 10700 | 11220 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limit (MHz) | 12950 | 12090 | 4140 | 3700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limit (MHz) | 15850 | 16710 | 10900 | 11340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limit (MHz) | 7400 | 6680 | 1270 | 1850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limit (MHz) | 14200 | 14920 | 12550 | 13130 |

For 3rd harmonic, there is no need to study since band 46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS 36.101.

6.15.1.3	MSD

When uplink CA (band 2 and band 48) is paired with downlink CA (band 2 and band 46 and band 48), the 2nd and 3rd order IMD products by band 2 and band 48 falls into the own Rx frequency band 46.

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

6.15.1.3-1: IMD frequency range for UL CA band 2 and band 48 and DL CA band 2 and band 46 and band 48

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_2A-46E-48A, CA_2A-46D-48A CA_2A-46D-48C CA_2A-46C-48C CA_2A-46D-48A CA_2A-46A-48C CA_2A-46C-48A CA_2A-46A-48A | CA_2A-48A | 1*fc_2A + 1*fc_48A 2*fc_48A – 1*fc_2A | 1*BW_2A + 1*BW_48A 2*BW_48A + 1*BW_2A |

6.15.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 can be applied.

6.x LTE-A inter-band CA: Band 46 and Band 48 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_46A-48A-66A | CA_48A-66A | 46 |  |  |  | Yes |  | Yes | 60 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_46A-48C-66A | CA_48A-66A | 46 |  |  |  | Yes |  | Yes | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_46C-48A-66A | CA_48A-66A | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_46C-48C-66A | CA_48A-66A | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_46D-48A-66A | CA_48A-66A | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_46D-48C-66A | CA_48A-66A | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

6.x.1.2	Co-existence studies for LTE-A inter-band UL CA band 48 and band 66 and DL CA band 46 and band 48 and band 66

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for UL CA band 48 and band 66 and DL CA band 46 and band 48 and band 66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3550 | 3700 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 7100 | 7400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 10650 | 11100 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 1770 | 1990 | 5260 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 280 | 10 | 5320 | 5690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 6970 | 7260 | 8810 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limit (MHz) | 1430 | 1790 | 8870 | 9390 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limit (MHz) | 8680 | 9040 | 12360 | 12880 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limit (MHz) | 3980 | 3540 | 10520 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limit (MHz) | 13090 | 12420 | 3570 | 3140 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limit (MHz) | 15910 | 16580 | 10390 | 10820 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limit (MHz) | 7680 | 7090 | 1760 | 2270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limit (MHz) | 14070 | 14660 | 12230 | 12740 |

For 3rd harmonic, there is no need to study since band 46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS 36.101.

6.x.1.3	MSD

When uplink CA (band 48 and band 66) is paired with downlink CA (band 46 and band 48 and band 66), 2nd and 3rd order IMD products by band 48 and band 66 falls into the own Rx frequency band 46.

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

6.x.1.3-1: IMD frequency range for UL CA band 48 and band 66 and DL CA band 46 and band 48 and band 66

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_46D-48C-66A CA_46C-48C-66A CA_46A-48C-66A CA_46D-48A-66A CA_46C-48A-66A CA_46A-48A-66A | CA_48A-66A | 1*fc_48A + 1*fc_66A 2*fc_48A – 1*fc_66A | 1*BW_48A + 1*BW_66A 2*BW_48A + 1*BW_66A |

6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 can be applied.

----- Unchanged sections omitted -----

<End of TP>

- References

[1] R4-1908724, “TP on summary of self-interference analysis for new x bands DL (x=3,4,5) with 2 bands UL CA”, LG Electronics