3GPP TSG-RAN WG4 Meeting #92bis	R4-1911523

Chongqing, P.R. China 14th – 18th Sept 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Region 1 Extension for Power Class 3 Operation in FR2 and output power boosting

Agenda Item: 			8.14.9

Document for:	Approval

# 1	Introduction

In previous RAN4 meeting a WF [1] was approved aiming to study possible 0 dB MPR region extension from current REGION1 to FR1 like INNER concept [2]. Furthermore, companies were encouraged to evaluate the impact of removing IBE as gating factor to enable power boosting concept [3]. In this paper we present our simulation results for both topics.

# 2	Discussion

## 2.1	Region 1 Extension for Power Class 3 Operation

### 2.1.1	Background

Figure 1 [1] illustrates nicely the intention of the extension. Current Region 1 is such that the allocation size CLRB is at maximum 1/3 of the Maximum transmission bandwidth configuration NRB and allocation is located on the middle 1/3 of the channel bandwidth. “Inner”-like allocations are such that IMD3 products of the allocation fall always within the channel bandwidth and do not reach the OOB region. Also for “Inner”-like allocations the maximum allocation size CLRB is 1/3 of the Maximum transmission bandwidth configuration NRB  and this is the point where Region 1 and “inner” coincide at top part of the triangles in Fig 1. Note also for “Inner”-like allocations MPRnarrow is unchanged.

Figure 1: Region 1 vs. "Inner"

### 2.1.2	Simulation assumptions

PA calibration point was 100 MHz, 120 kHz, QPSK, DFT-S-OFMA, 20RB23with 0 dB MPR

IQ-Image and LO leakage = 25 dBc

CIM3 = 60 dBc, CIM3=40 dBc was checked no impact to results.

Standard emission requirements, OBW, SEM, ACLR, Spurious emissions, EVM, IBE (also without IBE in case of boosting)

### 2.1.3	Results

Firstly, we simulated the needed backoff (BO) and compared it against current FR 2 MPR requirement. Delta was calculated as follows d=MPR – BO and is plotted in Figure 2. Green indicates that MPR is sufficient. Plot for QPSK has some red dots which are at maximum 0.2 dB this would mean that MPR is not sufficient but in case simulation time would have been longer this “noise” would have disappeared hence in our view this “noise” tell that PA calibration point is not too optimistic.

Figure 2: delta against current Region 1 MPR.

Then we did same exercise against proposed “Inner”-like MPR and delta d=MPR – BO  is presented in Figure 3. Delta behaviour is very similar as in Figure 2 including the “MPR-noise” coming from limited simulation time. Considering the 0.2 dB noise as 0 dB all simulated BO cases are within the proposed “Inner”-like MPR. Note we simulated all possible allocations.

Figure 3: delta against current “Inner”-like MPR.

Complete set of triangles can be found from Annex.

Based on the results presented in this paper we propose.

Proposal: REGION 1 MPR for FR2 is extended as described in [1]

## 2.2	Output power boosting

In this section we present the simulation results for possible output power boosting in case IBE is omitted. Firstly, we checked if IBE really is limiting factor and from Figure 4 we can see that especially for QPSK this is the case. For BPSK it is not so clear that IBE is limiting factor for all “Inner”-like MPR.

Figure 4: Gating factors from MPR

In figure 5 (Figure 6 is zoomed Figure 5) we present BO difference for the case when IBE was included in simulations against the case when IBE was omitted. Thus the difference is the output power “Boost”. From the results we can see that BPSK does not gain much “Boost” if IBE is omitted but QPSK benefits from IBE suspension. Inside “Inner”-like region “Boost” ranges between 0.9 dB – 1.8 dB but what is also notable is that there are boost opportunities outside “Inner” regions, this maps also to gator Figure 4.

Figure 5: Output power gain (Boost) when IBE is suspended

Figure 6: Zoomed Fig 5 QPSK

# 3	Conclusion

In this contribution we have presented simulation results for 0 dB MPR region extension from current REGION1 to FR1 like INNER concept and based on our results we propose

Proposal: REGION 1 MPR for FR2 is extended as described in [1]

We have also presented results for power “Boost” when IBE is suspended. We can see “Boost” opportunities for QPSK but not so much for BPSK.

# 4	References

[1] R4-1910280, WF on Region 1 Extension for Power Class 3 Operation in FR2, Skyworks, Nokia, Qualcomm, RAN4#92

[2] R4-1909945	Inner RB Allocations Region 1 Extension for Power Class 3 Operation in FR2, R4 #92, Skyworks Solutions Inc

[3] R4-1909927	Power Boost and IBE in FR2, R4 #92, Qualcomm.

# Appendix

Here are plots where needed BO is compared against new 0 dB MPR region proposal. Green indicated that MPR is sufficient.

Hera are plots for possible Boost when IBE is suspended.