3GPP TSG-RAN WG4 Meeting #92bis	R4-1911915

Chongqing , China, 14th – 18th Oct, 2019

Title: 	Initial simulation results of SyncRef UE identification for NR V2X

Source: 	Huawei, HiSilicon

Agenda item:	8.4.5.2

Document for:	Discussion

Introduction

In this paper, we present the simulation results of SyncRef UE identification performance.

Discussion

Simulation Assumption

According to the agreements in [1], we provide the simulation assumptions for evaluating SyncRef UE identification performance in this section.

Table 1: Simulation assumptions for SynchRef UE identification

| Parameter | Unit | SyncRef UE 1 | SyncRef UE 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 5.9 GHz | The same frequency as for SyncRef UE 1 is used |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz; 60 kHz; | The same values as for SyncRef UE 1 are used |
| System bandwidth | RB | 24 | 24 |
| S-SSB bandwidth | RB | 11 | 11 |
| Utilization of non S-SSB slots for data transmission | % | 100 | 100 |
| Relative PSD on non S-SSB slots relative to SLSS slots | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted S-SSB within a S-SSB period | - | 1 | 1 |
| S-SSB period | ms | 160 | 160 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Eor/Ioc | dB | 6.4 | 1.25; 7.25 (SINR: -6dB; 0dB) |
| Number of Tx antennas | - | 1 | 1 |
| SLSS ID | - | See Table below | See Table below |
| Propagation Condition | - | AWGN TDLC30-1400 Note1 |  |
| Ioc Model | - | AWGN noise |  |
| NOTE 1: The channel models of TDL are used for simplified link level evaluations, which are defined in TR 38.900. |  |  |  |

Table 2: SLSS IDs of the SyncRef UEs

| Synchronous/Asynchronous | Case | SyncRef UE 1 | SyncRef UE 2 |
| --- | --- | --- | --- |
| Synchronous | Case 1 | 336+112 | 112 |
|  | Case 2 | 112 | 224 |
| Asynchronous | Case 3 | 336+112 | 112 |
|  | Case 4 | 112 | 224 |

Table 3: Additional simulation parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of SyncRef UE 1 by the UE | Yes |
| UE having apriori knowledge of SyncRef UEs being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile of the required time for SyncRef UE 2 identification |
| Receive antennas | 2 (uncorrelated) |

Simulation Results

In this section, we provide the 90% percentile of S-PSS/S-SSS detection time for SyncRef UE identification. The results in Table 4 show the number of required S-SSB occasions for S-PSS/S-SSS detection.

Table 4: 90% S-PSS/S-SSS detection time for NR V2X

| SL SCS | Case | AWGN |  | TDLC30-1400 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SINR= -6dB | SINR= 0dB | SINR= -6dB | SINR= 0dB |
| 15KHz | #1 | 1 | 1 | 3 | 2 |
|  | #2 | 1 | 1 | 3 | 2 |
|  | #3 | 1 | 1 | 2 | 1 |
|  | #4 | 1 | 1 | 2 | 1 |
| 30KHz | #1 | 1 | 1 | 3 | 1 |
|  | #2 | 1 | 1 | 3 | 2 |
|  | #3 | 1 | 1 | 2 | 1 |
|  | #4 | 1 | 1 | 2 | 1 |
| 60KHz | #1 | 1 | 1 | 3 | 1 |
|  | #2 | 1 | 1 | 3 | 1 |
|  | #3 | 1 | 1 | 2 | 1 |
|  | #4 | 1 | 1 | 2 | 1 |

From the table, we can see the following observation.

Observation 1: For AWGN channel, the S-PSS/S-SSS detection time for SyncRef UE identification requires one S-SSB occasion with SINR ≥ -6dB.

Observation 2: For fading channel TDL-C 1400Hz, the S-PSS/S-SSS detection time for SyncRef UE identification requires two S-SSB occasions with SINR ≥ 0dB.

Conclusions

In this contribution, we provide the simulation results of S-PSS/S-SSS detection performance for NR V2X sidelink. The following are provided:

Observation 1: For AWGN channel, the S-PSS/S-SSS detection time for SyncRef UE identification requires one S-SSB occasion with SINR ≥ -6dB.

Observation 2: For fading channel TDL-C 1400Hz, the S-PSS/S-SSS detection time for SyncRef UE identification requires two S-SSB occasions with SINR ≥ 0dB.

Reference

- R4-1909994, “WF on NR V2X RRM requirement scope”, LG Electronics