3GPP TSG-RAN WG4 Meeting #92bis                                  R4-1912369

Chongqing, China, 14th – 18th October, 2019

Source: 	CATT

Title: 	TP on BS RF requirements for adding channel bandwidth in band n28

Agenda Item:	9.21.2

Document for:	Approval

Introduction

A new WI on adding wider channel bandwidth in NR band n28 was approved in RAN#85 meeting with the following scope [1].

- Introduce 30MHz and 40MHz BS channel bandwidth.

- Introduce 30MHz UE channel bandwidth.

Note: Used only to lowermost and uppermost channels in band n28.

- Specify UE RF requirements if necessary, including the consideration of MPR/A-MPR and RENSENS to address the potential impact on RF requirements.

- Investigate whether it is possible and, if possible, enable UE support of 30MHz channel bandwidth in Rel-15 in a release independent manner [RAN2].

This paper would like to discuss the BS RF requirements due to this WI.

Discussion

It is seen from this WI scope that BS will support larger maximum channel bandwidth (40MHz) than UE (30MHz) in this band. This is feasible due to the NR specification design on support of wideband operation and future proof signalling design on adding channel bandwidth. UEs can be configured to operate with any channel bandwidth that is no larger than the gNB channel bandwidth.

Regarding the impact on BS specification, it is very limited due to the fact that the BS RF requirements for different channel bandwidth and SCS are defined in a band agnostic manner, e.g.

- The reference frequency point for the out of band emissions is the channel edge without dependency on the channel bandwidth.

- Other requirements are defined for each channel bandwidth and SCS in a generic way. The requirements for a specific channel bandwidth will be the same across all the bands.

If we decide to support a channel bandwidth in an operating band and the channel bandwidth has already been in the specification, the only thing to be done is to indicate the support of such channel bandwidth in Table 5.3.5-1 of 38.104.

Text proposal

--------------------------- Start of text proposal -----------------------

- 5.2	BS requirements

- 5.2.1	Channel bandwidth

The following BS channel bandwidth as specified in Table 5.2.1-1 will be supported for Band n28, where 30MHz and 40MHz are newly introduced channel bandwidth.

Table 5.2.1-1: BS channel bandwidths and SCS for n28

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | 15 | Yes | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

---------------------------- End of text proposal ------------------------

- References

- RP-192261, New WI proposal: Addition of wider channel bandwidth in NR band n28, CBN, Huawei, HiSilicon