3GPP TSG-RAN WG4 Meeting #93                                                                 R4-1913318

Reno, USA, 18th -22nd Nov, 2019

Agenda Item:	11.3.1

Source:	Samsung

Title:	TP for TR 37.815 high power UE (PC2) for EN-DC FDD-TDD

Document for:	Approval

# Introduction

The contribution provides the TP for TR 37.815 high power UE (PC2) for EN-DC FDD-TDD based on [1].

# Reference

1.   R4-1913317 Discussion on high power UE (PC2) for EN-DC FDD-TDD   Samsung

# Text Proposal

# <Start of Text Proposal>

# 9 Summary

The following options have been discussed for UE reporting capability solutions to solve the SAR issue with  PC2 FDD-TDD HPUE:

- Option1 report EN-DC total Duty cycle(Duty threshold) based on DutyLTE*(PLTE/ P26) + DutyNR*(PNR/ P26) ≤ Duty threshold

- Option2 report DutyLTE based on NR TDD sub-frame configuration

- Option3 report DutyNR based on LTE fixed dutycycle with LTE maximum transmit power 23dBm

- Option4 report SARratio based on DutyLTE*(PLTE/ P26) + SARratio*DutyNR*(PNR/ P26) ≤ 50%

- Option5 report SARratio and EN-DC total Duty cycle(Duty threshold) based on DutyLTE*(PLTE/ P26) + SARratio*DutyNR*(PNR/ P26) ≤ Duty threshold

- Option6 configure Plte lower than 23dBm based on LTE 100% dutycycle

Under further discussion the Option 2 and 4 were precluded and moreover the following options were studied. Option 6 is implementation-based solution to calculate allowable NR duty cycle with certain LTE maximum power setting, which is different from other UE capability reporting options providing UE capability to solving PC2 FDD-TDD EN-DC SAR issue. Option 5 is not considered as UE capability reporting solution for FDD-TDD EN-DC HPUE, due to the questionable feasibility for accurate SAR ratio reporting. Between option 1 and option 3, option 3 is selected and the number of LTE reference configurations is restricted to be no more than 2.

# <End of Text Proposal>